//------Digital Temperature Monitor Template-----

/*

* Copyright (c) 2025 Silicon University, Odisha, India

*/

//`timescale 1ns / 1ps

//Put your DEFINES here

`define RST_COUNT	5'd0

`define MAX_COUNT	5'd28

`define CS_LOW_COUNT    5'd4

`define CS_HIGH_COUNT   5'd20

`define SPI_LATCH_COUNT 5'd22

//

`define SPI_IDLE	2'b00

`define SPI_READ	2'b01

`define SPI_LATCH	2'b10

// DO NOT CHANGE THIS MODULE

module digital_temp_monitor_top (

input  wire [7:0] ui_in,    // Dedicated inputs

output wire [7:0] uo_out,   // Dedicated outputs

input  wire [7:0] uio_in,   // IOs: Input path

output wire [7:0] uio_out,  // IOs: Output path

output wire [7:0] uio_oe,   // IOs: Enable path (active high: 0=input, 1=output)

input  wire       ena,      // will go high when the design is enabled

input  wire       clk,      // clock e.g. provide a 10 kHz clock

input  wire       rst_n     // reset_n - low to reset

);

// All output pins must be assigned. If not used, assign to 0.

// The enables may not be auto checked.

assign uio_oe  = 8'b00111011;

assign uio_out[7:6] = 2'b00;

assign uio_out[2] = 1'b0;

//---------------------------------------

// Internal signals

reg [4:0] count;

reg [1:0] spi_state;

reg [7:0] shift_reg;

reg [7:0] tempC_bin_latch;

wire CS, SIO;

reg SCK;

wire [3:0] bcd_msb;

wire [3:0] bcd_lsb;

wire bcd_lsb_carry;

wire [3:0] bcd_data;

assign uio_out[0] = CS;              //CS-->chip select for LM70

assign uio_out[1] = SCK;             //SCK--> clock for LM70

assign SIO = uio_in[2];

//--------------Declaration of intenal signal to ports--------------

// Although the inputs can directly be used in the code,

// always good to internally assign them to a more readable port.

//Internal signal-to-port assignment

assign sel_ob_LSB  = ui_in[1];       //DIP switch-2: if ui_in[0]=0: 1-> LSB, 0-> MSB

// IMPLEMENT CODE HERE

//BCD Logic

//Temp/10 approx. 1/16 + 1/32

assign bcd_msb = (tempC_bin_latch + (tempC_bin_latch>>1))>>4;

//LSB = temp - 10*MSB = temp - (8*MSB + 2*MSB)

assign bcd_lsb = tempC_bin_latch - ((bcd_msb<<3) + (bcd_msb<<1));

// Capturing overflow bit

assign bcd_lsb_carry = bcd_lsb > 4'h9;

// MUX for LSB or MSB

assign bcd_data = sel_ob_LSB ? bcd_lsb : bcd_msb + bcd_lsb_carry;

//SHIFT REGISTER

//Converts input data (uio_in[2]) from serial to parallel.

always @(posedge SCK or negedge rst_n)

if (~rst_n)

shift_reg <= 8'h00;

else

begin

shift_reg <= shift_reg<<1;

shift_reg[0] <= SIO ;

end

//SPI CLOCK SCK generator

always @(negedge clk or negedge rst_n)

if (~rst_n)

SCK <= 1'b0;

else if(CS)

SCK <= 1'b0;

else

SCK <= ~SCK;

// Create CS

assign CS = ~(spi_state == `SPI_READ);

//State machine to create IDLE,READ,LATCH states ot generate CS, SCK

always @(posedge clk or negedge rst_n)

if (~rst_n)

begin

spi_state <= `SPI_IDLE;

tempC_bin_latch <= 8'h00;

end

else if ((count >= `CS_LOW_COUNT) && (count < `CS_HIGH_COUNT) )

begin

spi_state <= `SPI_READ;

end

else if (count == `SPI_LATCH_COUNT)

begin

spi_state <= `SPI_LATCH;

tempC_bin_latch <= shift_reg<<1;

end

else

begin

spi_state <= `SPI_IDLE;

end

//5-b counter

always @(posedge clk or negedge rst_n)

if (~rst_n)

count <= `RST_COUNT;

else if (count == `MAX_COUNT)

count <= `RST_COUNT;

else

count <= count + 1'b1;

Endmodule

module lm70_temp_to_data (

input  signed [7:0]  temp_c,        // Temperature in °C (e.g. 25, 28, 35)

output reg   [15:0]   lm70_word,      // Full 16-bit LM70 output

output reg   [7:0]    lm70_msb_only   // MSB-only (2°C resolution)

);

// Internal registers

reg signed [10:0] temp_11bit;   // 11-bit temperature (0.25°C resolution)

reg signed [7:0]  temp_msb;     // Truncated MSB (2°C resolution)

always @(*) begin

// STEP 1: Convert °C to 0.25°C units

// Example: 25°C -> 25 * 4 = 100

temp_11bit = temp_c <<< 2;  // multiply by 4

// STEP 2: Truncate lower 3 bits (keep MSB 8 bits)

// Equivalent to dividing by 8 (TRUNCATION)

temp_msb = temp_11bit >>> 3;

// STEP 3: Form LM70 16-bit word

// D15–D5 : temperature (11-bit)

// D4–D2  : 111 (fixed)

// D1–D0  : 11  (don’t care)

lm70_word = { temp_msb, 5'b11111 };

// STEP 4: Output MSB-only byte

lm70_msb_only = temp_msb;

end

endmodule

`timescale 1ns/1ns

module tb ();

// Wire up the inputs and outputs:

reg clk;

reg rst_n;

reg ena;

reg [7:0] ui_in;

reg [7:0] uio_in;

wire [7:0] uo_out;

wire [7:0] uio_out;

wire [7:0] uio_oe;

reg [15:0]TEMP_SET;

wire SCK;

wire CS;

wire SIO;

// Instatiation of the DUT

digital_temp_monitor_top  dut(

.ui_in  (ui_in),    // Dedicated inputs

.uo_out (uo_out),   // Dedicated outputs

.uio_in (uio_in),   // IOs: Input path

.uio_out(uio_out),  // IOs: Output path

.uio_oe (uio_oe),   // IOs: Enable path (active high: 0=input, 1=output)

.ena    (ena),      // enable - goes high when design is selected

.clk    (clk),      // clock

.rst_n  (rst_n)     // not reset

);

//Instiate Temperature Sensor LM07

LM07 tsense(.TEMP_SET(TEMP_SET),.CS(CS), .SCK(SCK), .SIO(SIO));

//DUT <-> LM07 Connections

assign CS = uio_out[0];

assign SCK = uio_out[1];

//uio_in[2] is reg so cannot be 'assigned'

always @(*)

begin uio_in[2] <= SIO;

end

//********INITIALS****************

//Initialize CS

// Dump the signals to a VCD file. You can view it with gtkwave.

initial begin

$dumpfile("tb.vcd");

$dumpvars(0, tb);

rst_n = 1'b0;

ui_in = 8'h02;

TEMP_SET = 16'h0C00;

clk = 1'b1;

ena = 1'b1;

#10;

rst_n = 1'b1;

#1450;

$finish(2);

end

//Generate test clock

initial forever #10 clk = ~clk;

endmodule

// end tb

///////TEMP SENSOR LM70 DUMMY MODEL/////////////////

//Define

// In this design we only read the 8-MSBs

// which has a resolution of 2-deg C

////////////////////////////////////////////////////////////////////////////

// Verilog model for the SPI-based temperature

// sensor LM07 or it's equivalent family.

//

module LM07(TEMP_SET,CS, SCK, SIO);

output SIO;

input SCK, CS;

input [15:0] TEMP_SET;

// lm07_reg represents the register that stores

// temperature value after A2D conversion

// FIXME: Model the A2D

reg [15:0] shift_reg;

wire clk_gated;

//Reset at startup

initial begin

shift_reg = TEMP_SET;

//shift_reg = shift_reg>>1;

end

//SIO bit of the LM07 is hardwired output of

// the MSB of the shift register

assign SIO = shift_reg[15];

//Gate the clock with CS

assign clk_gated = ~CS & SCK;

// When CS goes low, load temp_shift_reg with lm07_reg

// If high, reset

always @(CS)

begin

shift_reg = TEMP_SET;

//shift_reg = shift_reg>>1;

end

//Shift register to shift the loaded temp reg

//every negedge of the gated clock

always @(negedge clk_gated)

begin

shift_reg = shift_reg<<1;

end

/*initial begin

$monitor("data=%0b,dataseg=%0b,dataout=%0b",SIO,dataSeg,dbugout);

end*/

endmodule