Zero-RISCY PULP Architecture Toolchain

- STEP-1 : Update Ubuntu (important)

- ->  sudo apt update && sudo apt upgrade -y

- STEP-2 : Install required dependencies (VERY IMPORTANT)

- The toolchain will fail if even one dependency is missing.

- -> sudo apt install -y \

- autoconf \

- automake \

- autotools-dev \

- curl \

- python3 \

- python3-pip \

- libmpc-dev \

- libmpfr-dev \

- libgmp-dev \

- gawk \

- build-essential \

- bison \

- flex \

- texinfo \

- gperf \

- libtool \

- patchutils \

- bc \

- zlib1g-dev \

- libexpat-dev \

- ninja-build \

- git \

- cmake

- STEP-3 : Create a workspace directory
-> /home/amitvlsi01/RISC_V

- STEP-4 : Clone PULP RISC-V GNU toolchain
-> git clone https://github.com/pulp-platform/pulp-riscv-gnu-toolchain.git

- -> cd pulp-riscv-gnu-toolchain

- STEP-5 : Configure for Zero-RISCY
->  ./configure \

- --prefix=$HOME/tools/pulp-riscv \

- --with-arch=rv32imc \

- --with-abi=ilp32

- --prefix -> where toolchain will be installed

- rv32imc -> Zero-RISCY ISA

- ilp32 -> 32-bit ABI (Application Binary Interface)

- ABI decides how binary code behaves, not how source code looks.

- ISA : Instructions CPU supports.

- ABI : How software uses CPU.

- STEP-6 : Build the toolchain (long step)

- -> make -j$(nproc)

20–45 minutes (depends on PC). Laptop should have good cooling system (any cooling pad can be used).

- STEP-7 : Add toolchain to PATH
-> nano ~/.bashrc
-> export PATH=$HOME/tools/pulp-riscv/bin:$PATH
CTRL + O -> Enter -> CTRL + X
->  source ~/.bashrc

- STEP-8 : Verify installation
-> riscv32-unknown-elf-gcc --version
Result : -> riscv32-unknown-elf-gcc (GCC) 10.x.x

- Compile your FIRST program (Zero-RISCY)

- STEP-1 : Create a test file
-> nano first.c

- STEP-2 : Simple Program
`` int main() {
   volatile int x = 10;
   volatile int y = 20;
   volatile int z = x + y;
   while (1);  }   ``

- STEP-3 : Compile for Zero-RISCY 
->  riscv32-unknown-elf-gcc \
    -march=rv32imc \
    -mabi=ilp32 \
    -nostdlib \
    -Ttext=0x0 \
     first.c \
    -o first.elf

| - Flag | - Reason |
| --- | --- |
| - rv32imc | - Zero-RISCY ISA |
| - ilp32 | - 32-bit ABI |
| - -nostdlib | - bare-metal |
| - -ffreestanding | - no OS |
| - -O0 | - readable assembly |

- Bare-metal = your program runs directly on the CPU hardware, with NO operating system.

- STEP-4 : Inspect ELF
-> riscv32-unknown-elf-objdump -d first.elf
Result : 00000000 <_start>:
0: 1141        addi sp,sp,-16
2: c606        sw   ra,12(sp)
4: 419c        lw   a5,0(a1)
6: 4198        lw   a4,0(a1)
8: 00e787b3    add  a5,a5,a4

- ELF = Executable and Linkable Format
This is the file format that quietly connects : C code  ->  compiler  ->  linker  ->  RTL  -> silicon.
An ELF file is a container that holds:
- your compiled machine code
- Data
- Addresses
- Symbols
- debug info

- STEP-5 : Verify memory behavior
-> riscv32-unknown-elf-nm add.elf
Result : 
00000010 D a
00000014 D b
00000018 B c