`include "zeroriscy_config.sv"

import zeroriscy_defines::*;

module zeroriscy_alu

(

input  logic [ALU_OP_WIDTH-1:0]  operator_i,

input  logic [31:0]              operand_a_i,

input  logic [31:0]              operand_b_i,

input  logic [32:0]              multdiv_operand_a_i,

input  logic [32:0]              multdiv_operand_b_i,

input  logic                     multdiv_en_i,

output logic [31:0]              adder_result_o,

output logic [33:0]              adder_result_ext_o,

output logic [31:0]              result_o,

output logic                     comparison_result_o,

output logic                     is_equal_result_o

);

logic [31:0] operand_a_rev;

logic [32:0] operand_b_neg;

// bit reverse operand_a for left shifts and bit counting

generate

genvar k;

for(k = 0; k < 32; k++)

begin : g_revloop

assign operand_a_rev[k] = operand_a_i[31-k];

end

endgenerate

/////////////////////////////////////

//      _       _     _            //

//     / \   __| | __| | ___ _ __  //

//    / _ \ / _` |/ _` |/ _ \ '__| //

//   / ___ \ (_| | (_| |  __/ |    //

//  /_/   \_\__,_|\__,_|\___|_|    //

//                                 //

/////////////////////////////////////

logic        adder_op_b_negate;

logic [32:0] adder_in_a, adder_in_b;

logic [31:0] adder_result;

always_comb

begin

adder_op_b_negate = 1'b0;

unique case (operator_i)

// Adder OPs

ALU_SUB,

// Comparator OPs

ALU_EQ,    ALU_NE,

ALU_GTU,   ALU_GEU,

ALU_LTU,   ALU_LEU,

ALU_GTS,   ALU_GES,

ALU_LTS,   ALU_LES,

ALU_SLTS,  ALU_SLTU,

ALU_SLETS, ALU_SLETU: adder_op_b_negate = 1'b1;

default: ;

endcase

end

// prepare operand a

assign adder_in_a    = multdiv_en_i ? multdiv_operand_a_i : {operand_a_i,1'b1};

// prepare operand b

assign operand_b_neg = {operand_b_i,1'b0} ^ {33{adder_op_b_negate}};

assign adder_in_b    = multdiv_en_i ? multdiv_operand_b_i : operand_b_neg ;

// actual adder

assign adder_result_ext_o = $unsigned(adder_in_a) + $unsigned(adder_in_b);

assign adder_result       = adder_result_ext_o[32:1];

assign adder_result_o     = adder_result;

////////////////////////////////////////

//  ____  _   _ ___ _____ _____       //

// / ___|| | | |_ _|  ___|_   _|      //

// \___ \| |_| || || |_    | |        //

//  ___) |  _  || ||  _|   | |        //

// |____/|_| |_|___|_|     |_|        //

//                                    //

////////////////////////////////////////

logic        shift_left;         // should we shift left

logic        shift_arithmetic;

logic [31:0] shift_amt;          // amount of shift, to the right

logic [31:0] shift_op_a;         // input of the shifter

logic [31:0] shift_result;

logic [31:0] shift_right_result;

logic [31:0] shift_left_result;

assign shift_amt = operand_b_i;

assign shift_left = (operator_i == ALU_SLL);

assign shift_arithmetic = (operator_i == ALU_SRA);

// choose the bit reversed or the normal input for shift operand a

assign shift_op_a    = shift_left ? operand_a_rev : operand_a_i;

// right shifts, we let the synthesizer optimize this

logic [32:0] shift_op_a_32;

assign shift_op_a_32 = { shift_arithmetic & shift_op_a[31], shift_op_a};

assign shift_right_result = $signed(shift_op_a_32) >>> shift_amt[4:0];

// bit reverse the shift_right_result for left shifts

genvar       j;

generate

for(j = 0; j < 32; j++)

begin : g_resrevloop

assign shift_left_result[j] = shift_right_result[31-j];

end

endgenerate

assign shift_result = shift_left ? shift_left_result : shift_right_result;

//////////////////////////////////////////////////////////////////

//   ____ ___  __  __ ____   _    ____  ___ ____   ___  _   _   //

//  / ___/ _ \|  \/  |  _ \ / \  |  _ \|_ _/ ___| / _ \| \ | |  //

// | |  | | | | |\/| | |_) / _ \ | |_) || |\___ \| | | |  \| |  //

// | |__| |_| | |  | |  __/ ___ \|  _ < | | ___) | |_| | |\  |  //

//  \____\___/|_|  |_|_| /_/   \_\_| \_\___|____/ \___/|_| \_|  //

//                                                              //

//////////////////////////////////////////////////////////////////

logic is_equal;

logic is_greater_equal;  // handles both signed and unsigned forms

logic cmp_signed;

always_comb

begin

cmp_signed = 1'b0;

unique case (operator_i)

ALU_GTS,

ALU_GES,

ALU_LTS,

ALU_LES,

ALU_SLTS,

ALU_SLETS: begin

cmp_signed = 1'b1;

end

default:;

endcase

end

assign is_equal = (adder_result == 32'b0);

assign is_equal_result_o = is_equal;

// Is greater equal

always_comb

begin

if ((operand_a_i[31] ^ operand_b_i[31]) == 0)

is_greater_equal = (adder_result[31] == 0);

else

is_greater_equal = operand_a_i[31] ^ (cmp_signed);

end

// GTE unsigned:

// (a[31] == 1 && b[31] == 1) => adder_result[31] == 0

// (a[31] == 0 && b[31] == 0) => adder_result[31] == 0

// (a[31] == 1 && b[31] == 0) => 1

// (a[31] == 0 && b[31] == 1) => 0

// GTE signed:

// (a[31] == 1 && b[31] == 1) => adder_result[31] == 0

// (a[31] == 0 && b[31] == 0) => adder_result[31] == 0

// (a[31] == 1 && b[31] == 0) => 0

// (a[31] == 0 && b[31] == 1) => 1

// generate comparison result

logic cmp_result;

always_comb

begin

cmp_result = is_equal;

unique case (operator_i)

ALU_EQ:            cmp_result = is_equal;

ALU_NE:            cmp_result = (~is_equal);

ALU_GTS, ALU_GTU:  cmp_result = is_greater_equal && (~is_equal);

ALU_GES, ALU_GEU:  cmp_result = is_greater_equal;

ALU_LTS, ALU_SLTS,

ALU_LTU, ALU_SLTU: cmp_result = (~is_greater_equal);

ALU_SLETS,

ALU_SLETU,

ALU_LES, ALU_LEU:  cmp_result = (~is_greater_equal) || is_equal;

default: ;

endcase

end

assign comparison_result_o = cmp_result;

////////////////////////////////////////////////////////

//   ____                 _ _     __  __              //

//  |  _ \ ___  ___ _   _| | |_  |  \/  |_   ___  __  //

//  | |_) / _ \/ __| | | | | __| | |\/| | | | \ \/ /  //

//  |  _ <  __/\__ \ |_| | | |_  | |  | | |_| |>  <   //

//  |_| \_\___||___/\__,_|_|\__| |_|  |_|\__,_/_/\_\  //

//                                                    //

////////////////////////////////////////////////////////

always_comb

begin

result_o   = '0;

unique case (operator_i)

// Standard Operations

ALU_AND:  result_o = operand_a_i & operand_b_i;

ALU_OR:   result_o = operand_a_i | operand_b_i;

ALU_XOR:  result_o = operand_a_i ^ operand_b_i;

// Adder Operations

ALU_ADD, ALU_SUB: result_o = adder_result;

// Shift Operations

ALU_SLL,

ALU_SRL, ALU_SRA: result_o = shift_result;

// Comparison Operations

ALU_EQ,    ALU_NE,

ALU_GTU,   ALU_GEU,

ALU_LTU,   ALU_LEU,

ALU_GTS,   ALU_GES,

ALU_LTS,   ALU_LES,

ALU_SLTS,  ALU_SLTU,

ALU_SLETS, ALU_SLETU: result_o = cmp_result;

default: ; // default case to suppress unique warning

endcase

end

Endmodule

import zeroriscy_defines::*;

module zeroriscy_compressed_decoder

(

input  logic [31:0] instr_i,

output logic [31:0] instr_o,

output logic        is_compressed_o,

output logic        illegal_instr_o

);

//////////////////////////////////////////////////////////////////////////////////////////////////////

//   ____                                                 _   ____                     _            //

//  / ___|___  _ __ ___  _ __  _ __ ___  ___ ___  ___  __| | |  _ \  ___  ___ ___   __| | ___ _ __  //

// | |   / _ \| '_ ` _ \| '_ \| '__/ _ \/ __/ __|/ _ \/ _` | | | | |/ _ \/ __/ _ \ / _` |/ _ \ '__| //

// | |__| (_) | | | | | | |_) | | |  __/\__ \__ \  __/ (_| | | |_| |  __/ (_| (_) | (_| |  __/ |    //

//  \____\___/|_| |_| |_| .__/|_|  \___||___/___/\___|\__,_| |____/ \___|\___\___/ \__,_|\___|_|    //

//                      |_|                                                                         //

//////////////////////////////////////////////////////////////////////////////////////////////////////

always_comb

begin

illegal_instr_o = 1'b0;

instr_o         = '0;

unique case (instr_i[1:0])

// C0

2'b00: begin

unique case (instr_i[15:13])

3'b000: begin

// c.addi4spn -> addi rd', x2, imm

instr_o = {2'b0, instr_i[10:7], instr_i[12:11], instr_i[5], instr_i[6], 2'b00, 5'h02, 3'b000, 2'b01, instr_i[4:2], OPCODE_OPIMM};

if (instr_i[12:5] == 8'b0)  illegal_instr_o = 1'b1;

end

3'b010: begin

// c.lw -> lw rd', imm(rs1')

instr_o = {5'b0, instr_i[5], instr_i[12:10], instr_i[6], 2'b00, 2'b01, instr_i[9:7], 3'b010, 2'b01, instr_i[4:2], OPCODE_LOAD};

end

3'b110: begin

// c.sw -> sw rs2', imm(rs1')

instr_o = {5'b0, instr_i[5], instr_i[12], 2'b01, instr_i[4:2], 2'b01, instr_i[9:7], 3'b010, instr_i[11:10], instr_i[6], 2'b00, OPCODE_STORE};

end

default: begin

illegal_instr_o = 1'b1;

end

endcase

end

// C1

2'b01: begin

unique case (instr_i[15:13])

3'b000: begin

// c.addi -> addi rd, rd, nzimm

// c.nop

instr_o = {{6 {instr_i[12]}}, instr_i[12], instr_i[6:2], instr_i[11:7], 3'b0, instr_i[11:7], OPCODE_OPIMM};

end

3'b001, 3'b101: begin

// 001: c.jal -> jal x1, imm

// 101: c.j   -> jal x0, imm

instr_o = {instr_i[12], instr_i[8], instr_i[10:9], instr_i[6], instr_i[7], instr_i[2], instr_i[11], instr_i[5:3], {9 {instr_i[12]}}, 4'b0, ~instr_i[15], OPCODE_JAL};

end

3'b010: begin

// c.li -> addi rd, x0, nzimm

instr_o = {{6 {instr_i[12]}}, instr_i[12], instr_i[6:2], 5'b0, 3'b0, instr_i[11:7], OPCODE_OPIMM};

if (instr_i[11:7] == 5'b0)  illegal_instr_o = 1'b1;

end

3'b011: begin

// c.lui -> lui rd, imm

instr_o = {{15 {instr_i[12]}}, instr_i[6:2], instr_i[11:7], OPCODE_LUI};

if (instr_i[11:7] == 5'h02) begin

// c.addi16sp -> addi x2, x2, nzimm

instr_o = {{3 {instr_i[12]}}, instr_i[4:3], instr_i[5], instr_i[2], instr_i[6], 4'b0, 5'h02, 3'b000, 5'h02, OPCODE_OPIMM};

end else if (instr_i[11:7] == 5'b0) begin

illegal_instr_o = 1'b1;

end

if ({instr_i[12], instr_i[6:2]} == 6'b0) illegal_instr_o = 1'b1;

end

3'b100: begin

unique case (instr_i[11:10])

2'b00,

2'b01: begin

// 00: c.srli -> srli rd, rd, shamt

// 01: c.srai -> srai rd, rd, shamt

instr_o = {1'b0, instr_i[10], 5'b0, instr_i[6:2], 2'b01, instr_i[9:7], 3'b101, 2'b01, instr_i[9:7], OPCODE_OPIMM};

if (instr_i[12] == 1'b1)  illegal_instr_o = 1'b1;

if (instr_i[6:2] == 5'b0) illegal_instr_o = 1'b1;

end

2'b10: begin

// c.andi -> andi rd, rd, imm

instr_o = {{6 {instr_i[12]}}, instr_i[12], instr_i[6:2], 2'b01, instr_i[9:7], 3'b111, 2'b01, instr_i[9:7], OPCODE_OPIMM};

end

2'b11: begin

unique case ({instr_i[12], instr_i[6:5]})

3'b000: begin

// c.sub -> sub rd', rd', rs2'

instr_o = {2'b01, 5'b0, 2'b01, instr_i[4:2], 2'b01, instr_i[9:7], 3'b000, 2'b01, instr_i[9:7], OPCODE_OP};

end

3'b001: begin

// c.xor -> xor rd', rd', rs2'

instr_o = {7'b0, 2'b01, instr_i[4:2], 2'b01, instr_i[9:7], 3'b100, 2'b01, instr_i[9:7], OPCODE_OP};

end

3'b010: begin

// c.or  -> or  rd', rd', rs2'

instr_o = {7'b0, 2'b01, instr_i[4:2], 2'b01, instr_i[9:7], 3'b110, 2'b01, instr_i[9:7], OPCODE_OP};

end

3'b011: begin

// c.and -> and rd', rd', rs2'

instr_o = {7'b0, 2'b01, instr_i[4:2], 2'b01, instr_i[9:7], 3'b111, 2'b01, instr_i[9:7], OPCODE_OP};

end

3'b100,

3'b101,

3'b110,

3'b111: begin

// 100: c.subw

// 101: c.addw

illegal_instr_o = 1'b1;

end

endcase

end

endcase

end

3'b110, 3'b111: begin

// 0: c.beqz -> beq rs1', x0, imm

// 1: c.bnez -> bne rs1', x0, imm

instr_o = {{4 {instr_i[12]}}, instr_i[6:5], instr_i[2], 5'b0, 2'b01, instr_i[9:7], 2'b00, instr_i[13], instr_i[11:10], instr_i[4:3], instr_i[12], OPCODE_BRANCH};

end

default: begin

illegal_instr_o = 1'b1;

end

endcase

end

// C2

2'b10: begin

unique case (instr_i[15:13])

3'b000: begin

// c.slli -> slli rd, rd, shamt

instr_o = {7'b0, instr_i[6:2], instr_i[11:7], 3'b001, instr_i[11:7], OPCODE_OPIMM};

if (instr_i[11:7] == 5'b0)  illegal_instr_o = 1'b1;

if (instr_i[12] == 1'b1 || instr_i[6:2] == 5'b0)  illegal_instr_o = 1'b1;

end

3'b010: begin

// c.lwsp -> lw rd, imm(x2)

instr_o = {4'b0, instr_i[3:2], instr_i[12], instr_i[6:4], 2'b00, 5'h02, 3'b010, instr_i[11:7], OPCODE_LOAD};

if (instr_i[11:7] == 5'b0)  illegal_instr_o = 1'b1;

end

3'b100: begin

if (instr_i[12] == 1'b0) begin

// c.mv -> add rd/rs1, x0, rs2

instr_o = {7'b0, instr_i[6:2], 5'b0, 3'b0, instr_i[11:7], OPCODE_OP};

if (instr_i[6:2] == 5'b0) begin

// c.jr -> jalr x0, rd/rs1, 0

instr_o = {12'b0, instr_i[11:7], 3'b0, 5'b0, OPCODE_JALR};

end

end else begin

// c.add -> add rd, rd, rs2

instr_o = {7'b0, instr_i[6:2], instr_i[11:7], 3'b0, instr_i[11:7], OPCODE_OP};

if (instr_i[11:7] == 5'b0) begin

// c.ebreak -> ebreak

instr_o = {32'h00_10_00_73};

if (instr_i[6:2] != 5'b0)

illegal_instr_o = 1'b1;

end else if (instr_i[6:2] == 5'b0) begin

// c.jalr -> jalr x1, rs1, 0

instr_o = {12'b0, instr_i[11:7], 3'b000, 5'b00001, OPCODE_JALR};

end

end

end

3'b110: begin

// c.swsp -> sw rs2, imm(x2)

instr_o = {4'b0, instr_i[8:7], instr_i[12], instr_i[6:2], 5'h02, 3'b010, instr_i[11:9], 2'b00, OPCODE_STORE};

end

default: begin

illegal_instr_o = 1'b1;

end

endcase

end

default: begin

// 32 bit (or more) instruction

instr_o = instr_i;

end

endcase

end

assign is_compressed_o = (instr_i[1:0] != 2'b11);

Endmodule

`include "zeroriscy_config.sv"

import zeroriscy_defines::*;

module zeroriscy_controller

#(

parameter REG_ADDR_WIDTH      = 5

)

(

input  logic        clk,

input  logic        rst_n,

input  logic        fetch_enable_i,             // Start the decoding

output logic        ctrl_busy_o,                // Core is busy processing instructions

output logic        first_fetch_o,              // Core is at the FIRST FETCH stage

output logic        is_decoding_o,              // Core is in decoding state

// decoder related signals

output logic        deassert_we_o,              // deassert write enable for next instruction

input  logic        illegal_insn_i,             // decoder encountered an invalid instruction

input  logic        ecall_insn_i,               // ecall encountered an mret instruction

input  logic        mret_insn_i,                // decoder encountered an mret instruction

input  logic        pipe_flush_i,               // decoder wants to do a pipe flush

input  logic        ebrk_insn_i,                // decoder encountered an ebreak instruction

input  logic        csr_status_i,              // decoder encountered an csr status instruction

// from IF/ID pipeline

input  logic        instr_valid_i,              // instruction coming from IF/ID pipeline is valid

// from prefetcher

output logic        instr_req_o,                // Start fetching instructions

// to prefetcher

output logic        pc_set_o,                   // jump to address set by pc_mux

output logic [2:0]  pc_mux_o,                   // Selector in the Fetch stage to select the rigth PC (normal, jump ...)

output logic [1:0]  exc_pc_mux_o,               // Selects target PC for exception

// LSU

input  logic        data_misaligned_i,

// jump/branch signals

input  logic        branch_in_id_i,             // branch in id

input  logic        branch_taken_ex_i,          // branch taken signal

input  logic        branch_set_i,               // branch taken set signal

input  logic        jump_set_i,                 // jump taken set signal

input  logic        instr_multicyle_i,          // multicycle instructions active

// Interrupt Controller Signals

input  logic        irq_req_ctrl_i,

input  logic [4:0]  irq_id_ctrl_i,

input  logic        m_IE_i,                     // interrupt enable bit from CSR (M mode)

output logic        irq_ack_o,

output logic [4:0]  irq_id_o,

output logic [5:0]  exc_cause_o,

output logic        exc_ack_o,

output logic        exc_kill_o,

output logic        csr_save_if_o,

output logic        csr_save_id_o,

output logic [5:0]  csr_cause_o,

output logic        csr_restore_mret_id_o,

output logic        csr_save_cause_o,

// Debug Signals

input  logic        dbg_req_i,                  // a trap was hit, so we have to flush EX and WB

output logic        dbg_ack_o,                  // we stopped and give control to debug now

input  logic        dbg_stall_i,                // Pipeline stall is requested

input  logic        dbg_jump_req_i,             // Change PC to value from debug unit

input  logic [DBG_SETS_W-1:0] dbg_settings_i,

output logic        dbg_trap_o,

// forwarding signals

output logic [1:0]  operand_a_fw_mux_sel_o,     // regfile ra data selector form ID stage

// stall signals

output logic        halt_if_o,

output logic        halt_id_o,

input  logic        id_ready_i,                 // ID stage is ready

// Performance Counters

output logic        perf_jump_o,                // we are executing a jump instruction   (j, jr, jal, jalr)

output logic        perf_tbranch_o             // we are executing a taken branch instruction

);

// FSM state encoding

enum  logic [3:0] { RESET, BOOT_SET, WAIT_SLEEP, SLEEP, FIRST_FETCH,

DECODE, FLUSH, IRQ_TAKEN,

DBG_SIGNAL, DBG_SIGNAL_SLEEP, DBG_WAIT, DBG_WAIT_BRANCH, DBG_WAIT_SLEEP } ctrl_fsm_cs, ctrl_fsm_ns;

logic irq_enable_int;

`ifndef SYNTHESIS

// synopsys translate_off

// make sure we are called later so that we do not generate messages for

// glitches

always_ff @(negedge clk)

begin

// print warning in case of decoding errors

if (is_decoding_o && illegal_insn_i) begin

$display("%t: Illegal instruction (core %0d) at PC 0x%h:", $time, zeroriscy_core.core_id_i,

zeroriscy_id_stage.pc_id_i);

end

end

// synopsys translate_on

`endif

////////////////////////////////////////////////////////////////////////////////////////////

//   ____ ___  ____  _____    ____ ___  _   _ _____ ____   ___  _     _     _____ ____    //

//  / ___/ _ \|  _ \| ____|  / ___/ _ \| \ | |_   _|  _ \ / _ \| |   | |   | ____|  _ \   //

// | |  | | | | |_) |  _|   | |  | | | |  \| | | | | |_) | | | | |   | |   |  _| | |_) |  //

// | |__| |_| |  _ <| |___  | |__| |_| | |\  | | | |  _ <| |_| | |___| |___| |___|  _ <   //

//  \____\___/|_| \_\_____|  \____\___/|_| \_| |_| |_| \_\\___/|_____|_____|_____|_| \_\  //

//                                                                                        //

////////////////////////////////////////////////////////////////////////////////////////////

always_comb

begin

// Default values

instr_req_o            = 1'b1;

exc_ack_o              = 1'b0;

exc_kill_o             = 1'b0;

csr_save_if_o          = 1'b0;

csr_save_id_o          = 1'b0;

csr_restore_mret_id_o  = 1'b0;

csr_save_cause_o       = 1'b0;

exc_cause_o            = '0;

exc_pc_mux_o           = EXC_PC_IRQ;

csr_cause_o            = '0;

pc_mux_o               = PC_BOOT;

pc_set_o               = 1'b0;

ctrl_fsm_ns            = ctrl_fsm_cs;

ctrl_busy_o            = 1'b1;

is_decoding_o          = 1'b0;

first_fetch_o          = 1'b0;

halt_if_o              = 1'b0;

halt_id_o              = 1'b0;

dbg_ack_o              = 1'b0;

irq_ack_o              = 1'b0;

irq_id_o               = irq_id_ctrl_i;

irq_enable_int         = m_IE_i;

// a trap towards the debug unit is generated when one of the

// following conditions are true:

// - ebreak instruction encountered

// - single-stepping mode enabled

// - illegal instruction exception and IIE bit is set

// - IRQ and INTE bit is set and no exception is currently running

// - Debuger requests halt

dbg_trap_o             = 1'b0;

perf_tbranch_o         = 1'b0;

perf_jump_o            = 1'b0;

unique case (ctrl_fsm_cs)

// We were just reset, wait for fetch_enable

RESET:

begin

ctrl_busy_o   = 1'b0;

instr_req_o   = 1'b0;

if (fetch_enable_i == 1'b1)

ctrl_fsm_ns = BOOT_SET;

else if (dbg_req_i) begin

// just go to debug even when we did not yet get a fetch enable

// this means that the NPC will not be set yet

ctrl_fsm_ns = DBG_SIGNAL;

end

end

// copy boot address to instr fetch address

BOOT_SET:

begin

instr_req_o   = 1'b1;

pc_mux_o      = PC_BOOT;

pc_set_o      = 1'b1;

ctrl_fsm_ns = FIRST_FETCH;

end

WAIT_SLEEP:

begin

ctrl_busy_o   = 1'b0;

instr_req_o   = 1'b0;

halt_if_o     = 1'b1;

halt_id_o     = 1'b1;

ctrl_fsm_ns   = SLEEP;

end

// instruction in if_stage is already valid

SLEEP:

begin

// we begin execution when either fetch_enable is high or an

// interrupt has arrived

ctrl_busy_o   = 1'b0;

instr_req_o   = 1'b0;

halt_if_o     = 1'b1;

halt_id_o     = 1'b1;

dbg_trap_o    = dbg_settings_i[DBG_SETS_SSTE];

if (dbg_req_i) begin

// debug request, now we need to check if we should stay sleeping or

// go to normal processing later

if (fetch_enable_i || irq_req_ctrl_i)

ctrl_fsm_ns = DBG_SIGNAL;

else

ctrl_fsm_ns = DBG_SIGNAL_SLEEP;

end else begin

// no debug request incoming, normal execution flow

if (fetch_enable_i || irq_req_ctrl_i)

begin

ctrl_fsm_ns  = FIRST_FETCH;

end

end

end

FIRST_FETCH:

begin

first_fetch_o = 1'b1;

// Stall because of IF miss

if ((id_ready_i == 1'b1) && (dbg_stall_i == 1'b0))

begin

ctrl_fsm_ns = DECODE;

end

if (irq_req_ctrl_i & irq_enable_int) begin

// This assumes that the pipeline is always flushed before

// going to sleep.

ctrl_fsm_ns = IRQ_TAKEN;

halt_if_o   = 1'b1;

halt_id_o   = 1'b1;

end

end

DECODE:

begin

is_decoding_o = 1'b0;

// decode and execute instructions only if the current conditional

// branch in the EX stage is either not taken, or there is no

// conditional branch in the EX stage

if (instr_valid_i)

begin // now analyze the current instruction in the ID stage

is_decoding_o = 1'b1;

unique case (1'b1)

branch_set_i: begin

pc_mux_o          = PC_JUMP;

pc_set_o          = 1'b1;

perf_tbranch_o    = 1'b1;

dbg_trap_o        = dbg_settings_i[DBG_SETS_SSTE];

if (dbg_req_i)

ctrl_fsm_ns = DBG_SIGNAL;

end

jump_set_i: begin

pc_mux_o          = PC_JUMP;

pc_set_o          = 1'b1;

perf_jump_o       = 1'b1;

dbg_trap_o        = dbg_settings_i[DBG_SETS_SSTE];

end

mret_insn_i | ecall_insn_i | pipe_flush_i | ebrk_insn_i | illegal_insn_i | csr_status_i: begin

ctrl_fsm_ns = FLUSH;

halt_if_o   = 1'b1;

halt_id_o   = 1'b1;

end

default: begin

dbg_trap_o = dbg_settings_i[DBG_SETS_SSTE];

unique case (1'b1)

irq_req_ctrl_i & irq_enable_int & ~instr_multicyle_i & ~branch_in_id_i: begin

ctrl_fsm_ns = IRQ_TAKEN;

halt_if_o   = 1'b1;

halt_id_o   = 1'b1;

end

dbg_req_i & ~branch_taken_ex_i: begin

halt_if_o = 1'b1;

if (id_ready_i) begin

ctrl_fsm_ns = DBG_SIGNAL;

end

end

default:

exc_kill_o    = irq_req_ctrl_i & ~instr_multicyle_i & ~branch_in_id_i ? 1'b1 : 1'b0;

endcase

end

endcase

end

else //~instr_valid_i

begin

if (irq_req_ctrl_i & irq_enable_int) begin

ctrl_fsm_ns = IRQ_TAKEN;

halt_if_o   = 1'b1;

halt_id_o   = 1'b1;

end

end

end

// now we can signal to the debugger that our pipeline is empty and it

// can examine our current state

DBG_SIGNAL:

begin

dbg_ack_o  = 1'b1;

halt_if_o  = 1'b1;

ctrl_fsm_ns = DBG_WAIT;

end

DBG_SIGNAL_SLEEP:

begin

dbg_ack_o  = 1'b1;

halt_if_o  = 1'b1;

ctrl_fsm_ns = DBG_WAIT_SLEEP;

end

// The Debugger is active in this state

// we wait until it is done and go back to SLEEP

DBG_WAIT_SLEEP:

begin

halt_if_o = 1'b1;

if (dbg_jump_req_i) begin

pc_mux_o     = PC_DBG_NPC;

pc_set_o     = 1'b1;

ctrl_fsm_ns  = DBG_WAIT;

end

if (dbg_stall_i == 1'b0) begin

ctrl_fsm_ns = SLEEP;

end

end

// The Debugger is active in this state

// we wait until it is done and go back to DECODE

DBG_WAIT:

begin

halt_if_o = 1'b1;

if (dbg_jump_req_i) begin

pc_mux_o     = PC_DBG_NPC;

pc_set_o     = 1'b1;

ctrl_fsm_ns  = DBG_WAIT;

end

if (dbg_stall_i == 1'b0) begin

ctrl_fsm_ns = DECODE;

end

end

IRQ_TAKEN:

begin

pc_mux_o          = PC_EXCEPTION;

pc_set_o          = 1'b1;

exc_pc_mux_o      = EXC_PC_IRQ;

exc_cause_o       = {1'b0,irq_id_ctrl_i};

csr_save_cause_o  = 1'b1;

csr_cause_o       = {1'b1,irq_id_ctrl_i};

csr_save_if_o     = 1'b1;

irq_ack_o         = 1'b1;

exc_ack_o         = 1'b1;

ctrl_fsm_ns       = DECODE;

end

// flush the pipeline, insert NOP

FLUSH:

begin

halt_if_o = fetch_enable_i ? dbg_req_i : 1'b1;

halt_id_o = 1'b1;

ctrl_fsm_ns = dbg_req_i ? DBG_SIGNAL : DECODE;

unique case(1'b1)

ecall_insn_i: begin

//ecall

pc_mux_o              = PC_EXCEPTION;

pc_set_o              = 1'b1;

csr_save_id_o         = 1'b1;

csr_save_cause_o      = 1'b1;

exc_pc_mux_o          = EXC_PC_ECALL;

exc_cause_o           = EXC_CAUSE_ECALL_MMODE;

csr_cause_o           = EXC_CAUSE_ECALL_MMODE;

dbg_trap_o            = dbg_settings_i[DBG_SETS_ECALL] | dbg_settings_i[DBG_SETS_SSTE];

end

illegal_insn_i: begin

//exceptions

pc_mux_o              = PC_EXCEPTION;

pc_set_o              = 1'b1;

csr_save_id_o         = 1'b1;

csr_save_cause_o      = 1'b1;

exc_pc_mux_o          = EXC_PC_ILLINSN;

exc_cause_o           = EXC_CAUSE_ILLEGAL_INSN;

csr_cause_o           = EXC_CAUSE_ILLEGAL_INSN;

dbg_trap_o            = dbg_settings_i[DBG_SETS_EILL] | dbg_settings_i[DBG_SETS_SSTE];

end

mret_insn_i: begin

//mret

pc_mux_o              = PC_ERET;

pc_set_o              = 1'b1;

csr_restore_mret_id_o = 1'b1;

dbg_trap_o            = dbg_settings_i[DBG_SETS_SSTE];

end

ebrk_insn_i: begin

dbg_trap_o    = dbg_settings_i[DBG_SETS_EBRK] | dbg_settings_i[DBG_SETS_SSTE];;

exc_cause_o   = EXC_CAUSE_BREAKPOINT;

end

csr_status_i: begin

dbg_trap_o    = dbg_settings_i[DBG_SETS_SSTE];

end

pipe_flush_i: begin

dbg_trap_o    = dbg_settings_i[DBG_SETS_SSTE];

end

default:;

endcase

if(fetch_enable_i) begin

if(dbg_req_i)

ctrl_fsm_ns = DBG_SIGNAL;

else

ctrl_fsm_ns = DECODE;

end else begin

if(dbg_req_i)

ctrl_fsm_ns = DBG_SIGNAL_SLEEP;

else

ctrl_fsm_ns = (mret_insn_i | pipe_flush_i) ? WAIT_SLEEP : DECODE;

end

end

default: begin

instr_req_o = 1'b0;

ctrl_fsm_ns = RESET;

end

endcase

end

/////////////////////////////////////////////////////////////

//  ____  _        _ _    ____            _             _  //

// / ___|| |_ __ _| | |  / ___|___  _ __ | |_ _ __ ___ | | //

// \___ \| __/ _` | | | | |   / _ \| '_ \| __| '__/ _ \| | //

//  ___) | || (_| | | | | |__| (_) | | | | |_| | | (_) | | //

// |____/ \__\__,_|_|_|  \____\___/|_| |_|\__|_|  \___/|_| //

//                                                         //

/////////////////////////////////////////////////////////////

always_comb

begin

deassert_we_o  = 1'b0;

// deassert WE when the core is not decoding instructions

if (~is_decoding_o)

deassert_we_o = 1'b1;

// deassert WE in case of illegal instruction

if (illegal_insn_i)

deassert_we_o = 1'b1;

end

// Forwarding control unit

assign operand_a_fw_mux_sel_o = data_misaligned_i ? SEL_MISALIGNED : SEL_REGFILE;

// update registers

always_ff @(posedge clk , negedge rst_n)

begin : UPDATE_REGS

if ( rst_n == 1'b0 )

begin

ctrl_fsm_cs <= RESET;

//jump_done_q <= 1'b0;

end

else

begin

ctrl_fsm_cs <= ctrl_fsm_ns;

// clear when id is valid (no instruction incoming)

//jump_done_q <= jump_done & (~id_ready_i);

end

end

//----------------------------------------------------------------------------

// Assertions

//----------------------------------------------------------------------------

`ifndef VERILATOR

assert property (

@(posedge clk) (~(dbg_req_i & irq_req_ctrl_i)) ) else $warning("Both dbg_req_i and irq_req_ctrl_i are active");

`endif

endmodule // controller

`include "zeroriscy_config.sv"

import zeroriscy_defines::*;

module zeroriscy_core

#(

parameter N_EXT_PERF_COUNTERS = 0,

parameter RV32E               = 0,

parameter RV32M               = 1

)

(

// Clock and Reset

input  logic        clk_i,

input  logic        rst_ni,

input  logic        clock_en_i,    // enable clock, otherwise it is gated

input  logic        test_en_i,     // enable all clock gates for testing

// Core ID, Cluster ID and boot address are considered more or less static

input  logic [ 3:0] core_id_i,

input  logic [ 5:0] cluster_id_i,

input  logic [31:0] boot_addr_i,

// Instruction memory interface

output logic                         instr_req_o,

input  logic                         instr_gnt_i,

input  logic                         instr_rvalid_i,

output logic                  [31:0] instr_addr_o,

input  logic                  [31:0] instr_rdata_i,

// Data memory interface

output logic        data_req_o,

input  logic        data_gnt_i,

input  logic        data_rvalid_i,

output logic        data_we_o,

output logic [3:0]  data_be_o,

output logic [31:0] data_addr_o,

output logic [31:0] data_wdata_o,

input  logic [31:0] data_rdata_i,

input  logic        data_err_i,

// Interrupt inputs

input  logic        irq_i,                 // level sensitive IR lines

input  logic [4:0]  irq_id_i,

output logic        irq_ack_o,             // irq ack

output logic [4:0]  irq_id_o,

// Debug Interface

input  logic        debug_req_i,

output logic        debug_gnt_o,

output logic        debug_rvalid_o,

input  logic [14:0] debug_addr_i,

input  logic        debug_we_i,

input  logic [31:0] debug_wdata_i,

output logic [31:0] debug_rdata_o,

output logic        debug_halted_o,

input  logic        debug_halt_i,

input  logic        debug_resume_i,

// CPU Control Signals

input  logic        fetch_enable_i,

output logic        core_busy_o,

input  logic [N_EXT_PERF_COUNTERS-1:0] ext_perf_counters_i

);

localparam N_HWLP      = 2;

localparam N_HWLP_BITS = $clog2(N_HWLP);

// IF/ID signals

logic              instr_valid_id;

logic [31:0]       instr_rdata_id;    // Instruction sampled inside IF stage

logic              is_compressed_id;

logic              illegal_c_insn_id; // Illegal compressed instruction sent to ID stage

logic [31:0]       pc_if;             // Program counter in IF stage

logic [31:0]       pc_id;             // Program counter in ID stage

logic              clear_instr_valid;

logic              pc_set;

logic [2:0]        pc_mux_id;     // Mux selector for next PC

logic [1:0]        exc_pc_mux_id;     // Mux selector for exception PC

logic [5:0]        exc_cause;

logic              lsu_load_err;

logic              lsu_store_err;

// ID performance counter signals

logic        is_decoding;

logic        data_misaligned;

logic [31:0] misaligned_addr;

// Jump and branch target and decision (EX->IF)

logic [31:0] jump_target_ex;

logic        branch_in_ex;

logic        branch_decision;

logic        ctrl_busy;

logic        if_busy;

logic        lsu_busy;

// ALU Control

logic [ALU_OP_WIDTH-1:0] alu_operator_ex;

logic [31:0] alu_operand_a_ex;

logic [31:0] alu_operand_b_ex;

logic [31:0] alu_adder_result_ex; // Used to forward computed address to LSU

logic [31:0] regfile_wdata_ex;

// Multiplier Control

logic        mult_en_ex;

logic        div_en_ex;

logic [1:0]  multdiv_operator_ex;

logic [1:0]  multdiv_signed_mode_ex;

logic [31:0] multdiv_operand_a_ex;

logic [31:0] multdiv_operand_b_ex;

// CSR control

logic        csr_access_ex;

logic  [1:0] csr_op_ex;

logic        csr_access;

logic  [1:0] csr_op;

logic [11:0] csr_addr;

logic [11:0] csr_addr_int;

logic [31:0] csr_rdata;

logic [31:0] csr_wdata;

// Data Memory Control

logic        data_we_ex;

logic [1:0]  data_type_ex;

logic        data_sign_ext_ex;

logic [1:0]  data_reg_offset_ex;

logic        data_req_ex;

logic [31:0] data_wdata_ex;

logic        data_load_event_ex;

logic        data_misaligned_ex;

logic [31:0] regfile_wdata_lsu;

// stall control

logic        halt_if;

logic        id_ready;

logic        ex_ready;

logic        if_valid;

logic        id_valid;

logic        wb_valid;

logic        lsu_ready_ex;

logic        data_valid_lsu;

// Signals between instruction core interface and pipe (if and id stages)

logic        instr_req_int;    // Id stage asserts a req to instruction core interface

// Interrupts

logic        m_irq_enable;

logic [31:0] mepc;

logic        csr_save_cause;

logic        csr_save_if;

logic        csr_save_id;

logic [5:0]  csr_cause;

logic        csr_restore_mret_id;

logic        csr_restore_uret_id;

// Debug Unit

logic [DBG_SETS_W-1:0] dbg_settings;

logic        dbg_req;

logic        dbg_ack;

logic        dbg_stall;

logic        dbg_trap;

// Debug GPR Read Access

logic        dbg_reg_rreq;

logic [4:0]  dbg_reg_raddr;

logic [31:0] dbg_reg_rdata;

// Debug GPR Write Access

logic        dbg_reg_wreq;

logic [4:0]  dbg_reg_waddr;

logic [31:0] dbg_reg_wdata;

// Debug CSR Access

logic        dbg_csr_req;

logic [11:0] dbg_csr_addr;

logic        dbg_csr_we;

logic [31:0] dbg_csr_wdata;

logic [31:0] dbg_jump_addr;

logic        dbg_jump_req;

// Performance Counters

logic        perf_imiss;

logic        perf_jump;

logic        perf_branch;

logic        perf_tbranch;

//core busy signals

logic        core_ctrl_firstfetch, core_busy_int, core_busy_q;

//////////////////////////////////////////////////////////////////////////////////////////////

//   ____ _            _      __  __                                                   _    //

//  / ___| | ___   ___| | __ |  \/  | __ _ _ __   __ _  __ _  ___ _ __ ___   ___ _ __ | |_  //

// | |   | |/ _ \ / __| |/ / | |\/| |/ _` | '_ \ / _` |/ _` |/ _ \ '_ ` _ \ / _ \ '_ \| __| //

// | |___| | (_) | (__|   <  | |  | | (_| | | | | (_| | (_| |  __/ | | | | |  __/ | | | |_  //

//  \____|_|\___/ \___|_|\_\ |_|  |_|\__,_|_| |_|\__,_|\__, |\___|_| |_| |_|\___|_| |_|\__| //

//                                                     |___/                                //

//////////////////////////////////////////////////////////////////////////////////////////////

logic        clk;

logic        clock_en;

logic        dbg_busy;

logic        sleeping;

// if we are sleeping on a barrier let's just wait on the instruction

// interface to finish loading instructions

assign core_busy_int = (data_load_event_ex & data_req_o) ? if_busy : (if_busy | ctrl_busy | lsu_busy);

always_ff @(posedge clk, negedge rst_ni)

begin

if (rst_ni == 1'b0) begin

core_busy_q <= 1'b0;

end else begin

core_busy_q <= core_busy_int;

end

end

assign core_busy_o = core_ctrl_firstfetch ? 1'b1 : core_busy_q;

assign dbg_busy    = dbg_req | dbg_csr_req | dbg_jump_req | dbg_reg_wreq | debug_req_i;

assign clock_en    = clock_en_i | core_busy_o | dbg_busy;

assign sleeping    = (~fetch_enable_i) & (~core_busy_o);

// main clock gate of the core

// generates all clocks except the one for the debug unit which is

// independent

cluster_clock_gating core_clock_gate_i

(

.clk_i     ( clk_i           ),

.en_i      ( clock_en        ),

.test_en_i ( test_en_i       ),

.clk_o     ( clk             )

);

//////////////////////////////////////////////////

//   ___ _____   ____ _____  _    ____ _____    //

//  |_ _|  ___| / ___|_   _|/ \  / ___| ____|   //

//   | || |_    \___ \ | | / _ \| |  _|  _|     //

//   | ||  _|    ___) || |/ ___ \ |_| | |___    //

//  |___|_|     |____/ |_/_/   \_\____|_____|   //

//                                              //

//////////////////////////////////////////////////

zeroriscy_if_stage if_stage_i

(

.clk                 ( clk               ),

.rst_n               ( rst_ni            ),

// boot address (trap vector location)

.boot_addr_i         ( boot_addr_i       ),

// instruction request control

.req_i               ( instr_req_int     ),

// instruction cache interface

.instr_req_o         ( instr_req_o       ),

.instr_addr_o        ( instr_addr_o      ),

.instr_gnt_i         ( instr_gnt_i       ),

.instr_rvalid_i      ( instr_rvalid_i    ),

.instr_rdata_i       ( instr_rdata_i     ),

// outputs to ID stage

.instr_valid_id_o    ( instr_valid_id    ),

.instr_rdata_id_o    ( instr_rdata_id    ),

.is_compressed_id_o  ( is_compressed_id  ),

.illegal_c_insn_id_o ( illegal_c_insn_id ),

.pc_if_o             ( pc_if             ),

.pc_id_o             ( pc_id             ),

// control signals

.clear_instr_valid_i ( clear_instr_valid ),

.pc_set_i            ( pc_set            ),

.exception_pc_reg_i  ( mepc              ), // exception return address

.pc_mux_i            ( pc_mux_id         ), // sel for pc multiplexer

.exc_pc_mux_i        ( exc_pc_mux_id     ),

.exc_vec_pc_mux_i    ( exc_cause[4:0]    ),

// from debug unit

.dbg_jump_addr_i     ( dbg_jump_addr     ),

// Jump targets

.jump_target_ex_i    ( jump_target_ex    ),

// pipeline stalls

.halt_if_i           ( halt_if           ),

.id_ready_i          ( id_ready          ),

.if_valid_o          ( if_valid          ),

.if_busy_o           ( if_busy           ),

.perf_imiss_o        ( perf_imiss        )

);

/////////////////////////////////////////////////

//   ___ ____    ____ _____  _    ____ _____   //

//  |_ _|  _ \  / ___|_   _|/ \  / ___| ____|  //

//   | || | | | \___ \ | | / _ \| |  _|  _|    //

//   | || |_| |  ___) || |/ ___ \ |_| | |___   //

//  |___|____/  |____/ |_/_/   \_\____|_____|  //

//                                             //

/////////////////////////////////////////////////

zeroriscy_id_stage

#(

.RV32E(RV32E),

.RV32M(RV32M)

)

id_stage_i

(

.clk                          ( clk                  ),

.rst_n                        ( rst_ni               ),

.test_en_i                    ( test_en_i            ),

// Processor Enable

.fetch_enable_i               ( fetch_enable_i       ),

.ctrl_busy_o                  ( ctrl_busy            ),

.core_ctrl_firstfetch_o       ( core_ctrl_firstfetch ),

.is_decoding_o                ( is_decoding          ),

// Interface to instruction memory

.instr_valid_i                ( instr_valid_id       ),

.instr_rdata_i                ( instr_rdata_id       ),

.instr_req_o                  ( instr_req_int        ),

// Jumps and branches

.branch_in_ex_o               ( branch_in_ex         ),

.branch_decision_i            ( branch_decision      ),

// IF and ID control signals

.clear_instr_valid_o          ( clear_instr_valid    ),

.pc_set_o                     ( pc_set               ),

.pc_mux_o                     ( pc_mux_id            ),

.exc_pc_mux_o                 ( exc_pc_mux_id        ),

.exc_cause_o                  ( exc_cause            ),

.illegal_c_insn_i             ( illegal_c_insn_id    ),

.is_compressed_i              ( is_compressed_id     ),

.pc_id_i                      ( pc_id                ),

// Stalls

.halt_if_o                    ( halt_if              ),

.id_ready_o                   ( id_ready             ),

.ex_ready_i                   ( ex_ready             ),

.id_valid_o                   ( id_valid             ),

.alu_operator_ex_o            ( alu_operator_ex      ),

.alu_operand_a_ex_o           ( alu_operand_a_ex     ),

.alu_operand_b_ex_o           ( alu_operand_b_ex     ),

.mult_en_ex_o                 ( mult_en_ex             ),

.div_en_ex_o                  ( div_en_ex              ),

.multdiv_operator_ex_o        ( multdiv_operator_ex    ),

.multdiv_signed_mode_ex_o     ( multdiv_signed_mode_ex ),

.multdiv_operand_a_ex_o       ( multdiv_operand_a_ex   ),

.multdiv_operand_b_ex_o       ( multdiv_operand_b_ex   ),

// CSR ID/EX

.csr_access_ex_o              ( csr_access_ex        ),

.csr_op_ex_o                  ( csr_op_ex            ),

.csr_cause_o                  ( csr_cause            ),

.csr_save_if_o                ( csr_save_if          ), // control signal to save pc

.csr_save_id_o                ( csr_save_id          ), // control signal to save pc

.csr_restore_mret_id_o        ( csr_restore_mret_id  ), // control signal to restore pc

.csr_save_cause_o             ( csr_save_cause       ),

// LSU

.data_req_ex_o                ( data_req_ex          ), // to load store unit

.data_we_ex_o                 ( data_we_ex           ), // to load store unit

.data_type_ex_o               ( data_type_ex         ), // to load store unit

.data_sign_ext_ex_o           ( data_sign_ext_ex     ), // to load store unit

.data_reg_offset_ex_o         ( data_reg_offset_ex   ), // to load store unit

.data_load_event_ex_o         ( data_load_event_ex   ), // to load store unit

.data_wdata_ex_o              ( data_wdata_ex        ), // to load store unit

.data_misaligned_i            ( data_misaligned      ),

.misaligned_addr_i            ( misaligned_addr      ),

// Interrupt Signals

.irq_i                        ( irq_i                ), // incoming interrupts

.irq_id_i                     ( irq_id_i             ),

.m_irq_enable_i               ( m_irq_enable         ),

.irq_ack_o                    ( irq_ack_o            ),

.irq_id_o                     ( irq_id_o             ),

.lsu_load_err_i               ( lsu_load_err         ),

.lsu_store_err_i              ( lsu_store_err        ),

// Debug Unit Signals

.dbg_settings_i               ( dbg_settings         ),

.dbg_req_i                    ( dbg_req              ),

.dbg_ack_o                    ( dbg_ack              ),

.dbg_stall_i                  ( dbg_stall            ),

.dbg_trap_o                   ( dbg_trap             ),

.dbg_reg_rreq_i               ( dbg_reg_rreq         ),

.dbg_reg_raddr_i              ( dbg_reg_raddr        ),

.dbg_reg_rdata_o              ( dbg_reg_rdata        ),

.dbg_reg_wreq_i               ( dbg_reg_wreq         ),

.dbg_reg_waddr_i              ( dbg_reg_waddr        ),

.dbg_reg_wdata_i              ( dbg_reg_wdata        ),

.dbg_jump_req_i               ( dbg_jump_req         ),

// write data to commit in the register file

.regfile_wdata_lsu_i          ( regfile_wdata_lsu    ),

.regfile_wdata_ex_i           ( regfile_wdata_ex     ),

.csr_rdata_i                  ( csr_rdata            ),

// Performance Counters

.perf_jump_o                  ( perf_jump            ),

.perf_branch_o                ( perf_branch          ),

.perf_tbranch_o               ( perf_tbranch         )

);

zeroriscy_ex_block

#(

//change the localparam MULT_TYPE to 0 or 1

//if you want a SLOW or FAST multiplier

.RV32M(RV32M)

)

ex_block_i

(

.clk                        ( clk                   ),

.rst_n                      ( rst_ni                ),

// Alu signals from ID stage

//TODO: hot encoding

.alu_operator_i             ( alu_operator_ex       ),

.multdiv_operator_i         ( multdiv_operator_ex   ),

.alu_operand_a_i            ( alu_operand_a_ex      ),

.alu_operand_b_i            ( alu_operand_b_ex      ),

// Multipler

.mult_en_i                  ( mult_en_ex            ),

.div_en_i                   ( div_en_ex             ),

.multdiv_signed_mode_i      ( multdiv_signed_mode_ex),

.multdiv_operand_a_i        ( multdiv_operand_a_ex  ),

.multdiv_operand_b_i        ( multdiv_operand_b_ex  ),

.alu_adder_result_ex_o      ( alu_adder_result_ex   ), // from ALU to LSU

.regfile_wdata_ex_o         ( regfile_wdata_ex      ),

// To IF: Jump and branch target and decision

.jump_target_o              ( jump_target_ex        ),

.branch_decision_o          ( branch_decision       ),

.lsu_en_i                   ( data_req_ex           ),

.lsu_ready_ex_i             ( data_valid_lsu        ),

.ex_ready_o                 ( ex_ready              )

);

////////////////////////////////////////////////////////////////////////////////////////

//    _     ___    _    ____    ____ _____ ___  ____  _____   _   _ _   _ ___ _____   //

//   | |   / _ \  / \  |  _ \  / ___|_   _/ _ \|  _ \| ____| | | | | \ | |_ _|_   _|  //

//   | |  | | | |/ _ \ | | | | \___ \ | || | | | |_) |  _|   | | | |  \| || |  | |    //

//   | |__| |_| / ___ \| |_| |  ___) || || |_| |  _ <| |___  | |_| | |\  || |  | |    //

//   |_____\___/_/   \_\____/  |____/ |_| \___/|_| \_\_____|  \___/|_| \_|___| |_|    //

//                                                                                    //

////////////////////////////////////////////////////////////////////////////////////////

zeroriscy_load_store_unit  load_store_unit_i

(

.clk                   ( clk                ),

.rst_n                 ( rst_ni             ),

//output to data memory

.data_req_o            ( data_req_o         ),

.data_gnt_i            ( data_gnt_i         ),

.data_rvalid_i         ( data_rvalid_i      ),

.data_err_i            ( data_err_i         ),

.data_addr_o           ( data_addr_o        ),

.data_we_o             ( data_we_o          ),

.data_be_o             ( data_be_o          ),

.data_wdata_o          ( data_wdata_o       ),

.data_rdata_i          ( data_rdata_i       ),

// signal from ex stage

.data_we_ex_i          ( data_we_ex         ),

.data_type_ex_i        ( data_type_ex       ),

.data_wdata_ex_i       ( data_wdata_ex      ),

.data_reg_offset_ex_i  ( data_reg_offset_ex ),

.data_sign_ext_ex_i    ( data_sign_ext_ex   ),

.data_rdata_ex_o       ( regfile_wdata_lsu  ),

.data_req_ex_i         ( data_req_ex        ),

.adder_result_ex_i     ( alu_adder_result_ex),

.data_misaligned_o     ( data_misaligned    ),

.misaligned_addr_o     ( misaligned_addr    ),

// exception signals

.load_err_o            ( lsu_load_err       ),

.store_err_o           ( lsu_store_err      ),

// control signals

.data_valid_o          ( data_valid_lsu     ),

.lsu_update_addr_o     (                    ),

.busy_o                ( lsu_busy           )

);

//////////////////////////////////////

//        ____ ____  ____           //

//       / ___/ ___||  _ \ ___      //

//      | |   \___ \| |_) / __|     //

//      | |___ ___) |  _ <\__ \     //

//       \____|____/|_| \_\___/     //

//                                  //

//   Control and Status Registers   //

//////////////////////////////////////

zeroriscy_cs_registers

#(

.N_EXT_CNT       ( N_EXT_PERF_COUNTERS   )

)

cs_registers_i

(

.clk                     ( clk                ),

.rst_n                   ( rst_ni             ),

// Core and Cluster ID from outside

.core_id_i               ( core_id_i          ),

.cluster_id_i            ( cluster_id_i       ),

// boot address

.boot_addr_i             ( boot_addr_i[31:8]  ),

// Interface to CSRs (SRAM like)

.csr_access_i            ( csr_access         ),

.csr_addr_i              ( csr_addr           ),

.csr_wdata_i             ( csr_wdata          ),

.csr_op_i                ( csr_op             ),

.csr_rdata_o             ( csr_rdata          ),

// Interrupt related control signals

.m_irq_enable_o          ( m_irq_enable       ),

.mepc_o                  ( mepc               ),

.pc_if_i                 ( pc_if              ),

.pc_id_i                 ( pc_id              ),

.csr_save_if_i           ( csr_save_if        ),

.csr_save_id_i           ( csr_save_id        ),

.csr_restore_mret_i      ( csr_restore_mret_id ),

.csr_cause_i             ( csr_cause          ),

.csr_save_cause_i        ( csr_save_cause     ),

// performance counter related signals

.if_valid_i              ( if_valid           ),

.id_valid_i              ( id_valid           ),

.is_compressed_i         ( is_compressed_id   ),

.is_decoding_i           ( is_decoding        ),

.imiss_i                 ( perf_imiss         ),

.pc_set_i                ( pc_set             ),

.jump_i                  ( perf_jump          ),

.branch_i                ( perf_branch        ),

.branch_taken_i          ( perf_tbranch       ),

.mem_load_i              ( data_req_o & data_gnt_i & (~data_we_o) ),

.mem_store_i             ( data_req_o & data_gnt_i & data_we_o    ),

.ext_counters_i          ( ext_perf_counters_i                    )

);

// Mux for CSR access through Debug Unit

assign csr_access   = (dbg_csr_req == 1'b0) ? csr_access_ex : 1'b1;

assign csr_addr     = (dbg_csr_req == 1'b0) ? csr_addr_int     : dbg_csr_addr;

assign csr_wdata    = (dbg_csr_req == 1'b0) ? alu_operand_a_ex : dbg_csr_wdata;

assign csr_op       = (dbg_csr_req == 1'b0) ? csr_op_ex

: (dbg_csr_we == 1'b1 ? CSR_OP_WRITE

: CSR_OP_NONE );

assign csr_addr_int = csr_access_ex ? alu_operand_b_ex[11:0] : '0;

/////////////////////////////////////////////////////////////

//  ____  _____ ____  _   _  ____   _   _ _   _ ___ _____  //

// |  _ \| ____| __ )| | | |/ ___| | | | | \ | |_ _|_   _| //

// | | | |  _| |  _ \| | | | |  _  | | | |  \| || |  | |   //

// | |_| | |___| |_) | |_| | |_| | | |_| | |\  || |  | |   //

// |____/|_____|____/ \___/ \____|  \___/|_| \_|___| |_|   //

//                                                         //

/////////////////////////////////////////////////////////////

zeroriscy_debug_unit debug_unit_i

(

.clk               ( clk_i              ), // always-running clock for debug

.rst_n             ( rst_ni             ),

// Debug Interface

.debug_req_i       ( debug_req_i        ),

.debug_gnt_o       ( debug_gnt_o        ),

.debug_rvalid_o    ( debug_rvalid_o     ),

.debug_addr_i      ( debug_addr_i       ),

.debug_we_i        ( debug_we_i         ),

.debug_wdata_i     ( debug_wdata_i      ),

.debug_rdata_o     ( debug_rdata_o      ),

.debug_halt_i      ( debug_halt_i       ),

.debug_resume_i    ( debug_resume_i     ),

.debug_halted_o    ( debug_halted_o     ),

// To/From Core

.settings_o        ( dbg_settings       ),

.trap_i            ( dbg_trap           ),

.exc_cause_i       ( exc_cause          ),

.stall_o           ( dbg_stall          ),

.dbg_req_o         ( dbg_req            ),

.dbg_ack_i         ( dbg_ack            ),

// register file read port

.regfile_rreq_o    ( dbg_reg_rreq       ),

.regfile_raddr_o   ( dbg_reg_raddr      ),

.regfile_rdata_i   ( dbg_reg_rdata      ),

// register file write port

.regfile_wreq_o    ( dbg_reg_wreq       ),

.regfile_waddr_o   ( dbg_reg_waddr      ),

.regfile_wdata_o   ( dbg_reg_wdata      ),

// CSR read/write port

.csr_req_o         ( dbg_csr_req        ),

.csr_addr_o        ( dbg_csr_addr       ),

.csr_we_o          ( dbg_csr_we         ),

.csr_wdata_o       ( dbg_csr_wdata      ),

.csr_rdata_i       ( csr_rdata          ),

// signals for PPC and NPC

.pc_if_i           ( pc_if              ), // from IF stage

.pc_id_i           ( pc_id              ), // from ID stage

.instr_valid_id_i  ( instr_valid_id     ),

.sleeping_i        ( sleeping           ),

.jump_addr_o       ( dbg_jump_addr      ), // PC from debug unit

.jump_req_o        ( dbg_jump_req       )  // set PC to new value

);

`ifndef VERILATOR

`ifdef TRACE_EXECUTION

zeroriscy_tracer zeroriscy_tracer_i

(

.clk            ( clk_i                                ), // always-running clock for tracing

.rst_n          ( rst_ni                               ),

.fetch_enable   ( fetch_enable_i                       ),

.core_id        ( core_id_i                            ),

.cluster_id     ( cluster_id_i                         ),

.pc             ( id_stage_i.pc_id_i                   ),

.instr          ( id_stage_i.instr                     ),

.compressed     ( id_stage_i.is_compressed_i           ),

.id_valid       ( id_stage_i.id_valid_o                ),

.is_decoding    ( id_stage_i.is_decoding_o             ),

.is_branch      ( id_stage_i.branch_in_id              ),

.branch_taken   ( id_stage_i.branch_set_q              ),

.pipe_flush     ( id_stage_i.controller_i.pipe_flush_i ),

.mret_insn      ( id_stage_i.controller_i.mret_insn_i  ),

.ecall_insn     ( id_stage_i.controller_i.ecall_insn_i ),

.ebrk_insn      ( id_stage_i.controller_i.ebrk_insn_i  ),

.csr_status     ( id_stage_i.controller_i.csr_status_i ),

.rs1_value      ( id_stage_i.operand_a_fw_id           ),

.rs2_value      ( id_stage_i.operand_b_fw_id           ),

.lsu_value      ( data_wdata_ex                        ),

.ex_reg_addr    ( id_stage_i.regfile_waddr_mux         ),

.ex_reg_we      ( id_stage_i.regfile_we_mux            ),

.ex_reg_wdata   ( id_stage_i.regfile_wdata_mux         ),

.data_valid_lsu ( data_valid_lsu                       ),

.ex_data_addr   ( data_addr_o                          ),

.ex_data_req    ( data_req_o                           ),

.ex_data_gnt    ( data_gnt_i                           ),

.ex_data_we     ( data_we_o                            ),

.ex_data_wdata  ( data_wdata_o                         ),

.lsu_reg_wdata  ( regfile_wdata_lsu                    ),

.imm_u_type     ( id_stage_i.imm_u_type                ),

.imm_uj_type    ( id_stage_i.imm_uj_type               ),

.imm_i_type     ( id_stage_i.imm_i_type                ),

.imm_iz_type    ( id_stage_i.imm_iz_type[11:0]         ),

.imm_z_type     ( id_stage_i.imm_z_type                ),

.imm_s_type     ( id_stage_i.imm_s_type                ),

.imm_sb_type    ( id_stage_i.imm_sb_type               )

);

`endif

`endif

Endmodule

`include "zeroriscy_config.sv"

import zeroriscy_defines::*;

`ifndef PULP_FPGA_EMUL

`ifdef SYNTHESIS

`define ASIC_SYNTHESIS

`endif

`endif

module zeroriscy_cs_registers

#(

parameter N_EXT_CNT    = 0

)

(

// Clock and Reset

input  logic        clk,

input  logic        rst_n,

// Core and Cluster ID

input  logic  [3:0] core_id_i,

input  logic  [5:0] cluster_id_i,

// Used for boot address

input  logic [23:0] boot_addr_i,

// Interface to registers (SRAM like)

input  logic        csr_access_i,

input  logic [11:0] csr_addr_i,

input  logic [31:0] csr_wdata_i,

input  logic  [1:0] csr_op_i,

output logic [31:0] csr_rdata_o,

// Interrupts

output logic        m_irq_enable_o,

output logic [31:0] mepc_o,

input  logic [31:0] pc_if_i,

input  logic [31:0] pc_id_i,

input  logic        csr_save_if_i,

input  logic        csr_save_id_i,

input  logic        csr_restore_mret_i,

input  logic [5:0]  csr_cause_i,

input  logic        csr_save_cause_i,

// Performance Counters

input  logic                 if_valid_i,        // IF stage gives a new instruction

input  logic                 id_valid_i,        // ID stage is done

input  logic                 is_compressed_i,   // compressed instruction in ID

input  logic                 is_decoding_i,     // controller is in DECODE state

input  logic                 imiss_i,           // instruction fetch

input  logic                 pc_set_i,          // pc was set to a new value

input  logic                 jump_i,            // jump instruction seen   (j, jr, jal, jalr)

input  logic                 branch_i,          // branch instruction seen (bf, bnf)

input  logic                 branch_taken_i,    // branch was taken

input  logic                 mem_load_i,        // load from memory in this cycle

input  logic                 mem_store_i,       // store to memory in this cycle

input  logic [N_EXT_CNT-1:0] ext_counters_i

);

localparam N_PERF_COUNTERS = 11 + N_EXT_CNT;

`ifdef ASIC_SYNTHESIS

localparam N_PERF_REGS     = 1;

`else

localparam N_PERF_REGS     = N_PERF_COUNTERS;

`endif

`define MSTATUS_UIE_BITS        0

`define MSTATUS_SIE_BITS        1

`define MSTATUS_MIE_BITS        3

`define MSTATUS_UPIE_BITS       4

`define MSTATUS_SPIE_BITS       5

`define MSTATUS_MPIE_BITS       7

`define MSTATUS_SPP_BITS        8

`define MSTATUS_MPP_BITS    12:11

typedef struct packed {

//logic uie;       - unimplemented, hardwired to '0

// logic sie;      - unimplemented, hardwired to '0

// logic hie;      - unimplemented, hardwired to '0

logic mie;

//logic upie;     - unimplemented, hardwired to '0

// logic spie;     - unimplemented, hardwired to '0

// logic hpie;     - unimplemented, hardwired to '0

logic mpie;

// logic spp;      - unimplemented, hardwired to '0

// logic[1:0] hpp; - unimplemented, hardwired to '0

PrivLvl_t mpp;

} Status_t;

// Performance Counter Signals

logic                          id_valid_q;

logic [N_PERF_COUNTERS-1:0]    PCCR_in;  // input signals for each counter category

logic [N_PERF_COUNTERS-1:0]    PCCR_inc, PCCR_inc_q; // should the counter be increased?

logic [N_PERF_REGS-1:0] [31:0] PCCR_q, PCCR_n; // performance counters counter register

logic [1:0]                    PCMR_n, PCMR_q; // mode register, controls saturation and global enable

logic [N_PERF_COUNTERS-1:0]    PCER_n, PCER_q; // selected counter input

logic [31:0]                   perf_rdata;

logic [4:0]                    pccr_index;

logic                          pccr_all_sel;

logic                          is_pccr;

logic                          is_pcer;

logic                          is_pcmr;

// CSR update logic

logic [31:0] csr_wdata_int;

logic [31:0] csr_rdata_int;

logic        csr_we_int;

// Interrupt control signals

logic [31:0] mepc_q, mepc_n;

logic [ 5:0] mcause_q, mcause_n;

Status_t mstatus_q, mstatus_n;

////////////////////////////////////////////

//   ____ ____  ____    ____              //

//  / ___/ ___||  _ \  |  _ \ ___  __ _   //

// | |   \___ \| |_) | | |_) / _ \/ _` |  //

// | |___ ___) |  _ <  |  _ <  __/ (_| |  //

//  \____|____/|_| \_\ |_| \_\___|\__, |  //

//                                |___/   //

////////////////////////////////////////////

// read logic

always_comb

begin

csr_rdata_int = '0;

case (csr_addr_i)

// mstatus: always M-mode, contains IE bit

12'h300: csr_rdata_int = {

19'b0,

mstatus_q.mpp,

3'b0,

mstatus_q.mpie,

3'h0,

mstatus_q.mie,

3'h0

};

// mtvec: machine trap-handler base address

12'h305: csr_rdata_int = {boot_addr_i, 8'h0};

// mepc: exception program counter

12'h341: csr_rdata_int = mepc_q;

// mcause: exception cause

12'h342: csr_rdata_int = {mcause_q[5], 26'b0, mcause_q[4:0]};

// mhartid: unique hardware thread id

12'hF14: csr_rdata_int = {21'b0, cluster_id_i[5:0], 1'b0, core_id_i[3:0]};

default: ;

endcase

end

// write logic

always_comb

begin

mepc_n       = mepc_q;

mstatus_n    = mstatus_q;

mcause_n     = mcause_q;

case (csr_addr_i)

// mstatus: IE bit

12'h300: if (csr_we_int) begin

mstatus_n = '{

mie:  csr_wdata_int[`MSTATUS_MIE_BITS],

mpie: csr_wdata_int[`MSTATUS_MPIE_BITS],

mpp:  PrivLvl_t'(PRIV_LVL_M)

};

end

// mepc: exception program counter

12'h341: if (csr_we_int) mepc_n = csr_wdata_int;

// mcause

12'h342: if (csr_we_int) mcause_n = {csr_wdata_int[31], csr_wdata_int[4:0]};

default: ;

endcase

// exception controller gets priority over other writes

unique case (1'b1)

csr_save_cause_i: begin

unique case (1'b1)

csr_save_if_i:

mepc_n = pc_if_i;

csr_save_id_i:

mepc_n = pc_id_i;

default:;

endcase

mstatus_n.mpie = mstatus_q.mie;

mstatus_n.mie  = 1'b0;

mcause_n       = csr_cause_i;

end //csr_save_cause_i

csr_restore_mret_i: begin //MRET

mstatus_n.mie  = mstatus_q.mpie;

mstatus_n.mpie = 1'b1;

end //csr_restore_mret_i

default:;

endcase

end

// CSR operation logic

always_comb

begin

csr_wdata_int = csr_wdata_i;

csr_we_int    = 1'b1;

unique case (csr_op_i)

CSR_OP_WRITE: csr_wdata_int = csr_wdata_i;

CSR_OP_SET:   csr_wdata_int = csr_wdata_i | csr_rdata_o;

CSR_OP_CLEAR: csr_wdata_int = (~csr_wdata_i) & csr_rdata_o;

CSR_OP_NONE: begin

csr_wdata_int = csr_wdata_i;

csr_we_int    = 1'b0;

end

default:;

endcase

end

// output mux

always_comb

begin

csr_rdata_o = csr_rdata_int;

// performance counters

if (is_pccr || is_pcer || is_pcmr)

csr_rdata_o = perf_rdata;

end

// directly output some registers

assign m_irq_enable_o  = mstatus_q.mie;

assign mepc_o          = mepc_q;

// actual registers

always_ff @(posedge clk, negedge rst_n)

begin

if (rst_n == 1'b0)

begin

mstatus_q  <= '{

mie:  1'b0,

mpie: 1'b0,

mpp:  PRIV_LVL_M

};

mepc_q     <= '0;

mcause_q   <= '0;

end

else

begin

// update CSRs

mstatus_q  <= '{

mie:  mstatus_n.mie,

mpie: mstatus_n.mpie,

mpp:  PRIV_LVL_M

};

mepc_q     <= mepc_n;

mcause_q   <= mcause_n;

end

end

/////////////////////////////////////////////////////////////////

//   ____            __     ____                  _            //

// |  _ \ ___ _ __ / _|   / ___|___  _   _ _ __ | |_ ___ _ __  //

// | |_) / _ \ '__| |_   | |   / _ \| | | | '_ \| __/ _ \ '__| //

// |  __/  __/ |  |  _|  | |__| (_) | |_| | | | | ||  __/ |    //

// |_|   \___|_|  |_|(_)  \____\___/ \__,_|_| |_|\__\___|_|    //

//                                                             //

/////////////////////////////////////////////////////////////////

assign PCCR_in[0]  = 1'b1;                          // cycle counter

assign PCCR_in[1]  = if_valid_i;                    // instruction counter

assign PCCR_in[2]  = 1'b0;                          // Reserved

assign PCCR_in[3]  = 1'b0;                          // Reserved

assign PCCR_in[4]  = imiss_i & (~pc_set_i);         // cycles waiting for instruction fetches, excluding jumps and branches

assign PCCR_in[5]  = mem_load_i;                    // nr of loads

assign PCCR_in[6]  = mem_store_i;                   // nr of stores

assign PCCR_in[7]  = jump_i;                        // nr of jumps (unconditional)

assign PCCR_in[8]  = branch_i;                      // nr of branches (conditional)

assign PCCR_in[9]  = branch_taken_i;                // nr of taken branches (conditional)

assign PCCR_in[10] = id_valid_i & is_decoding_i & is_compressed_i; // compressed instruction counter

// assign external performance counters

generate

genvar i;

for (i = 0; i < N_EXT_CNT; i++)

begin : g_extcounters

assign PCCR_in[N_PERF_COUNTERS - N_EXT_CNT + i] = ext_counters_i[i];

end

endgenerate

// address decoder for performance counter registers

always_comb

begin

is_pccr      = 1'b0;

is_pcmr      = 1'b0;

is_pcer      = 1'b0;

pccr_all_sel = 1'b0;

pccr_index   = '0;

perf_rdata   = '0;

// only perform csr access if we actually care about the read data

if (csr_access_i) begin

unique case (csr_addr_i)

12'h7A0: begin

is_pcer = 1'b1;

perf_rdata[15:0] = PCER_q;

end

12'h7A1: begin

is_pcmr = 1'b1;

perf_rdata[1:0] = PCMR_q;

end

12'h79F: begin

is_pccr = 1'b1;

pccr_all_sel = 1'b1;

end

default:;

endcase

// look for 780 to 79F, Performance Counter Counter Registers

if (csr_addr_i[11:5] == 7'b0111100) begin

is_pccr     = 1'b1;

pccr_index = csr_addr_i[4:0];

`ifdef  ASIC_SYNTHESIS

perf_rdata = PCCR_q[0];

`else

perf_rdata = PCCR_q[csr_addr_i[4:0]];

`endif

end

end

end

// performance counter counter update logic

`ifdef ASIC_SYNTHESIS

// for synthesis we just have one performance counter register

assign PCCR_inc[0] = (|(PCCR_in & PCER_q)) & PCMR_q[0];

always_comb

begin

PCCR_n[0]   = PCCR_q[0];

if ((PCCR_inc_q[0] == 1'b1) && ((PCCR_q[0] != 32'hFFFFFFFF) || (PCMR_q[1] == 1'b0)))

PCCR_n[0] = PCCR_q[0] + 1;

if (is_pccr == 1'b1) begin

unique case (csr_op_i)

CSR_OP_NONE:   ;

CSR_OP_WRITE:  PCCR_n[0] = csr_wdata_i;

CSR_OP_SET:    PCCR_n[0] = csr_wdata_i | PCCR_q[0];

CSR_OP_CLEAR:  PCCR_n[0] = csr_wdata_i & ~(PCCR_q[0]);

endcase

end

end

`else

always_comb

begin

for(int i = 0; i < N_PERF_COUNTERS; i++)

begin : PERF_CNT_INC

PCCR_inc[i] = PCCR_in[i] & PCER_q[i] & PCMR_q[0];

PCCR_n[i]   = PCCR_q[i];

if ((PCCR_inc_q[i] == 1'b1) && ((PCCR_q[i] != 32'hFFFFFFFF) || (PCMR_q[1] == 1'b0)))

PCCR_n[i] = PCCR_q[i] + 1;

if (is_pccr == 1'b1 && (pccr_all_sel == 1'b1 || pccr_index == i)) begin

unique case (csr_op_i)

CSR_OP_NONE:   ;

CSR_OP_WRITE:  PCCR_n[i] = csr_wdata_i;

CSR_OP_SET:    PCCR_n[i] = csr_wdata_i | PCCR_q[i];

CSR_OP_CLEAR:  PCCR_n[i] = csr_wdata_i & ~(PCCR_q[i]);

endcase

end

end

end

`endif

// update PCMR and PCER

always_comb

begin

PCMR_n = PCMR_q;

PCER_n = PCER_q;

if (is_pcmr) begin

unique case (csr_op_i)

CSR_OP_NONE:   ;

CSR_OP_WRITE:  PCMR_n = csr_wdata_i[1:0];

CSR_OP_SET:    PCMR_n = csr_wdata_i[1:0] | PCMR_q;

CSR_OP_CLEAR:  PCMR_n = csr_wdata_i[1:0] & ~(PCMR_q);

endcase

end

if (is_pcer) begin

unique case (csr_op_i)

CSR_OP_NONE:   ;

CSR_OP_WRITE:  PCER_n = csr_wdata_i[N_PERF_COUNTERS-1:0];

CSR_OP_SET:    PCER_n = csr_wdata_i[N_PERF_COUNTERS-1:0] | PCER_q;

CSR_OP_CLEAR:  PCER_n = csr_wdata_i[N_PERF_COUNTERS-1:0] & ~(PCER_q);

endcase

end

end

// Performance Counter Registers

always_ff @(posedge clk, negedge rst_n)

begin

if (rst_n == 1'b0)

begin

id_valid_q <= 1'b0;

PCER_q <= '0;

PCMR_q <= 2'h3;

for(int i = 0; i < N_PERF_REGS; i++)

begin

PCCR_q[i]     <= '0;

PCCR_inc_q[i] <= '0;

end

end

else

begin

id_valid_q <= id_valid_i;

PCER_q <= PCER_n;

PCMR_q <= PCMR_n;

for(int i = 0; i < N_PERF_REGS; i++)

begin

PCCR_q[i]     <= PCCR_n[i];

PCCR_inc_q[i] <= PCCR_inc[i];

end

end

end

Endmodule

`include "zeroriscy_config.sv"

import zeroriscy_defines::*;

module zeroriscy_debug_unit

#(

parameter REG_ADDR_WIDTH      = 5

)

(

input logic         clk,

input logic         rst_n,

// Debug Interface

input  logic        debug_req_i,

output logic        debug_gnt_o,

output logic        debug_rvalid_o,

input  logic [14:0] debug_addr_i,

input  logic        debug_we_i,

input  logic [31:0] debug_wdata_i,

output logic [31:0] debug_rdata_o,

output logic        debug_halted_o,

input  logic        debug_halt_i,

input  logic        debug_resume_i,

// signals to core

output logic [DBG_SETS_W-1:0] settings_o,

input  logic        trap_i,      // trap found, need to stop the core now

input  logic [5:0]  exc_cause_i, // if it was a trap, then the exception controller knows more

output logic        stall_o,     // after we got control, we control the stall signal

output logic        dbg_req_o,

input  logic        dbg_ack_i,

// register file read port

output logic        regfile_rreq_o,

output logic [(REG_ADDR_WIDTH-1):0] regfile_raddr_o,

input  logic [31:0] regfile_rdata_i,

// register file write port

output logic        regfile_wreq_o,

output logic [(REG_ADDR_WIDTH-1):0] regfile_waddr_o,

output logic [31:0] regfile_wdata_o,

// CSR read/write port

output logic        csr_req_o,

output logic [11:0] csr_addr_o,

output logic        csr_we_o,

output logic [31:0] csr_wdata_o,

input  logic [31:0] csr_rdata_i,

// Signals for PPC & NPC register

input  logic [31:0] pc_if_i,

input  logic [31:0] pc_id_i,

input  logic        instr_valid_id_i,

input  logic        sleeping_i,

output logic        jump_req_o,

output logic [31:0] jump_addr_o

);

enum logic [2:0] {RD_NONE, RD_CSR, RD_GPR, RD_DBGA, RD_DBGS} rdata_sel_q, rdata_sel_n;

enum logic [0:0] {FIRST, SECOND} state_q, state_n;

logic [DBG_SETS_W-1:0] settings_q, settings_n;

// for timing critical register file access we need to keep those in FFs

logic [14:0] addr_q;

logic [31:0] wdata_q; // mainly for jumps

logic        regfile_rreq_q, regfile_rreq_n;

logic        jump_req_q, jump_req_n;

// not timing critical

logic        csr_req_q, csr_req_n;

logic        regfile_wreq;

enum logic [1:0] {RUNNING, HALT_REQ, HALT} stall_cs, stall_ns;

logic [31:0] dbg_rdata;

logic        dbg_resume;

logic        dbg_halt;

logic [5:0]  dbg_cause_q, dbg_cause_n;

logic        dbg_ssth_q,  dbg_ssth_n;

logic        ssth_clear;

// ppc/npc tracking

logic [31:0] ppc_int, npc_int;

// address decoding, write and read controller

always_comb

begin

rdata_sel_n    = RD_NONE;

state_n        = FIRST;

debug_gnt_o    = 1'b0;

regfile_rreq_n = 1'b0;

regfile_wreq   = 1'b0;

csr_req_n      = 1'b0;

csr_we_o       = 1'b0;

jump_req_n     = 1'b0;

dbg_resume     = 1'b0;

dbg_halt       = 1'b0;

settings_n     = settings_q;

ssth_clear     = 1'b0;

if (debug_req_i) begin

if (debug_we_i) begin

//----------------------------------------------------------------------------

// write access

//----------------------------------------------------------------------------

if (debug_addr_i[14]) begin

// CSR access

if (state_q == FIRST) begin

// only grant in second cycle, address and data have been latched by then

debug_gnt_o = 1'b0;

state_n     = SECOND;

if (debug_halted_o) begin

// access to internal registers are only allowed when the core is halted

csr_req_n = 1'b1;

end

end else begin

debug_gnt_o = 1'b1; // grant it even when invalid access to not block

state_n     = FIRST;

csr_we_o    = 1'b1;

end

end else begin

// non-CSR access

unique case (debug_addr_i[13:8])

6'b00_0000: begin // Debug Registers, always accessible

debug_gnt_o = 1'b1;

unique case (debug_addr_i[6:2])

5'b0_0000: begin // DBG_CTRL

if (debug_wdata_i[16]) begin

// HALT set

if (~debug_halted_o) begin

// not halt, so STOP

dbg_halt = 1'b1;

end

end else begin

// RESUME set

if (debug_halted_o) begin

dbg_resume = 1'b1;

end

end

settings_n[DBG_SETS_SSTE] = debug_wdata_i[0];

end

5'b0_0001: begin // DBG_HIT

ssth_clear = debug_wdata_i[0];

end

5'b0_0010: begin // DBG_IE

settings_n[DBG_SETS_ECALL] = debug_wdata_i[11];

settings_n[DBG_SETS_ELSU]  = debug_wdata_i[7] | debug_wdata_i[5];

settings_n[DBG_SETS_EBRK]  = debug_wdata_i[3];

settings_n[DBG_SETS_EILL]  = debug_wdata_i[2];

end

default:;

endcase

end

6'b10_0000: begin // Debug Registers, only accessible when in debug

debug_gnt_o = 1'b1; // grant it even when invalid access to not block

if (debug_halted_o) begin

unique case (debug_addr_i[6:2])

5'b0_0000: jump_req_n = 1'b1; // DNPC

default:;

endcase

end

end

6'b00_0100: begin // General-Purpose Registers

debug_gnt_o = 1'b1; // grant it even when invalid access to not block

if (debug_halted_o) begin

regfile_wreq = 1'b1;

end

end

default: debug_gnt_o = 1'b1; // grant it even when invalid access to not block

endcase

end

end else begin

//----------------------------------------------------------------------------

// read access

//----------------------------------------------------------------------------

if (debug_addr_i[14]) begin

debug_gnt_o = 1'b1; // grant it even when invalid access to not block

// CSR access

if (debug_halted_o) begin

// access to internal registers are only allowed when the core is halted

csr_req_n   = 1'b1;

rdata_sel_n = RD_CSR;

end

end else begin

// non-CSR access

unique case (debug_addr_i[13:8])

6'b00_0000: begin // Debug Registers, always accessible

debug_gnt_o = 1'b1;

rdata_sel_n = RD_DBGA;

end

6'b10_0000: begin // Debug Registers, only accessible when in debug

debug_gnt_o = 1'b1; // grant it even when invalid access to not block

rdata_sel_n = RD_DBGS;

end

6'b00_0100: begin // General-Purpose Registers

debug_gnt_o = 1'b1; // grant it even when invalid access to not block

if (debug_halted_o) begin

regfile_rreq_n = 1'b1;

rdata_sel_n    = RD_GPR;

end

end

default: debug_gnt_o = 1'b1; // grant it even when invalid access to not block

endcase

end

end

end

end

//----------------------------------------------------------------------------

// debug register read access

//

// Since those are combinational, we can do it in the cycle where we set

// rvalid. The address has been latched into addr_q

//----------------------------------------------------------------------------

always_comb

begin

dbg_rdata = '0;

case (rdata_sel_q)

RD_DBGA: begin

unique case (addr_q[6:2])

5'h00: dbg_rdata[31:0] = {15'b0, debug_halted_o, 15'b0, settings_q[DBG_SETS_SSTE]}; // DBG_CTRL

5'h01: dbg_rdata[31:0] = {15'b0, sleeping_i, 15'b0, dbg_ssth_q}; // DBG_HIT

5'h02: begin // DBG_IE

dbg_rdata[31:16] = '0;

dbg_rdata[15:12] = '0;

dbg_rdata[11]    = settings_q[DBG_SETS_ECALL];

dbg_rdata[10: 8] = '0;

dbg_rdata[ 7]    = settings_q[DBG_SETS_ELSU];

dbg_rdata[ 6]    = 1'b0;

dbg_rdata[ 5]    = settings_q[DBG_SETS_ELSU];

dbg_rdata[ 4]    = 1'b0;

dbg_rdata[ 3]    = settings_q[DBG_SETS_EBRK];

dbg_rdata[ 2]    = settings_q[DBG_SETS_EILL];

dbg_rdata[ 1: 0] = '0;

end

5'h03: dbg_rdata = {dbg_cause_q[5], 26'b0, dbg_cause_q[4:0]}; // DBG_CAUSE

5'h10: dbg_rdata = '0; // DBG_BPCTRL0

5'h12: dbg_rdata = '0; // DBG_BPCTRL1

5'h14: dbg_rdata = '0; // DBG_BPCTRL2

5'h16: dbg_rdata = '0; // DBG_BPCTRL3

5'h18: dbg_rdata = '0; // DBG_BPCTRL4

5'h1A: dbg_rdata = '0; // DBG_BPCTRL5

5'h1C: dbg_rdata = '0; // DBG_BPCTRL6

5'h1E: dbg_rdata = '0; // DBG_BPCTRL7

default:;

endcase

end

RD_DBGS: begin

unique case (addr_q[2:2])

1'b0: dbg_rdata = npc_int; // DBG_NPC

1'b1: dbg_rdata = ppc_int; // DBG_PPC

default:;

endcase

end

default:;

endcase

end

//----------------------------------------------------------------------------

// read data mux

//----------------------------------------------------------------------------

always_comb

begin

debug_rdata_o = '0;

case (rdata_sel_q)

RD_CSR:  debug_rdata_o = csr_rdata_i;

RD_GPR:  debug_rdata_o = regfile_rdata_i;

RD_DBGA: debug_rdata_o = dbg_rdata;

RD_DBGS: debug_rdata_o = dbg_rdata;

default: ;

endcase

end

//----------------------------------------------------------------------------

// rvalid generation

//----------------------------------------------------------------------------

always_ff @(posedge clk, negedge rst_n)

begin

if (~rst_n) begin

debug_rvalid_o <= 1'b0;

end else begin

debug_rvalid_o <= debug_gnt_o; // always give the rvalid one cycle after gnt

end

end

//----------------------------------------------------------------------------

// stall control

//----------------------------------------------------------------------------

always_comb

begin

stall_ns       = stall_cs;

dbg_req_o      = 1'b0;

stall_o        = 1'b0;

debug_halted_o = 1'b0;

dbg_cause_n    = dbg_cause_q;

dbg_ssth_n     = dbg_ssth_q;

case (stall_cs)

RUNNING: begin

dbg_ssth_n = 1'b0;

if (dbg_halt | debug_halt_i | trap_i) begin

dbg_req_o   = 1'b1;

stall_ns    = HALT_REQ;

if (trap_i) begin

if (settings_q[DBG_SETS_SSTE])

dbg_ssth_n = 1'b1;

dbg_cause_n = exc_cause_i;

end else begin

dbg_cause_n = DBG_CAUSE_HALT;

end

end

end

HALT_REQ: begin

dbg_req_o = 1'b1;

if (dbg_ack_i)

stall_ns = HALT;

if (dbg_resume | debug_resume_i)

stall_ns = RUNNING;

end

HALT: begin

stall_o        = 1'b1;

debug_halted_o = 1'b1;

if (dbg_resume | debug_resume_i) begin

stall_ns = RUNNING;

stall_o  = 1'b0;

end

end

endcase

if (ssth_clear)

dbg_ssth_n = 1'b0;

end

always_ff @(posedge clk, negedge rst_n)

begin

if (~rst_n) begin

stall_cs    <= RUNNING;

dbg_cause_q <= DBG_CAUSE_HALT;

dbg_ssth_q  <= 1'b0;

end else begin

stall_cs    <= stall_ns;

dbg_cause_q <= dbg_cause_n;

dbg_ssth_q  <= dbg_ssth_n;

end

end

//----------------------------------------------------------------------------

// NPC/PPC selection

//----------------------------------------------------------------------------

assign ppc_int = pc_id_i;

assign npc_int = pc_if_i;

always_ff @(posedge clk, negedge rst_n)

begin

if (~rst_n) begin

addr_q             <= '0;

wdata_q            <= '0;

state_q            <= FIRST;

rdata_sel_q        <= RD_NONE;

regfile_rreq_q     <= 1'b0;

csr_req_q          <= 1'b0;

jump_req_q         <= 1'b0;

settings_q         <= 1'b0;

end else begin

// settings

settings_q         <= settings_n;

// for the actual interface

if (debug_req_i) begin

addr_q  <= debug_addr_i;

wdata_q <= debug_wdata_i;

state_q <= state_n;

end

if (debug_req_i | debug_rvalid_o) begin

// wait for either req or rvalid to set those FFs

// This makes sure that they are only triggered once when there is

// only one request, and the FFs can be properly clock gated otherwise

regfile_rreq_q <= regfile_rreq_n;

csr_req_q      <= csr_req_n;

jump_req_q     <= jump_req_n;

rdata_sel_q    <= rdata_sel_n;

end

end

end

assign regfile_rreq_o  = regfile_rreq_q;

assign regfile_raddr_o = addr_q[6:2];

assign regfile_wreq_o  = regfile_wreq;

assign regfile_waddr_o = debug_addr_i[6:2];

assign regfile_wdata_o = debug_wdata_i;

assign csr_req_o       = csr_req_q;

assign csr_addr_o      = addr_q[13:2];

assign csr_wdata_o     = wdata_q;

assign jump_req_o      = jump_req_q;

assign jump_addr_o     = wdata_q;

assign settings_o      = settings_q;

//----------------------------------------------------------------------------

// Assertions

//----------------------------------------------------------------------------

`ifndef VERILATOR

// check that no registers are accessed when we are not in debug mode

assert property (

@(posedge clk) (debug_req_i) |-> ((debug_halted_o == 1'b1) ||

((debug_addr_i[14] != 1'b1) &&

(debug_addr_i[13:7] != 5'b0_1001)  &&

(debug_addr_i[13:7] != 5'b0_1000)) ) )

else $warning("Trying to access internal debug registers while core is not stalled");

// check that all accesses are word-aligned

assert property (

@(posedge clk) (debug_req_i) |-> (debug_addr_i[1:0] == 2'b00) );

`endif

endmodule // debug_unit

`include "zeroriscy_config.sv"

import zeroriscy_defines::*;

module zeroriscy_decoder

#(

parameter RV32M      = 1

)

(

// singals running to/from controller

input  logic        deassert_we_i,           // deassert we, we are stalled or not active

input  logic        data_misaligned_i,       // misaligned data load/store in progress

input  logic        branch_mux_i,

input  logic        jump_mux_i,

output logic        illegal_insn_o,          // illegal instruction encountered

output logic        ebrk_insn_o,             // trap instruction encountered

output logic        mret_insn_o,             // return from exception instruction encountered

output logic        ecall_insn_o,            // environment call (syscall) instruction encountered

output logic        pipe_flush_o,            // pipeline flush is requested

// from IF/ID pipeline

input  logic [31:0] instr_rdata_i,           // instruction read from instr memory/cache

input  logic        illegal_c_insn_i,        // compressed instruction decode failed

// ALU signals

output logic [ALU_OP_WIDTH-1:0] alu_operator_o, // ALU operation selection

output logic [2:0]  alu_op_a_mux_sel_o,      // operand a selection: reg value, PC, immediate or zero

output logic [2:0]  alu_op_b_mux_sel_o,      // oNOperand b selection: reg value or immediate

output logic [0:0]  imm_a_mux_sel_o,         // immediate selection for operand a

output logic [3:0]  imm_b_mux_sel_o,         // immediate selection for operand b

// MUL, DIV related control signals

output logic        mult_int_en_o,          // perform integer multiplication

output logic        div_int_en_o,           // perform integer division or reminder

output logic [1:0]  multdiv_operator_o,

output logic [1:0]  multdiv_signed_mode_o,

// register file related signals

output logic        regfile_we_o,            // write enable for regfile

// CSR manipulation

output logic        csr_access_o,            // access to CSR

output logic [1:0]  csr_op_o,                // operation to perform on CSR

output logic        csr_status_o,            // access to xstatus CSR

// LD/ST unit signals

output logic        data_req_o,              // start transaction to data memory

output logic        data_we_o,               // data memory write enable

output logic [1:0]  data_type_o,             // data type on data memory: byte, half word or word

output logic        data_sign_extension_o,   // sign extension on read data from data memory

output logic [1:0]  data_reg_offset_o,       // offset in byte inside register for stores

output logic        data_load_event_o,       // data request is in the special event range

// jump/branches

output logic        jump_in_id_o,            // jump is being calculated in ALU

output logic        branch_in_id_o

);

// write enable/request control

logic       regfile_we;

logic       data_req;

logic       mult_int_en;

logic       div_int_en;

logic       branch_in_id;

logic       jump_in_id;

logic [1:0] csr_op;

logic       csr_illegal;

/////////////////////////////////////////////

//   ____                     _            //

//  |  _ \  ___  ___ ___   __| | ___ _ __  //

//  | | | |/ _ \/ __/ _ \ / _` |/ _ \ '__| //

//  | |_| |  __/ (_| (_) | (_| |  __/ |    //

//  |____/ \___|\___\___/ \__,_|\___|_|    //

//                                         //

/////////////////////////////////////////////

always_comb

begin

jump_in_id                  = 1'b0;

branch_in_id                = 1'b0;

alu_operator_o              = ALU_SLTU;

alu_op_a_mux_sel_o          = OP_A_REGA_OR_FWD;

alu_op_b_mux_sel_o          = OP_B_REGB_OR_FWD;

imm_a_mux_sel_o             = IMMA_ZERO;

imm_b_mux_sel_o             = IMMB_I;

mult_int_en                 = 1'b0;

div_int_en                  = 1'b0;

multdiv_operator_o          = MD_OP_MULL;

multdiv_signed_mode_o       = 2'b00;

regfile_we                  = 1'b0;

csr_access_o                = 1'b0;

csr_status_o                = 1'b0;

csr_illegal                 = 1'b0;

csr_op                      = CSR_OP_NONE;

data_we_o                   = 1'b0;

data_type_o                 = 2'b00;

data_sign_extension_o       = 1'b0;

data_reg_offset_o           = 2'b00;

data_req                    = 1'b0;

data_load_event_o           = 1'b0;

illegal_insn_o              = 1'b0;

ebrk_insn_o                 = 1'b0;

mret_insn_o                 = 1'b0;

ecall_insn_o                = 1'b0;

pipe_flush_o                = 1'b0;

unique case (instr_rdata_i[6:0])

//////////////////////////////////////

//      _ _   _ __  __ ____  ____   //

//     | | | | |  \/  |  _ \/ ___|  //

//  _  | | | | | |\/| | |_) \___ \  //

// | |_| | |_| | |  | |  __/ ___) | //

//  \___/ \___/|_|  |_|_|   |____/  //

//                                  //

//////////////////////////////////////

OPCODE_JAL: begin   // Jump and Link

jump_in_id            = 1'b1;

if(jump_mux_i) begin

// Calculate jump target

alu_op_a_mux_sel_o  = OP_A_CURRPC;

alu_op_b_mux_sel_o  = OP_B_IMM;

imm_b_mux_sel_o     = IMMB_UJ;

alu_operator_o      = ALU_ADD;

regfile_we          = 1'b0;

end else begin

// Calculate and store PC+4

alu_op_a_mux_sel_o  = OP_A_CURRPC;

alu_op_b_mux_sel_o  = OP_B_IMM;

imm_b_mux_sel_o     = IMMB_PCINCR;

alu_operator_o      = ALU_ADD;

regfile_we          = 1'b1;

end

end

OPCODE_JALR: begin  // Jump and Link Register

jump_in_id            = 1'b1;

if(jump_mux_i) begin

// Calculate jump target

alu_op_a_mux_sel_o  = OP_A_REGA_OR_FWD;

alu_op_b_mux_sel_o  = OP_B_IMM;

imm_b_mux_sel_o     = IMMB_I;

alu_operator_o      = ALU_ADD;

regfile_we          = 1'b0;

end else begin

// Calculate and store PC+4

alu_op_a_mux_sel_o  = OP_A_CURRPC;

alu_op_b_mux_sel_o  = OP_B_IMM;

imm_b_mux_sel_o     = IMMB_PCINCR;

alu_operator_o      = ALU_ADD;

regfile_we          = 1'b1;

end

if (instr_rdata_i[14:12] != 3'b0) begin

jump_in_id       = 1'b0;

regfile_we       = 1'b0;

illegal_insn_o   = 1'b1;

end

end

OPCODE_BRANCH: begin // Branch

branch_in_id          = 1'b1;

if (branch_mux_i)

begin

unique case (instr_rdata_i[14:12])

3'b000: alu_operator_o = ALU_EQ;

3'b001: alu_operator_o = ALU_NE;

3'b100: alu_operator_o = ALU_LTS;

3'b101: alu_operator_o = ALU_GES;

3'b110: alu_operator_o = ALU_LTU;

3'b111: alu_operator_o = ALU_GEU;

default: begin

illegal_insn_o = 1'b1;

end

endcase

end

else begin

// Calculate jump target in EX

alu_op_a_mux_sel_o  = OP_A_CURRPC;

alu_op_b_mux_sel_o  = OP_B_IMM;

imm_b_mux_sel_o     = IMMB_SB;

alu_operator_o      = ALU_ADD;

regfile_we          = 1'b0;

end

end

//////////////////////////////////

//  _     ____    ______ _____  //

// | |   |  _ \  / / ___|_   _| //

// | |   | | | |/ /\___ \ | |   //

// | |___| |_| / /  ___) || |   //

// |_____|____/_/  |____/ |_|   //

//                              //

//////////////////////////////////

OPCODE_STORE: begin

data_req       = 1'b1;

data_we_o      = 1'b1;

alu_operator_o = ALU_ADD;

if (instr_rdata_i[14] == 1'b0) begin

// offset from immediate

imm_b_mux_sel_o     = IMMB_S;

alu_op_b_mux_sel_o  = OP_B_IMM;

end

// Register offset is illegal since no register c available

else begin

data_req       = 1'b0;

data_we_o      = 1'b0;

illegal_insn_o = 1'b1;

end

// store size

unique case (instr_rdata_i[13:12])

2'b00: data_type_o = 2'b10; // SB

2'b01: data_type_o = 2'b01; // SH

2'b10: data_type_o = 2'b00; // SW

default: begin

data_req       = 1'b0;

data_we_o      = 1'b0;

illegal_insn_o = 1'b1;

end

endcase

end

OPCODE_LOAD: begin

data_req        = 1'b1;

regfile_we      = 1'b1;

data_type_o     = 2'b00;

// offset from immediate

alu_operator_o      = ALU_ADD;

alu_op_b_mux_sel_o  = OP_B_IMM;

imm_b_mux_sel_o     = IMMB_I;

// sign/zero extension

data_sign_extension_o = ~instr_rdata_i[14];

// load size

unique case (instr_rdata_i[13:12])

2'b00:   data_type_o = 2'b10; // LB

2'b01:   data_type_o = 2'b01; // LH

2'b10:   data_type_o = 2'b00; // LW

default: data_type_o = 2'b00; // illegal or reg-reg

endcase

// reg-reg load (different encoding)

if (instr_rdata_i[14:12] == 3'b111) begin

// offset from RS2

alu_op_b_mux_sel_o = OP_B_REGB_OR_FWD;

// sign/zero extension

data_sign_extension_o = ~instr_rdata_i[30];

// load size

unique case (instr_rdata_i[31:25])

7'b0000_000,

7'b0100_000: data_type_o = 2'b10; // LB, LBU

7'b0001_000,

7'b0101_000: data_type_o = 2'b01; // LH, LHU

7'b0010_000: data_type_o = 2'b00; // LW

default: begin

illegal_insn_o = 1'b1;

end

endcase

end

// special p.elw (event load)

if (instr_rdata_i[14:12] == 3'b110)

data_load_event_o = 1'b1;

if (instr_rdata_i[14:12] == 3'b011) begin

// LD -> RV64 only

illegal_insn_o = 1'b1;

end

end

//////////////////////////

//     _    _    _   _  //

//    / \  | |  | | | | //

//   / _ \ | |  | | | | //

//  / ___ \| |__| |_| | //

// /_/   \_\_____\___/  //

//                      //

//////////////////////////

OPCODE_LUI: begin  // Load Upper Immediate

alu_op_a_mux_sel_o  = OP_A_IMM;

alu_op_b_mux_sel_o  = OP_B_IMM;

imm_a_mux_sel_o     = IMMA_ZERO;

imm_b_mux_sel_o     = IMMB_U;

alu_operator_o      = ALU_ADD;

regfile_we          = 1'b1;

end

OPCODE_AUIPC: begin  // Add Upper Immediate to PC

alu_op_a_mux_sel_o  = OP_A_CURRPC;

alu_op_b_mux_sel_o  = OP_B_IMM;

imm_b_mux_sel_o     = IMMB_U;

alu_operator_o      = ALU_ADD;

regfile_we          = 1'b1;

end

OPCODE_OPIMM: begin // Register-Immediate ALU Operations

alu_op_b_mux_sel_o  = OP_B_IMM;

imm_b_mux_sel_o     = IMMB_I;

regfile_we          = 1'b1;

unique case (instr_rdata_i[14:12])

3'b000: alu_operator_o = ALU_ADD;  // Add Immediate

3'b010: alu_operator_o = ALU_SLTS; // Set to one if Lower Than Immediate

3'b011: alu_operator_o = ALU_SLTU; // Set to one if Lower Than Immediate Unsigned

3'b100: alu_operator_o = ALU_XOR;  // Exclusive Or with Immediate

3'b110: alu_operator_o = ALU_OR;   // Or with Immediate

3'b111: alu_operator_o = ALU_AND;  // And with Immediate

3'b001: begin

alu_operator_o = ALU_SLL;  // Shift Left Logical by Immediate

if (instr_rdata_i[31:25] != 7'b0)

illegal_insn_o = 1'b1;

end

3'b101: begin

if (instr_rdata_i[31:25] == 7'b0)

alu_operator_o = ALU_SRL;  // Shift Right Logical by Immediate

else if (instr_rdata_i[31:25] == 7'b010_0000)

alu_operator_o = ALU_SRA;  // Shift Right Arithmetically by Immediate

else

illegal_insn_o = 1'b1;

end

default: illegal_insn_o = 1'b1;

endcase

end

OPCODE_OP: begin  // Register-Register ALU operation

regfile_we     = 1'b1;

if (instr_rdata_i[31]) begin

illegal_insn_o = 1'b1;

end

else

begin // non bit-manipulation instructions

if (~instr_rdata_i[28])

unique case ({instr_rdata_i[30:25], instr_rdata_i[14:12]})

// RV32I ALU operations

{6'b00_0000, 3'b000}: alu_operator_o = ALU_ADD;   // Add

{6'b10_0000, 3'b000}: alu_operator_o = ALU_SUB;   // Sub

{6'b00_0000, 3'b010}: alu_operator_o = ALU_SLTS;  // Set Lower Than

{6'b00_0000, 3'b011}: alu_operator_o = ALU_SLTU;  // Set Lower Than Unsigned

{6'b00_0000, 3'b100}: alu_operator_o = ALU_XOR;   // Xor

{6'b00_0000, 3'b110}: alu_operator_o = ALU_OR;    // Or

{6'b00_0000, 3'b111}: alu_operator_o = ALU_AND;   // And

{6'b00_0000, 3'b001}: alu_operator_o = ALU_SLL;   // Shift Left Logical

{6'b00_0000, 3'b101}: alu_operator_o = ALU_SRL;   // Shift Right Logical

{6'b10_0000, 3'b101}: alu_operator_o = ALU_SRA;   // Shift Right Arithmetic

// supported RV32M instructions

{6'b00_0001, 3'b000}: begin // mul

alu_operator_o        = ALU_ADD;

multdiv_operator_o    = MD_OP_MULL;

mult_int_en           = 1'b1;

multdiv_signed_mode_o = 2'b00;

illegal_insn_o        = RV32M ? 1'b0 : 1'b1;

end

{6'b00_0001, 3'b001}: begin // mulh

alu_operator_o        = ALU_ADD;

multdiv_operator_o    = MD_OP_MULH;

mult_int_en           = 1'b1;

multdiv_signed_mode_o = 2'b11;

illegal_insn_o        = RV32M ? 1'b0 : 1'b1;

end

{6'b00_0001, 3'b010}: begin // mulhsu

alu_operator_o        = ALU_ADD;

multdiv_operator_o    = MD_OP_MULH;

mult_int_en           = 1'b1;

multdiv_signed_mode_o = 2'b01;

illegal_insn_o        = RV32M ? 1'b0 : 1'b1;

end

{6'b00_0001, 3'b011}: begin // mulhu

alu_operator_o        = ALU_ADD;

multdiv_operator_o    = MD_OP_MULH;

mult_int_en           = 1'b1;

multdiv_signed_mode_o = 2'b00;

illegal_insn_o        = RV32M ? 1'b0 : 1'b1;

end

{6'b00_0001, 3'b100}: begin // div

alu_operator_o        = ALU_ADD;

multdiv_operator_o    = MD_OP_DIV;

div_int_en            = 1'b1;

multdiv_signed_mode_o = 2'b11;

illegal_insn_o        = RV32M ? 1'b0 : 1'b1;

end

{6'b00_0001, 3'b101}: begin // divu

alu_operator_o        = ALU_ADD;

multdiv_operator_o    = MD_OP_DIV;

div_int_en            = 1'b1;

multdiv_signed_mode_o = 2'b00;

illegal_insn_o        = RV32M ? 1'b0 : 1'b1;

end

{6'b00_0001, 3'b110}: begin // rem

alu_operator_o        = ALU_ADD;

multdiv_operator_o    = MD_OP_REM;

div_int_en            = 1'b1;

multdiv_signed_mode_o = 2'b11;

illegal_insn_o        = RV32M ? 1'b0 : 1'b1;

end

{6'b00_0001, 3'b111}: begin // remu

alu_operator_o        = ALU_ADD;

multdiv_operator_o    = MD_OP_REM;

div_int_en            = 1'b1;

multdiv_signed_mode_o = 2'b00;

illegal_insn_o        = RV32M ? 1'b0 : 1'b1;

end

default: begin

illegal_insn_o = 1'b1;

end

endcase

end

end

////////////////////////////////////////////////

//  ____  ____  _____ ____ ___    _    _      //

// / ___||  _ \| ____/ ___|_ _|  / \  | |     //

// \___ \| |_) |  _|| |    | |  / _ \ | |     //

//  ___) |  __/| |__| |___ | | / ___ \| |___  //

// |____/|_|   |_____\____|___/_/   \_\_____| //

//                                            //

////////////////////////////////////////////////

OPCODE_SYSTEM: begin

if (instr_rdata_i[14:12] == 3'b000)

begin

// non CSR related SYSTEM instructions

unique case (instr_rdata_i[31:20])

12'h000:  // ECALL

begin

// environment (system) call

ecall_insn_o = 1'b1;

end

12'h001:  // ebreak

begin

// debugger trap

ebrk_insn_o = 1'b1;

end

12'h302:  // mret

begin

mret_insn_o = 1'b1;

end

12'h105:  // wfi

begin

// flush pipeline

pipe_flush_o = 1'b1;

end

default:

begin

illegal_insn_o = 1'b1;

end

endcase

end

else

begin

// instruction to read/modify CSR

csr_access_o        = 1'b1;

regfile_we          = 1'b1;

alu_op_b_mux_sel_o  = OP_B_IMM;

imm_a_mux_sel_o     = IMMA_Z;

imm_b_mux_sel_o     = IMMB_I;    // CSR address is encoded in I imm

if (instr_rdata_i[14] == 1'b1) begin

// rs1 field is used as immediate

alu_op_a_mux_sel_o = OP_A_IMM;

end else begin

alu_op_a_mux_sel_o = OP_A_REGA_OR_FWD;

end

unique case (instr_rdata_i[13:12])

2'b01:   csr_op   = CSR_OP_WRITE;

2'b10:   csr_op   = CSR_OP_SET;

2'b11:   csr_op   = CSR_OP_CLEAR;

default: csr_illegal = 1'b1;

endcase

if(~csr_illegal)

if (instr_rdata_i[31:20] == 12'h300)

//access to mstatus

csr_status_o = 1'b1;

illegal_insn_o = csr_illegal;

end

end

default: begin

illegal_insn_o = 1'b1;

end

endcase

// make sure invalid compressed instruction causes an exception

if (illegal_c_insn_i) begin

illegal_insn_o = 1'b1;

end

// misaligned access was detected by the LSU

// TODO: this section should eventually be moved out of the decoder

if (data_misaligned_i == 1'b1)

begin

// only part of the pipeline is unstalled, make sure that the

// correct operands are sent to the AGU

alu_op_a_mux_sel_o  = OP_A_REGA_OR_FWD;

alu_op_b_mux_sel_o  = OP_B_IMM;

imm_b_mux_sel_o     = IMMB_PCINCR;

// if prepost increments are used, we do not write back the

// second address since the first calculated address was

// the correct one

regfile_we = 1'b0;

end

end

// deassert we signals (in case of stalls)

assign regfile_we_o      = (deassert_we_i) ? 1'b0          : regfile_we;

assign mult_int_en_o     = RV32M ? ((deassert_we_i) ? 1'b0 : mult_int_en) : 1'b0;

assign div_int_en_o      = RV32M ? ((deassert_we_i) ? 1'b0 : div_int_en ) : 1'b0;

assign data_req_o        = (deassert_we_i) ? 1'b0          : data_req;

assign csr_op_o          = (deassert_we_i) ? CSR_OP_NONE   : csr_op;

assign jump_in_id_o      = (deassert_we_i) ? 1'b0          : jump_in_id;

assign branch_in_id_o    = (deassert_we_i) ? 1'b0          : branch_in_id;

endmodule // controller

`include "zeroriscy_config.sv"

import zeroriscy_defines::*;

module zeroriscy_ex_block

#(

parameter RV32M      = 1

)

(

input  logic        clk,

input  logic        rst_n,

// ALU signals from ID stage

input  logic [ALU_OP_WIDTH-1:0] alu_operator_i,

input  logic [1:0]              multdiv_operator_i,

input  logic                    mult_en_i,

input  logic                    div_en_i,

input  logic [31:0]             alu_operand_a_i,

input  logic [31:0]             alu_operand_b_i,

input  logic  [1:0]             multdiv_signed_mode_i,

input  logic [31:0]             multdiv_operand_a_i,

input  logic [31:0]             multdiv_operand_b_i,

output logic [31:0]             alu_adder_result_ex_o,

output logic [31:0]             regfile_wdata_ex_o,

// To IF: Jump and branch target and decision

output logic [31:0]             jump_target_o,

output logic                    branch_decision_o,

input  logic                    lsu_en_i,

// Stall Control

input  logic                    lsu_ready_ex_i, // LSU is done

output logic                    ex_ready_o      // EX stage gets new data

);

localparam MULT_TYPE = 1; //0 is SLOW

logic [31:0] alu_result, multdiv_result;

logic [32:0] multdiv_alu_operand_b, multdiv_alu_operand_a;

logic [33:0] alu_adder_result_ext;

logic        alu_cmp_result, alu_is_equal_result;

logic        multdiv_ready, multdiv_en_sel;

logic        multdiv_en;

/*

The multdiv_i output is never selected if RV32M=0

At synthesis time, all the combinational and sequential logic

from the multdiv_i module are eliminated

*/

generate

if (RV32M) begin

assign multdiv_en_sel     = MULT_TYPE == 0 ? mult_en_i | div_en_i : div_en_i;

assign multdiv_en         = mult_en_i | div_en_i;

end else begin

assign multdiv_en_sel     = 1'b0;

assign multdiv_en         = 1'b0;

end

endgenerate

assign regfile_wdata_ex_o = multdiv_en ? multdiv_result : alu_result;

// branch handling

assign branch_decision_o  = alu_cmp_result;

assign jump_target_o      = alu_adder_result_ex_o;

////////////////////////////

//     _    _    _   _    //

//    / \  | |  | | | |   //

//   / _ \ | |  | | | |   //

//  / ___ \| |__| |_| |   //

// /_/   \_\_____\___/    //

//                        //

////////////////////////////

zeroriscy_alu alu_i

(

.operator_i          ( alu_operator_i            ),

.operand_a_i         ( alu_operand_a_i           ),

.operand_b_i         ( alu_operand_b_i           ),

.multdiv_operand_a_i ( multdiv_alu_operand_a     ),

.multdiv_operand_b_i ( multdiv_alu_operand_b     ),

.multdiv_en_i        ( multdiv_en_sel            ),

.adder_result_o      ( alu_adder_result_ex_o     ),

.adder_result_ext_o  ( alu_adder_result_ext      ),

.result_o            ( alu_result                ),

.comparison_result_o ( alu_cmp_result            ),

.is_equal_result_o   ( alu_is_equal_result       )

);

////////////////////////////////////////////////////////////////

//  __  __ _   _ _   _____ ___ ____  _     ___ _____ ____     //

// |  \/  | | | | | |_   _|_ _|  _ \| |   |_ _| ____|  _ \    //

// | |\/| | | | | |   | |  | || |_) | |    | ||  _| | |_) |   //

// | |  | | |_| | |___| |  | ||  __/| |___ | || |___|  _ <    //

// |_|  |_|\___/|_____|_| |___|_|   |_____|___|_____|_| \_\   //

//                                                            //

////////////////////////////////////////////////////////////////

generate

if (MULT_TYPE == 0) begin : multdiv_slow

zeroriscy_multdiv_slow multdiv_i

(

.clk                ( clk                   ),

.rst_n              ( rst_n                 ),

.mult_en_i          ( mult_en_i             ),

.div_en_i           ( div_en_i              ),

.operator_i         ( multdiv_operator_i    ),

.signed_mode_i      ( multdiv_signed_mode_i ),

.op_a_i             ( multdiv_operand_a_i   ),

.op_b_i             ( multdiv_operand_b_i   ),

.alu_adder_ext_i    ( alu_adder_result_ext  ),

.alu_adder_i        ( alu_adder_result_ex_o ),

.equal_to_zero      ( alu_is_equal_result   ),

.ready_o            ( multdiv_ready         ),

.alu_operand_a_o    ( multdiv_alu_operand_a ),

.alu_operand_b_o    ( multdiv_alu_operand_b ),

.multdiv_result_o   ( multdiv_result        )

);

end else begin: multdiv_fast

zeroriscy_multdiv_fast multdiv_i

(

.clk                ( clk                   ),

.rst_n              ( rst_n                 ),

.mult_en_i          ( mult_en_i             ),

.div_en_i           ( div_en_i              ),

.operator_i         ( multdiv_operator_i    ),

.signed_mode_i      ( multdiv_signed_mode_i ),

.op_a_i             ( multdiv_operand_a_i   ),

.op_b_i             ( multdiv_operand_b_i   ),

.alu_operand_a_o    ( multdiv_alu_operand_a ),

.alu_operand_b_o    ( multdiv_alu_operand_b ),

.alu_adder_ext_i    ( alu_adder_result_ext  ),

.alu_adder_i        ( alu_adder_result_ex_o ),

.equal_to_zero      ( alu_is_equal_result   ),

.ready_o            ( multdiv_ready         ),

.multdiv_result_o   ( multdiv_result        )

);

end

endgenerate

always_comb

begin

unique case (1'b1)

multdiv_en:

ex_ready_o = multdiv_ready;

lsu_en_i:

ex_ready_o = lsu_ready_ex_i;

default:

//1 Cycle case

ex_ready_o = 1'b1;

endcase

end

Endmodule

`include "zeroriscy_config.sv"

// input port: send address one cycle before the data

// clear_i clears the FIFO for the following cycle. in_addr_i can be sent in

// this cycle already

module zeroriscy_fetch_fifo

(

input  logic        clk,

input  logic        rst_n,

// control signals

input  logic        clear_i,          // clears the contents of the fifo

// input port

input  logic [31:0] in_addr_i,

input  logic [31:0] in_rdata_i,

input  logic        in_valid_i,

output logic        in_ready_o,

// output port

output logic        out_valid_o,

input  logic        out_ready_i,

output logic [31:0] out_rdata_o,

output logic [31:0] out_addr_o,

output logic        out_valid_stored_o // same as out_valid_o, except that if something is incoming now it is not included. This signal is available immediately as it comes directly out of FFs

);

localparam DEPTH = 3; // must be 3 or greater

// index 0 is used for output

logic [DEPTH-1:0] [31:0]  addr_n,    addr_int,    addr_Q;

logic [DEPTH-1:0] [31:0]  rdata_n,   rdata_int,   rdata_Q;

logic [DEPTH-1:0]         valid_n,   valid_int,   valid_Q;

logic             [31:0]  addr_next;

logic             [31:0]  rdata, rdata_unaligned;

logic                     valid, valid_unaligned;

logic                     aligned_is_compressed, unaligned_is_compressed;

logic                     aligned_is_compressed_st, unaligned_is_compressed_st;

//////////////////////////////////////////////////////////////////////////////

// output port

//////////////////////////////////////////////////////////////////////////////

assign rdata = (valid_Q[0]) ? rdata_Q[0] : in_rdata_i;

assign valid = valid_Q[0] || in_valid_i;

assign rdata_unaligned = (valid_Q[1]) ? {rdata_Q[1][15:0], rdata[31:16]} : {in_rdata_i[15:0], rdata[31:16]};

// it is implied that rdata_valid_Q[0] is set

assign valid_unaligned = (valid_Q[1] || (valid_Q[0] && in_valid_i));

assign unaligned_is_compressed    = rdata[17:16] != 2'b11;

assign aligned_is_compressed      = rdata[1:0] != 2'b11;

assign unaligned_is_compressed_st = rdata_Q[0][17:16] != 2'b11;

assign aligned_is_compressed_st   = rdata_Q[0][1:0] != 2'b11;

//////////////////////////////////////////////////////////////////////////////

// instruction aligner (if unaligned)

//////////////////////////////////////////////////////////////////////////////

always_comb

begin

// serve the aligned case even though the output address is unaligned when

// the next instruction will be from a hardware loop target

// in this case the current instruction is already prealigned in element 0

if (out_addr_o[1]) begin

// unaligned case

out_rdata_o = rdata_unaligned;

if (unaligned_is_compressed)

out_valid_o = valid;

else

out_valid_o = valid_unaligned;

end else begin

// aligned case

out_rdata_o = rdata;

out_valid_o = valid;

end

end

assign out_addr_o    = (valid_Q[0]) ? addr_Q[0] : in_addr_i;

// this valid signal must not depend on signals from outside!

always_comb

begin

out_valid_stored_o = 1'b1;

if (out_addr_o[1]) begin

if (unaligned_is_compressed_st)

out_valid_stored_o = 1'b1;

else

out_valid_stored_o = valid_Q[1];

end else begin

out_valid_stored_o = valid_Q[0];

end

end

//////////////////////////////////////////////////////////////////////////////

// input port

//////////////////////////////////////////////////////////////////////////////

// we accept data as long as our fifo is not full

// we don't care about clear here as the data will be received one cycle

// later anyway

assign in_ready_o = ~valid_Q[DEPTH-2];

//////////////////////////////////////////////////////////////////////////////

// FIFO management

//////////////////////////////////////////////////////////////////////////////

always_comb

begin

int j;

addr_int    = addr_Q;

rdata_int   = rdata_Q;

valid_int   = valid_Q;

if (in_valid_i) begin

for(j = 0; j < DEPTH; j++) begin

if (~valid_Q[j]) begin

addr_int[j]  = in_addr_i;

rdata_int[j] = in_rdata_i;

valid_int[j] = 1'b1;

break;

end

end

end

end

assign addr_next = {addr_int[0][31:2], 2'b00} + 32'h4;

// move everything by one step

always_comb

begin

addr_n     = addr_int;

rdata_n    = rdata_int;

valid_n    = valid_int;

if (out_ready_i && out_valid_o) begin

begin

if (addr_int[0][1]) begin

// unaligned case

if (unaligned_is_compressed) begin

addr_n[0] = {addr_next[31:2], 2'b00};

end else begin

addr_n[0] = {addr_next[31:2], 2'b10};

end

rdata_n  = {32'b0, rdata_int[DEPTH-1:1]};

valid_n  = {1'b0,  valid_int[DEPTH-1:1]};

end else begin

if (aligned_is_compressed) begin

// just increase address, do not move to next entry in FIFO

addr_n[0] = {addr_int[0][31:2], 2'b10};

end else begin

// move to next entry in FIFO

addr_n[0] = {addr_next[31:2], 2'b00};

rdata_n   = {32'b0, rdata_int[DEPTH-1:1]};

valid_n   = {1'b0,  valid_int[DEPTH-1:1]};

end

end

end

end

end

//////////////////////////////////////////////////////////////////////////////

// registers

//////////////////////////////////////////////////////////////////////////////

always_ff @(posedge clk, negedge rst_n)

begin

if(rst_n == 1'b0)

begin

addr_Q    <= '{default: '0};

rdata_Q   <= '{default: '0};

valid_Q   <= '0;

end

else

begin

// on a clear signal from outside we invalidate the content of the FIFO

// completely and start from an empty state

if (clear_i) begin

valid_Q    <= '0;

end else begin

addr_Q    <= addr_n;

rdata_Q   <= rdata_n;

valid_Q   <= valid_n;

end

end

end

//----------------------------------------------------------------------------

// Assertions

//----------------------------------------------------------------------------

`ifndef VERILATOR

assert property (

@(posedge clk) (in_valid_i) |-> ((valid_Q[DEPTH-1] == 1'b0) || (clear_i == 1'b1)) );

`endif

Endmodule

`include "zeroriscy_config.sv"

// input port: send address one cycle before the data

// clear_i clears the FIFO for the following cycle. in_addr_i can be sent in

// this cycle already

module zeroriscy_fetch_fifo

(

input  logic        clk,

input  logic        rst_n,

// control signals

input  logic        clear_i,          // clears the contents of the fifo

// input port

input  logic [31:0] in_addr_i,

input  logic [31:0] in_rdata_i,

input  logic        in_valid_i,

output logic        in_ready_o,

// output port

output logic        out_valid_o,

input  logic        out_ready_i,

output logic [31:0] out_rdata_o,

output logic [31:0] out_addr_o,

output logic        out_valid_stored_o // same as out_valid_o, except that if something is incoming now it is not included. This signal is available immediately as it comes directly out of FFs

);

localparam DEPTH = 3; // must be 3 or greater

// index 0 is used for output

logic [DEPTH-1:0] [31:0]  addr_n,    addr_int,    addr_Q;

logic [DEPTH-1:0] [31:0]  rdata_n,   rdata_int,   rdata_Q;

logic [DEPTH-1:0]         valid_n,   valid_int,   valid_Q;

logic             [31:0]  addr_next;

logic             [31:0]  rdata, rdata_unaligned;

logic                     valid, valid_unaligned;

logic                     aligned_is_compressed, unaligned_is_compressed;

logic                     aligned_is_compressed_st, unaligned_is_compressed_st;

//////////////////////////////////////////////////////////////////////////////

// output port

//////////////////////////////////////////////////////////////////////////////

assign rdata = (valid_Q[0]) ? rdata_Q[0] : in_rdata_i;

assign valid = valid_Q[0] || in_valid_i;

assign rdata_unaligned = (valid_Q[1]) ? {rdata_Q[1][15:0], rdata[31:16]} : {in_rdata_i[15:0], rdata[31:16]};

// it is implied that rdata_valid_Q[0] is set

assign valid_unaligned = (valid_Q[1] || (valid_Q[0] && in_valid_i));

assign unaligned_is_compressed    = rdata[17:16] != 2'b11;

assign aligned_is_compressed      = rdata[1:0] != 2'b11;

assign unaligned_is_compressed_st = rdata_Q[0][17:16] != 2'b11;

assign aligned_is_compressed_st   = rdata_Q[0][1:0] != 2'b11;

//////////////////////////////////////////////////////////////////////////////

// instruction aligner (if unaligned)

//////////////////////////////////////////////////////////////////////////////

always_comb

begin

// serve the aligned case even though the output address is unaligned when

// the next instruction will be from a hardware loop target

// in this case the current instruction is already prealigned in element 0

if (out_addr_o[1]) begin

// unaligned case

out_rdata_o = rdata_unaligned;

if (unaligned_is_compressed)

out_valid_o = valid;

else

out_valid_o = valid_unaligned;

end else begin

// aligned case

out_rdata_o = rdata;

out_valid_o = valid;

end

end

assign out_addr_o    = (valid_Q[0]) ? addr_Q[0] : in_addr_i;

// this valid signal must not depend on signals from outside!

always_comb

begin

out_valid_stored_o = 1'b1;

if (out_addr_o[1]) begin

if (unaligned_is_compressed_st)

out_valid_stored_o = 1'b1;

else

out_valid_stored_o = valid_Q[1];

end else begin

out_valid_stored_o = valid_Q[0];

end

end

//////////////////////////////////////////////////////////////////////////////

// input port

//////////////////////////////////////////////////////////////////////////////

// we accept data as long as our fifo is not full

// we don't care about clear here as the data will be received one cycle

// later anyway

assign in_ready_o = ~valid_Q[DEPTH-2];

//////////////////////////////////////////////////////////////////////////////

// FIFO management

//////////////////////////////////////////////////////////////////////////////

always_comb

begin

int j;

addr_int    = addr_Q;

rdata_int   = rdata_Q;

valid_int   = valid_Q;

if (in_valid_i) begin

for(j = 0; j < DEPTH; j++) begin

if (~valid_Q[j]) begin

addr_int[j]  = in_addr_i;

rdata_int[j] = in_rdata_i;

valid_int[j] = 1'b1;

break;

end

end

end

end

assign addr_next = {addr_int[0][31:2], 2'b00} + 32'h4;

// move everything by one step

always_comb

begin

addr_n     = addr_int;

rdata_n    = rdata_int;

valid_n    = valid_int;

if (out_ready_i && out_valid_o) begin

begin

if (addr_int[0][1]) begin

// unaligned case

if (unaligned_is_compressed) begin

addr_n[0] = {addr_next[31:2], 2'b00};

end else begin

addr_n[0] = {addr_next[31:2], 2'b10};

end

rdata_n  = {32'b0, rdata_int[DEPTH-1:1]};

valid_n  = {1'b0,  valid_int[DEPTH-1:1]};

end else begin

if (aligned_is_compressed) begin

// just increase address, do not move to next entry in FIFO

addr_n[0] = {addr_int[0][31:2], 2'b10};

end else begin

// move to next entry in FIFO

addr_n[0] = {addr_next[31:2], 2'b00};

rdata_n   = {32'b0, rdata_int[DEPTH-1:1]};

valid_n   = {1'b0,  valid_int[DEPTH-1:1]};

end

end

end

end

end

//////////////////////////////////////////////////////////////////////////////

// registers

//////////////////////////////////////////////////////////////////////////////

always_ff @(posedge clk, negedge rst_n)

begin

if(rst_n == 1'b0)

begin

addr_Q    <= '{default: '0};

rdata_Q   <= '{default: '0};

valid_Q   <= '0;

end

else

begin

// on a clear signal from outside we invalidate the content of the FIFO

// completely and start from an empty state

if (clear_i) begin

valid_Q    <= '0;

end else begin

addr_Q    <= addr_n;

rdata_Q   <= rdata_n;

valid_Q   <= valid_n;

end

end

end

//----------------------------------------------------------------------------

// Assertions

//----------------------------------------------------------------------------

`ifndef VERILATOR

assert property (

@(posedge clk) (in_valid_i) |-> ((valid_Q[DEPTH-1] == 1'b0) || (clear_i == 1'b1)) );

`endif

Endmodule

`include "zeroriscy_config.sv"

import zeroriscy_defines::*;

// Source/Destination register instruction index

`define REG_S1 19:15

`define REG_S2 24:20

`define REG_D  11:07

module zeroriscy_id_stage

#(

parameter RV32M      = 1,

parameter RV32E      = 0

)

(

input  logic        clk,

input  logic        rst_n,

input  logic        test_en_i,

input  logic        fetch_enable_i,

output logic        ctrl_busy_o,

output logic        core_ctrl_firstfetch_o,

output logic        is_decoding_o,

// Interface to IF stage

input  logic              instr_valid_i,

input  logic       [31:0] instr_rdata_i,      // comes from pipeline of IF stage

output logic              instr_req_o,

// Jumps and branches

output logic        branch_in_ex_o,

input  logic        branch_decision_i,

// IF and ID stage signals

output logic        clear_instr_valid_o,

output logic        pc_set_o,

output logic [2:0]  pc_mux_o,

output logic [1:0]  exc_pc_mux_o,

input  logic        illegal_c_insn_i,

input  logic        is_compressed_i,

input  logic [31:0] pc_id_i,

// Stalls

output logic        halt_if_o,      // controller requests a halt of the IF stage

output logic        id_ready_o,     // ID stage is ready for the next instruction

input  logic        ex_ready_i,

output logic        id_valid_o,     // ID stage is done

// ALU

output logic [ALU_OP_WIDTH-1:0] alu_operator_ex_o,

output logic [31:0] alu_operand_a_ex_o,

output logic [31:0] alu_operand_b_ex_o,

// MUL, DIV

output logic        mult_en_ex_o,

output logic        div_en_ex_o,

output logic  [1:0] multdiv_operator_ex_o,

output logic  [1:0] multdiv_signed_mode_ex_o,

output logic [31:0] multdiv_operand_a_ex_o,

output logic [31:0] multdiv_operand_b_ex_o,

// CSR

output logic        csr_access_ex_o,

output logic [1:0]  csr_op_ex_o,

output logic [5:0]  csr_cause_o,

output logic        csr_save_if_o,

output logic        csr_save_id_o,

output logic        csr_restore_mret_id_o,

output logic        csr_save_cause_o,

// Interface to load store unit

output logic        data_req_ex_o,

output logic        data_we_ex_o,

output logic [1:0]  data_type_ex_o,

output logic        data_sign_ext_ex_o,

output logic [1:0]  data_reg_offset_ex_o,

output logic        data_load_event_ex_o,

output logic [31:0] data_wdata_ex_o,

input  logic        data_misaligned_i,

input  logic [31:0] misaligned_addr_i,

// Interrupt signals

input  logic        irq_i,

input  logic [4:0]  irq_id_i,

input  logic        m_irq_enable_i,

output logic        irq_ack_o,

output logic [4:0]  irq_id_o,

output logic [5:0]  exc_cause_o,

input  logic        lsu_load_err_i,

input  logic        lsu_store_err_i,

// Debug Unit Signals

input  logic [DBG_SETS_W-1:0] dbg_settings_i,

input  logic        dbg_req_i,

output logic        dbg_ack_o,

input  logic        dbg_stall_i,

output logic        dbg_trap_o,

input  logic        dbg_reg_rreq_i,

input  logic [4:0]  dbg_reg_raddr_i,

output logic [31:0] dbg_reg_rdata_o,

input  logic        dbg_reg_wreq_i,

input  logic [4:0]  dbg_reg_waddr_i,

input  logic [31:0] dbg_reg_wdata_i,

input  logic        dbg_jump_req_i,

// Write back signal

input  logic [31:0] regfile_wdata_lsu_i,

input  logic [31:0] regfile_wdata_ex_i,

input  logic [31:0] csr_rdata_i,

// Performance Counters

output logic        perf_jump_o,          // we are executing a jump instruction

output logic        perf_branch_o,        // we are executing a branch instruction

output logic        perf_tbranch_o        // we are executing a taken branch instruction

);

logic [31:0] instr;

// Decoder/Controller ID stage internal signals

logic        deassert_we;

logic        illegal_insn_dec;

logic        illegal_reg_rv32e;

logic        ebrk_insn;

logic        mret_insn_dec;

logic        ecall_insn_dec;

logic        pipe_flush_dec;

logic        branch_taken_ex;

logic        branch_in_id;

logic        branch_set_n;

logic        branch_set_q;

logic        branch_mux_dec;

logic        jump_set;

logic        jump_mux_dec;

logic        jump_in_id;

logic        instr_multicyle;

logic        load_stall;

logic        multdiv_stall;

logic        branch_stall;

logic        jump_stall;

logic        halt_id;

//FSM signals to write back multi cycles instructions

logic        regfile_we;

enum logic {RF_LSU, RF_EX} select_data_rf;

// Immediate decoding and sign extension

logic [31:0] imm_i_type;

logic [31:0] imm_iz_type;

logic [31:0] imm_s_type;

logic [31:0] imm_sb_type;

logic [31:0] imm_u_type;

logic [31:0] imm_uj_type;

logic [31:0] imm_z_type;

logic [31:0] imm_s2_type;

logic [31:0] imm_bi_type;

logic [31:0] imm_s3_type;

logic [31:0] imm_vs_type;

logic [31:0] imm_vu_type;

logic [31:0] imm_a;       // contains the immediate for operand b

logic [31:0] imm_b;       // contains the immediate for operand b

// Signals running between controller and exception controller

logic       irq_req_ctrl;

logic [4:0] irq_id_ctrl;

logic       exc_ack, exc_kill;// handshake

// Register file interface

logic [4:0]  regfile_addr_ra_id;

logic [4:0]  regfile_addr_rb_id;

logic [4:0]  regfile_alu_waddr_id;

logic        regfile_we_id;

logic [31:0] regfile_data_ra_id;

logic [31:0] regfile_data_rb_id;

// ALU Control

logic [ALU_OP_WIDTH-1:0] alu_operator;

logic [2:0]  alu_op_a_mux_sel;

logic [2:0]  alu_op_b_mux_sel;

logic [0:0]  imm_a_mux_sel;

logic [3:0]  imm_b_mux_sel;

// Multiplier Control

logic        mult_int_en;      // use integer multiplier

logic        div_int_en;      // use integer division or reminder

logic        multdiv_int_en;

logic [1:0]  multdiv_operator;

logic [1:0]  multdiv_signed_mode;

// Data Memory Control

logic        data_we_id;

logic [1:0]  data_type_id;

logic        data_sign_ext_id;

logic [1:0]  data_reg_offset_id;

logic        data_req_id;

logic        data_load_event_id;

// CSR control

logic        csr_access;

logic [1:0]  csr_op;

logic        csr_status;

// Forwarding

logic [1:0]  operand_a_fw_mux_sel;

logic [31:0] operand_a_fw_id;

logic [31:0] operand_b_fw_id;

logic [31:0] operand_b;

logic [31:0] alu_operand_a;

logic [31:0] alu_operand_b;

assign instr = instr_rdata_i;

// immediate extraction and sign extension

assign imm_i_type  = { {20 {instr[31]}}, instr[31:20] };

assign imm_iz_type = {            20'b0, instr[31:20] };

assign imm_s_type  = { {20 {instr[31]}}, instr[31:25], instr[11:7] };

assign imm_sb_type = { {19 {instr[31]}}, instr[31], instr[7], instr[30:25], instr[11:8], 1'b0 };

assign imm_u_type  = { instr[31:12], 12'b0 };

assign imm_uj_type = { {12 {instr[31]}}, instr[19:12], instr[20], instr[30:21], 1'b0 };

// immediate for CSR manipulatin (zero extended)

assign imm_z_type = { 27'b0, instr[`REG_S1] };

assign imm_s2_type = { 27'b0, instr[24:20] };

assign imm_bi_type = { {27{instr[24]}}, instr[24:20] };

assign imm_s3_type = { 27'b0, instr[29:25] };

assign imm_vs_type = { {26 {instr[24]}}, instr[24:20], instr[25] };

assign imm_vu_type = { 26'b0, instr[24:20], instr[25] };

//---------------------------------------------------------------------------

// source register selection

//---------------------------------------------------------------------------

assign regfile_addr_ra_id = instr[`REG_S1];

assign regfile_addr_rb_id = instr[`REG_S2];

//---------------------------------------------------------------------------

// destination registers

//---------------------------------------------------------------------------

assign regfile_alu_waddr_id = instr[`REG_D];

//if(RV32E)

//  assign illegal_reg_rv32e = (regfile_addr_ra_id[4] | regfile_addr_rb_id[4] | regfile_alu_waddr_id[4]);

//else

assign illegal_reg_rv32e = 1'b0;

// kill instruction in the IF/ID stage by setting the instr_valid_id control

// signal to 0 for instructions that are done

assign clear_instr_valid_o = id_ready_o | halt_id;

assign branch_taken_ex = branch_in_id & branch_decision_i;

////////////////////////////////////////////////////////

//   ___                                 _      _     //

//  / _ \ _ __   ___ _ __ __ _ _ __   __| |    / \    //

// | | | | '_ \ / _ \ '__/ _` | '_ \ / _` |   / _ \   //

// | |_| | |_) |  __/ | | (_| | | | | (_| |  / ___ \  //

//  \___/| .__/ \___|_|  \__,_|_| |_|\__,_| /_/   \_\ //

//       |_|                                          //

////////////////////////////////////////////////////////

// ALU_Op_a Mux

always_comb

begin : alu_operand_a_mux

case (alu_op_a_mux_sel)

OP_A_REGA_OR_FWD:  alu_operand_a = operand_a_fw_id;

OP_A_CURRPC:       alu_operand_a = pc_id_i;

OP_A_IMM:          alu_operand_a = imm_a;

default:           alu_operand_a = operand_a_fw_id;

endcase; // case (alu_op_a_mux_sel)

end

always_comb

begin : immediate_a_mux

unique case (imm_a_mux_sel)

IMMA_Z:      imm_a = imm_z_type;

IMMA_ZERO:   imm_a = '0;

default:     imm_a = '0;

endcase

end

// Operand a forwarding mux used with LSU instructions

always_comb

begin : operand_a_fw_mux

case (operand_a_fw_mux_sel)

SEL_MISALIGNED:    operand_a_fw_id = misaligned_addr_i;

SEL_REGFILE:       operand_a_fw_id = regfile_data_ra_id;

default:           operand_a_fw_id = regfile_data_ra_id;

endcase; // case (operand_a_fw_mux_sel)

end

//////////////////////////////////////////////////////

//   ___                                 _   ____   //

//  / _ \ _ __   ___ _ __ __ _ _ __   __| | | __ )  //

// | | | | '_ \ / _ \ '__/ _` | '_ \ / _` | |  _ \  //

// | |_| | |_) |  __/ | | (_| | | | | (_| | | |_) | //

//  \___/| .__/ \___|_|  \__,_|_| |_|\__,_| |____/  //

//       |_|                                        //

//////////////////////////////////////////////////////

// Immediate Mux for operand B

always_comb

begin : immediate_b_mux

unique case (imm_b_mux_sel)

IMMB_I:      imm_b = imm_i_type;

IMMB_S:      imm_b = imm_s_type;

IMMB_U:      imm_b = imm_u_type;

IMMB_PCINCR: imm_b = (is_compressed_i && (~data_misaligned_i)) ? 32'h2 : 32'h4;

IMMB_S2:     imm_b = imm_s2_type;

IMMB_BI:     imm_b = imm_bi_type;

IMMB_S3:     imm_b = imm_s3_type;

IMMB_VS:     imm_b = imm_vs_type;

IMMB_VU:     imm_b = imm_vu_type;

IMMB_UJ:     imm_b = imm_uj_type;

IMMB_SB:     imm_b = imm_sb_type;

default:     imm_b = imm_i_type;

endcase

end

// ALU_Op_b Mux

always_comb

begin : alu_operand_b_mux

case (alu_op_b_mux_sel)

OP_B_REGB_OR_FWD:  operand_b = regfile_data_rb_id;

OP_B_IMM:          operand_b = imm_b;

default:           operand_b = regfile_data_rb_id;

endcase // case (alu_op_b_mux_sel)

end

assign alu_operand_b   = operand_b;

assign operand_b_fw_id = regfile_data_rb_id;

/////////////////////////////////////////////////////////

//  ____  _____ ____ ___ ____ _____ _____ ____  ____   //

// |  _ \| ____/ ___|_ _/ ___|_   _| ____|  _ \/ ___|  //

// | |_) |  _|| |  _ | |\___ \ | | |  _| | |_) \___ \  //

// |  _ <| |__| |_| || | ___) || | | |___|  _ < ___) | //

// |_| \_\_____\____|___|____/ |_| |_____|_| \_\____/  //

//                                                     //

/////////////////////////////////////////////////////////

logic [31:0] regfile_wdata_mux;

logic        regfile_we_mux;

logic  [4:0] regfile_waddr_mux;

//TODO: add assertion

// Register File mux

always_comb

begin

if(dbg_reg_wreq_i) begin

regfile_wdata_mux   = dbg_reg_wdata_i;

regfile_waddr_mux   = dbg_reg_waddr_i;

regfile_we_mux      = 1'b1;

end else begin

regfile_we_mux      = regfile_we;

regfile_waddr_mux   = regfile_alu_waddr_id;

if (select_data_rf == RF_LSU)

regfile_wdata_mux = regfile_wdata_lsu_i;

else

if (csr_access)

regfile_wdata_mux = csr_rdata_i;

else

regfile_wdata_mux = regfile_wdata_ex_i;

end

end

zeroriscy_register_file

#(

.RV32E(RV32E)

)

registers_i

(

.clk          ( clk                ),

.rst_n        ( rst_n              ),

.test_en_i    ( test_en_i          ),

// Read port a

.raddr_a_i    ( regfile_addr_ra_id ),

.rdata_a_o    ( regfile_data_ra_id ),

// Read port b

.raddr_b_i    ( (dbg_reg_rreq_i == 1'b0) ? regfile_addr_rb_id : dbg_reg_raddr_i ),

.rdata_b_o    ( regfile_data_rb_id ),

// write port

.waddr_a_i    ( regfile_waddr_mux ),

.wdata_a_i    ( regfile_wdata_mux ),

.we_a_i       ( regfile_we_mux    )

);

assign dbg_reg_rdata_o = regfile_data_rb_id;

assign multdiv_int_en  = mult_int_en | div_int_en;

///////////////////////////////////////////////

//  ____  _____ ____ ___  ____  _____ ____   //

// |  _ \| ____/ ___/ _ \|  _ \| ____|  _ \  //

// | | | |  _|| |  | | | | | | |  _| | |_) | //

// | |_| | |__| |__| |_| | |_| | |___|  _ <  //

// |____/|_____\____\___/|____/|_____|_| \_\ //

//                                           //

///////////////////////////////////////////////

zeroriscy_decoder

#(

.RV32M(RV32M)

)

decoder_i

(

// controller related signals

.deassert_we_i                   ( deassert_we               ),

.data_misaligned_i               ( data_misaligned_i         ),

.branch_mux_i                    ( branch_mux_dec            ),

.jump_mux_i                      ( jump_mux_dec              ),

.illegal_insn_o                  ( illegal_insn_dec          ),

.ebrk_insn_o                     ( ebrk_insn                 ),

.mret_insn_o                     ( mret_insn_dec             ),

.ecall_insn_o                    ( ecall_insn_dec            ),

.pipe_flush_o                    ( pipe_flush_dec            ),

// from IF/ID pipeline

.instr_rdata_i                   ( instr                     ),

.illegal_c_insn_i                ( illegal_c_insn_i          ),

// ALU signals

.alu_operator_o                  ( alu_operator              ),

.alu_op_a_mux_sel_o              ( alu_op_a_mux_sel          ),

.alu_op_b_mux_sel_o              ( alu_op_b_mux_sel          ),

.imm_a_mux_sel_o                 ( imm_a_mux_sel             ),

.imm_b_mux_sel_o                 ( imm_b_mux_sel             ),

.mult_int_en_o                   ( mult_int_en               ),

.div_int_en_o                    ( div_int_en                ),

.multdiv_operator_o              ( multdiv_operator          ),

.multdiv_signed_mode_o           ( multdiv_signed_mode       ),

// Register file control signals

.regfile_we_o                    ( regfile_we_id             ),

// CSR control signals

.csr_access_o                    ( csr_access                ),

.csr_op_o                        ( csr_op                    ),

.csr_status_o                    ( csr_status                ),

// Data bus interface

.data_req_o                      ( data_req_id               ),

.data_we_o                       ( data_we_id                ),

.data_type_o                     ( data_type_id              ),

.data_sign_extension_o           ( data_sign_ext_id          ),

.data_reg_offset_o               ( data_reg_offset_id        ),

.data_load_event_o               ( data_load_event_id        ),

// jump/branches

.jump_in_id_o                    ( jump_in_id                ),

.branch_in_id_o                  ( branch_in_id              )

);

////////////////////////////////////////////////////////////////////

//    ____ ___  _   _ _____ ____   ___  _     _     _____ ____    //

//   / ___/ _ \| \ | |_   _|  _ \ / _ \| |   | |   | ____|  _ \   //

//  | |  | | | |  \| | | | | |_) | | | | |   | |   |  _| | |_) |  //

//  | |__| |_| | |\  | | | |  _ <| |_| | |___| |___| |___|  _ <   //

//   \____\___/|_| \_| |_| |_| \_\\___/|_____|_____|_____|_| \_\  //

//                                                                //

////////////////////////////////////////////////////////////////////

zeroriscy_controller controller_i

(

.clk                            ( clk                    ),

.rst_n                          ( rst_n                  ),

.fetch_enable_i                 ( fetch_enable_i         ),

.ctrl_busy_o                    ( ctrl_busy_o            ),

.first_fetch_o                  ( core_ctrl_firstfetch_o ),

.is_decoding_o                  ( is_decoding_o          ),

// decoder related signals

.deassert_we_o                  ( deassert_we            ),

.illegal_insn_i                 ( illegal_insn_dec | illegal_reg_rv32e ),

.ecall_insn_i                   ( ecall_insn_dec         ),

.mret_insn_i                    ( mret_insn_dec          ),

.pipe_flush_i                   ( pipe_flush_dec         ),

.ebrk_insn_i                    ( ebrk_insn              ),

.csr_status_i                   ( csr_status             ),

// from IF/ID pipeline

.instr_valid_i                  ( instr_valid_i          ),

// from prefetcher

.instr_req_o                    ( instr_req_o            ),

// to prefetcher

.pc_set_o                       ( pc_set_o               ),

.pc_mux_o                       ( pc_mux_o               ),

.exc_pc_mux_o                   ( exc_pc_mux_o           ),

.exc_cause_o                    ( exc_cause_o            ),

// LSU

.data_misaligned_i              ( data_misaligned_i      ),

// jump/branch control

.branch_in_id_i                 ( branch_in_id           ),

.branch_taken_ex_i              ( branch_taken_ex        ),

.branch_set_i                   ( branch_set_q           ),

.jump_set_i                     ( jump_set               ),

.instr_multicyle_i              ( instr_multicyle        ),

// Interrupt Controller Signals

.irq_req_ctrl_i                 ( irq_req_ctrl           ),

.irq_id_ctrl_i                  ( irq_id_ctrl            ),

.m_IE_i                         ( m_irq_enable_i         ),

.irq_ack_o                      ( irq_ack_o              ),

.irq_id_o                       ( irq_id_o               ),

.exc_ack_o                      ( exc_ack                ),

.exc_kill_o                     ( exc_kill               ),

// CSR Controller Signals

.csr_save_cause_o               ( csr_save_cause_o       ),

.csr_cause_o                    ( csr_cause_o            ),

.csr_save_if_o                  ( csr_save_if_o          ),

.csr_save_id_o                  ( csr_save_id_o          ),

.csr_restore_mret_id_o          ( csr_restore_mret_id_o  ),

// Debug Unit Signals

.dbg_req_i                      ( dbg_req_i              ),

.dbg_ack_o                      ( dbg_ack_o              ),

.dbg_stall_i                    ( dbg_stall_i            ),

.dbg_jump_req_i                 ( dbg_jump_req_i         ),

.dbg_settings_i                 ( dbg_settings_i         ),

.dbg_trap_o                     ( dbg_trap_o             ),

// Forwarding signals

.operand_a_fw_mux_sel_o         ( operand_a_fw_mux_sel   ),

// Stall signals

.halt_if_o                      ( halt_if_o              ),

.halt_id_o                      ( halt_id                ),

.id_ready_i                     ( id_ready_o             ),

// Performance Counters

.perf_jump_o                    ( perf_jump_o            ),

.perf_tbranch_o                 ( perf_tbranch_o         )

);

////////////////////////////////////////////////////////////////////////

//  _____      _       _____             _             _ _            //

// |_   _|    | |     /  __ \           | |           | | |           //

//   | | _ __ | |_    | /  \/ ___  _ __ | |_ _ __ ___ | | | ___ _ __  //

//   | || '_ \| __|   | |    / _ \| '_ \| __| '__/ _ \| | |/ _ \ '__| //

//  _| || | | | |_ _  | \__/\ (_) | | | | |_| | | (_) | | |  __/ |    //

//  \___/_| |_|\__(_)  \____/\___/|_| |_|\__|_|  \___/|_|_|\___|_|    //

//                                                                    //

////////////////////////////////////////////////////////////////////////

zeroriscy_int_controller int_controller_i

(

.clk                  ( clk                ),

.rst_n                ( rst_n              ),

// to controller

.irq_req_ctrl_o       ( irq_req_ctrl       ),

.irq_id_ctrl_o        ( irq_id_ctrl        ),

.ctrl_ack_i           ( exc_ack            ),

.ctrl_kill_i          ( exc_kill           ),

// Interrupt signals

.irq_i                ( irq_i              ),

.irq_id_i             ( irq_id_i           ),

.m_IE_i               ( m_irq_enable_i     )

);

/////////////////////////////////////

//   ___ ____        _______  __   //

//  |_ _|  _ \      | ____\ \/ /   //

//   | || | | |_____|  _|  \  /    //

//   | || |_| |_____| |___ /  \    //

//  |___|____/      |_____/_/\_\   //

//                                 //

/////////////////////////////////////

assign data_we_ex_o                = data_we_id;

assign data_type_ex_o              = data_type_id;

assign data_sign_ext_ex_o          = data_sign_ext_id;

assign data_wdata_ex_o             = regfile_data_rb_id;

assign data_req_ex_o               = data_req_id;

assign data_reg_offset_ex_o        = data_reg_offset_id;

assign data_load_event_ex_o        = data_load_event_id;

assign alu_operator_ex_o           = alu_operator;

assign alu_operand_a_ex_o          = alu_operand_a;

assign alu_operand_b_ex_o          = alu_operand_b;

assign csr_access_ex_o             = csr_access;

assign csr_op_ex_o                 = csr_op;

assign branch_in_ex_o              = branch_in_id;

assign mult_en_ex_o                = mult_int_en;

assign div_en_ex_o                 = div_int_en;

assign multdiv_operator_ex_o       = multdiv_operator;

assign multdiv_signed_mode_ex_o    = multdiv_signed_mode;

assign multdiv_operand_a_ex_o      = regfile_data_ra_id;

assign multdiv_operand_b_ex_o      = regfile_data_rb_id;

enum logic { IDLE, WAIT_MULTICYCLE } id_wb_fsm_cs, id_wb_fsm_ns;

///////////////////////////////////////

// ID-EX/WB Pipeline Register        //

///////////////////////////////////////

always_ff @(posedge clk, negedge rst_n)

begin : EX_WB_Pipeline_Register

if (~rst_n)

begin

id_wb_fsm_cs  <= IDLE;

branch_set_q  <= 1'b0;

end

else begin

id_wb_fsm_cs  <= id_wb_fsm_ns;

branch_set_q  <= branch_set_n;

end

end

///////////////////////////////////////

// ID-EX/WB FMS                      //

///////////////////////////////////////

always_comb

begin

id_wb_fsm_ns    = id_wb_fsm_cs;

regfile_we      = regfile_we_id;

load_stall      = 1'b0;

multdiv_stall   = 1'b0;

jump_stall      = 1'b0;

branch_stall    = 1'b0;

select_data_rf  = RF_EX;

instr_multicyle = 1'b0;

branch_set_n    = 1'b0;

branch_mux_dec  = 1'b0;

jump_set        = 1'b0;

jump_mux_dec    = 1'b0;

perf_branch_o   = 1'b0;

unique case (id_wb_fsm_cs)

IDLE:

begin

jump_mux_dec   = 1'b1;

branch_mux_dec = 1'b1;

unique case (1'b1)

data_req_id: begin

//LSU operation

regfile_we      = 1'b0;

id_wb_fsm_ns    = WAIT_MULTICYCLE;

load_stall      = 1'b1;

instr_multicyle = 1'b1;

end

branch_in_id: begin

//Cond Branch operation

id_wb_fsm_ns    = branch_decision_i ? WAIT_MULTICYCLE : IDLE;

branch_stall    = branch_decision_i;

instr_multicyle = branch_decision_i;

branch_set_n    = branch_decision_i;

perf_branch_o   = 1'b1;

end

multdiv_int_en: begin

//MUL or DIV operation

regfile_we      = 1'b0;

id_wb_fsm_ns    = WAIT_MULTICYCLE;

multdiv_stall   = 1'b1;

instr_multicyle = 1'b1;

end

jump_in_id: begin

//UnCond Branch operation

regfile_we      = 1'b0;

id_wb_fsm_ns    = WAIT_MULTICYCLE;

jump_stall      = 1'b1;

instr_multicyle = 1'b1;

jump_set        = 1'b1;

end

default:;

endcase

end

WAIT_MULTICYCLE:

begin

if(ex_ready_i) begin

regfile_we     = regfile_we_id;

id_wb_fsm_ns   = IDLE;

load_stall     = 1'b0;

multdiv_stall  = 1'b0;

select_data_rf = data_req_id ? RF_LSU : RF_EX;

end else begin

regfile_we      = 1'b0;

instr_multicyle = 1'b1;

unique case (1'b1)

data_req_id:

load_stall    = 1'b1;

multdiv_int_en:

multdiv_stall = 1'b1;

default:;

endcase

end

end

default:;

endcase

end

// stall control

assign id_ready_o = (~load_stall) & (~branch_stall) & (~jump_stall) & (~multdiv_stall);

assign id_valid_o = (~halt_id) & id_ready_o;

//----------------------------------------------------------------------------

// Assertions

//----------------------------------------------------------------------------

`ifndef VERILATOR

// make sure that branch decision is valid when jumping

assert property (

@(posedge clk) (branch_decision_i !== 1'bx || branch_in_id == 1'b0) ) else begin $display("Branch decision is X"); $stop; end

`ifdef CHECK_MISALIGNED

assert property (

@(posedge clk) (~data_misaligned_i) ) else $display("Misaligned memory access at %x",pc_id_i);

`endif

// the instruction delivered to the ID stage should always be valid

assert property (

@(posedge clk) (instr_valid_i & (~illegal_c_insn_i)) |-> (!$isunknown(instr_rdata_i)) ) else $display("Instruction is valid, but has at least one X");

// make sure multicycles enable signals are unique

assert property (

@(posedge clk) ~(data_req_ex_o & multdiv_int_en )) else $display("Multicycles enable signals are not unique");

`endif

Endmodule

`include "zeroriscy_config.sv"

import zeroriscy_defines::*;

module zeroriscy_if_stage

(

input  logic        clk,

input  logic        rst_n,

// the boot address is used to calculate the exception offsets

input  logic [31:0] boot_addr_i,

// instruction request control

input  logic        req_i,

// instruction cache interface

output logic                   instr_req_o,

output logic            [31:0] instr_addr_o,

input  logic                   instr_gnt_i,

input  logic                   instr_rvalid_i,

input  logic            [31:0] instr_rdata_i,

// Output of IF Pipeline stage

output logic              instr_valid_id_o,      // instruction in IF/ID pipeline is valid

output logic       [31:0] instr_rdata_id_o,      // read instruction is sampled and sent to ID stage for decoding

output logic              is_compressed_id_o,    // compressed decoder thinks this is a compressed instruction

output logic              illegal_c_insn_id_o,   // compressed decoder thinks this is an invalid instruction

output logic       [31:0] pc_if_o,

output logic       [31:0] pc_id_o,

// Forwarding ports - control signals

input  logic        clear_instr_valid_i,   // clear instruction valid bit in IF/ID pipe

input  logic        pc_set_i,              // set the program counter to a new value

input  logic [31:0] exception_pc_reg_i,    // address used to restore PC when the interrupt/exception is served

input  logic  [2:0] pc_mux_i,              // sel for pc multiplexer

input  logic  [1:0] exc_pc_mux_i,          // selects ISR address

input  logic  [4:0] exc_vec_pc_mux_i,      // selects ISR address for vectorized interrupt lines

// jump and branch target and decision

input  logic [31:0] jump_target_ex_i,      // jump target address

// from debug unit

input  logic [31:0] dbg_jump_addr_i,

// pipeline stall

input  logic        halt_if_i,

input  logic        id_ready_i,

output logic        if_valid_o,

// misc signals

output logic        if_busy_o,             // is the IF stage busy fetching instructions?

output logic        perf_imiss_o           // Instruction Fetch Miss

);

// offset FSM

enum logic[0:0] {WAIT, IDLE } offset_fsm_cs, offset_fsm_ns;

logic              valid;

logic              if_ready;

// prefetch buffer related signals

logic              prefetch_busy;

logic              branch_req;

logic       [31:0] fetch_addr_n;

logic              fetch_valid;

logic              fetch_ready;

logic       [31:0] fetch_rdata;

logic       [31:0] fetch_addr;

logic       [31:0] exc_pc;

// exception PC selection mux

always_comb

begin : EXC_PC_MUX

exc_pc = '0;

unique case (exc_pc_mux_i)

EXC_PC_ILLINSN: exc_pc = { boot_addr_i[31:8], EXC_OFF_ILLINSN };

EXC_PC_ECALL:   exc_pc = { boot_addr_i[31:8], EXC_OFF_ECALL   };

EXC_PC_IRQ:     exc_pc = { boot_addr_i[31:8], 1'b0, exc_vec_pc_mux_i[4:0], 2'b0 };

// TODO: Add case for EXC_PC_STORE and EXC_PC_LOAD as soon as they are supported

default:;

endcase

end

// fetch address selection

always_comb

begin

fetch_addr_n = '0;

unique case (pc_mux_i)

PC_BOOT:      fetch_addr_n = {boot_addr_i[31:8], EXC_OFF_RST};

PC_JUMP:      fetch_addr_n = jump_target_ex_i;

PC_EXCEPTION: fetch_addr_n = exc_pc;             // set PC to exception handler

PC_ERET:      fetch_addr_n = exception_pc_reg_i; // PC is restored when returning from IRQ/exception

PC_DBG_NPC:   fetch_addr_n = dbg_jump_addr_i;    // PC is taken from debug unit

default:;

endcase

end

// prefetch buffer, caches a fixed number of instructions

zeroriscy_prefetch_buffer prefetch_buffer_i

(

.clk               ( clk                         ),

.rst_n             ( rst_n                       ),

.req_i             ( req_i                       ),

.branch_i          ( branch_req                  ),

.addr_i            ( {fetch_addr_n[31:1], 1'b0}  ),

.ready_i           ( fetch_ready                 ),

.valid_o           ( fetch_valid                 ),

.rdata_o           ( fetch_rdata                 ),

.addr_o            ( fetch_addr                  ),

// goes to instruction memory / instruction cache

.instr_req_o       ( instr_req_o                 ),

.instr_addr_o      ( instr_addr_o                ),

.instr_gnt_i       ( instr_gnt_i                 ),

.instr_rvalid_i    ( instr_rvalid_i              ),

.instr_rdata_i     ( instr_rdata_i               ),

// Prefetch Buffer Status

.busy_o            ( prefetch_busy               )

);

// offset FSM state

always_ff @(posedge clk, negedge rst_n)

begin

if (rst_n == 1'b0) begin

offset_fsm_cs     <= IDLE;

end else begin

offset_fsm_cs     <= offset_fsm_ns;

end

end

// offset FSM state transition logic

always_comb

begin

offset_fsm_ns = offset_fsm_cs;

fetch_ready   = 1'b0;

branch_req    = 1'b0;

valid         = 1'b0;

unique case (offset_fsm_cs)

// no valid instruction data for ID stage

// assume aligned

IDLE: begin

if (req_i) begin

branch_req    = 1'b1;

offset_fsm_ns = WAIT;

end

end

// serving aligned 32 bit or 16 bit instruction, we don't know yet

WAIT: begin

if (fetch_valid) begin

valid   = 1'b1; // an instruction is ready for ID stage

if (req_i && if_valid_o) begin

fetch_ready   = 1'b1;

offset_fsm_ns = WAIT;

end

end

end

default: begin

offset_fsm_ns = IDLE;

end

endcase

// take care of jumps and branches

if (pc_set_i) begin

valid = 1'b0;

// switch to new PC from ID stage

branch_req = 1'b1;

offset_fsm_ns = WAIT;

end

end

assign pc_if_o         = fetch_addr;

assign if_busy_o       = prefetch_busy;

assign perf_imiss_o    = (~fetch_valid) | branch_req;

// compressed instruction decoding, or more precisely compressed instruction

// expander

//

// since it does not matter where we decompress instructions, we do it here

// to ease timing closure

logic [31:0] instr_decompressed;

logic        illegal_c_insn;

logic        instr_compressed_int;

zeroriscy_compressed_decoder compressed_decoder_i

(

.instr_i         ( fetch_rdata          ),

.instr_o         ( instr_decompressed   ),

.is_compressed_o ( instr_compressed_int ),

.illegal_instr_o ( illegal_c_insn       )

);

// IF-ID pipeline registers, frozen when the ID stage is stalled

always_ff @(posedge clk, negedge rst_n)

begin : IF_ID_PIPE_REGISTERS

if (rst_n == 1'b0)

begin

instr_valid_id_o      <= 1'b0;

instr_rdata_id_o      <= '0;

illegal_c_insn_id_o   <= 1'b0;

is_compressed_id_o    <= 1'b0;

pc_id_o               <= '0;

end

else

begin

if (if_valid_o)

begin

instr_valid_id_o    <= 1'b1;

instr_rdata_id_o    <= instr_decompressed;

illegal_c_insn_id_o <= illegal_c_insn;

is_compressed_id_o  <= instr_compressed_int;

pc_id_o             <= pc_if_o;

end else if (clear_instr_valid_i) begin

instr_valid_id_o    <= 1'b0;

end

end

end

assign if_ready = valid & id_ready_i;

assign if_valid_o = (~halt_if_i) & if_ready;

//----------------------------------------------------------------------------

// Assertions

//----------------------------------------------------------------------------

`ifndef VERILATOR

// there should never be a grant when there is no request

assert property (

@(posedge clk) (instr_gnt_i) |-> (instr_req_o) )

else $warning("There was a grant without a request");

`endif

Endmodule

import zeroriscy_defines::*;

module zeroriscy_int_controller

(

input  logic        clk,

input  logic        rst_n,

// irq_req for controller

output logic        irq_req_ctrl_o,

output logic  [4:0] irq_id_ctrl_o,

// handshake signals to controller

input  logic        ctrl_ack_i,

input  logic        ctrl_kill_i,

// external interrupt lines

input  logic        irq_i,          // level-triggered interrupt inputs

input  logic  [4:0] irq_id_i,       // interrupt id [0,1,....31]

input  logic        m_IE_i          // interrupt enable bit from CSR (M mode)

);

enum logic [1:0] { IDLE, IRQ_PENDING, IRQ_DONE} exc_ctrl_cs, exc_ctrl_ns;

logic irq_enable_ext;

logic [4:0] irq_id_q;

assign irq_enable_ext =  m_IE_i;

assign irq_req_ctrl_o = exc_ctrl_cs == IRQ_PENDING;

assign irq_id_ctrl_o  = irq_id_q;

always_ff @(posedge clk, negedge rst_n)

begin

if (rst_n == 1'b0) begin

irq_id_q    <= '0;

exc_ctrl_cs <= IDLE;

end else begin

unique case (exc_ctrl_cs)

IDLE:

begin

if(irq_enable_ext & irq_i) begin

exc_ctrl_cs <= IRQ_PENDING;

irq_id_q    <= irq_id_i;

end

end

IRQ_PENDING:

begin

unique case(1'b1)

ctrl_ack_i:

exc_ctrl_cs <= IRQ_DONE;

ctrl_kill_i:

exc_ctrl_cs <= IDLE;

default:

exc_ctrl_cs <= IRQ_PENDING;

endcase

end

IRQ_DONE:

begin

exc_ctrl_cs <= IDLE;

end

endcase

end

end

`ifndef SYNTHESIS

// synopsys translate_off

// evaluate at falling edge to avoid duplicates during glitches

// Removed this message as it pollutes too much the output and makes tests fail

//always_ff @(negedge clk)

//begin

//  if (rst_n && exc_ctrl_cs == IRQ_DONE)

//    $display("%t: Entering interrupt service routine. [%m]", $time);

//end

// synopsys translate_on

`endif

Endmodule

`include "zeroriscy_config.sv"

module zeroriscy_load_store_unit

(

input  logic         clk,

input  logic         rst_n,

// output to data memory

output logic         data_req_o,

input  logic         data_gnt_i,

input  logic         data_rvalid_i,

input  logic         data_err_i,

output logic [31:0]  data_addr_o,

output logic         data_we_o,

output logic [3:0]   data_be_o,

output logic [31:0]  data_wdata_o,

input  logic [31:0]  data_rdata_i,

// signals from ex stage

input  logic         data_we_ex_i,         // write enable                      -> from ex stage

input  logic [1:0]   data_type_ex_i,       // Data type word, halfword, byte    -> from ex stage

input  logic [31:0]  data_wdata_ex_i,      // data to write to memory           -> from ex stage

input  logic [1:0]   data_reg_offset_ex_i, // offset inside register for stores -> from ex stage

input  logic         data_sign_ext_ex_i,   // sign extension                    -> from ex stage

output logic [31:0]  data_rdata_ex_o,      // requested data                    -> to ex stage

input  logic         data_req_ex_i,        // data request                      -> from ex stage

input  logic [31:0]  adder_result_ex_i,

output logic         data_misaligned_o,    // misaligned access was detected    -> to controller

output logic [31:0]  misaligned_addr_o,

// exception signals

output logic         load_err_o,

output logic         store_err_o,

// stall signal

output logic         lsu_update_addr_o, // LSU ready for new data in EX stage

output logic         data_valid_o,

output logic         busy_o

);

logic [31:0]  data_addr_int;

// registers for data_rdata alignment and sign extension

logic [1:0]   data_type_q;

logic [1:0]   rdata_offset_q;

logic         data_sign_ext_q;

logic         data_we_q;

logic [1:0]   wdata_offset;   // mux control for data to be written to memory

logic [3:0]   data_be;

logic [31:0]  data_wdata;

logic         misaligned_st;   // high if we are currently performing the second part of a misaligned store

logic         data_misaligned, data_misaligned_q;

logic         increase_address;

enum logic [2:0]  { IDLE, WAIT_GNT_MIS, WAIT_RVALID_MIS, WAIT_GNT, WAIT_RVALID } CS, NS;

logic [31:0]  rdata_q;

///////////////////////////////// BE generation ////////////////////////////////

always_comb

begin

case (data_type_ex_i) // Data type 00 Word, 01 Half word, 11,10 byte

2'b00:

begin // Writing a word

if (misaligned_st == 1'b0)

begin // non-misaligned case

case (data_addr_int[1:0])

2'b00: data_be = 4'b1111;

2'b01: data_be = 4'b1110;

2'b10: data_be = 4'b1100;

2'b11: data_be = 4'b1000;

endcase; // case (data_addr_int[1:0])

end

else

begin // misaligned case

case (data_addr_int[1:0])

2'b00: data_be = 4'b0000; // this is not used, but included for completeness

2'b01: data_be = 4'b0001;

2'b10: data_be = 4'b0011;

2'b11: data_be = 4'b0111;

endcase; // case (data_addr_int[1:0])

end

end

2'b01:

begin // Writing a half word

if (misaligned_st == 1'b0)

begin // non-misaligned case

case (data_addr_int[1:0])

2'b00: data_be = 4'b0011;

2'b01: data_be = 4'b0110;

2'b10: data_be = 4'b1100;

2'b11: data_be = 4'b1000;

endcase; // case (data_addr_int[1:0])

end

else

begin // misaligned case

data_be = 4'b0001;

end

end

2'b10,

2'b11: begin // Writing a byte

case (data_addr_int[1:0])

2'b00: data_be = 4'b0001;

2'b01: data_be = 4'b0010;

2'b10: data_be = 4'b0100;

2'b11: data_be = 4'b1000;

endcase; // case (data_addr_int[1:0])

end

endcase; // case (data_type_ex_i)

end

// prepare data to be written to the memory

// we handle misaligned accesses, half word and byte accesses and

// register offsets here

assign wdata_offset = data_addr_int[1:0] - data_reg_offset_ex_i[1:0];

always_comb

begin

case (wdata_offset)

2'b00: data_wdata = data_wdata_ex_i[31:0];

2'b01: data_wdata = {data_wdata_ex_i[23:0], data_wdata_ex_i[31:24]};

2'b10: data_wdata = {data_wdata_ex_i[15:0], data_wdata_ex_i[31:16]};

2'b11: data_wdata = {data_wdata_ex_i[ 7:0], data_wdata_ex_i[31: 8]};

endcase; // case (wdata_offset)

end

// FF for rdata alignment and sign-extension

always_ff @(posedge clk, negedge rst_n)

begin

if(rst_n == 1'b0)

begin

data_type_q     <= '0;

rdata_offset_q  <= '0;

data_sign_ext_q <= '0;

data_we_q       <= 1'b0;

end

else if (data_gnt_i == 1'b1) // request was granted, we wait for rvalid and can continue to WB

begin

data_type_q     <= data_type_ex_i;

rdata_offset_q  <= data_addr_int[1:0];

data_sign_ext_q <= data_sign_ext_ex_i;

data_we_q       <= data_we_ex_i;

end

end

////////////////////////////////////////////////////////////////////////

//  ____  _               _____      _                 _              //

// / ___|(_) __ _ _ __   | ____|_  _| |_ ___ _ __  ___(_) ___  _ __   //

// \___ \| |/ _` | '_ \  |  _| \ \/ / __/ _ \ '_ \/ __| |/ _ \| '_ \  //

//  ___) | | (_| | | | | | |___ >  <| ||  __/ | | \__ \ | (_) | | | | //

// |____/|_|\__, |_| |_| |_____/_/\_\\__\___|_| |_|___/_|\___/|_| |_| //

//          |___/                                                     //

////////////////////////////////////////////////////////////////////////

logic [31:0] data_rdata_ext;

logic [31:0] rdata_w_ext; // sign extension for words, actually only misaligned assembly

logic [31:0] rdata_h_ext; // sign extension for half words

logic [31:0] rdata_b_ext; // sign extension for bytes

// take care of misaligned words

always_comb

begin

case (rdata_offset_q)

2'b00: rdata_w_ext = data_rdata_i[31:0];

2'b01: rdata_w_ext = {data_rdata_i[ 7:0], rdata_q[31:8]};

2'b10: rdata_w_ext = {data_rdata_i[15:0], rdata_q[31:16]};

2'b11: rdata_w_ext = {data_rdata_i[23:0], rdata_q[31:24]};

endcase

end

// sign extension for half words

always_comb

begin

case (rdata_offset_q)

2'b00:

begin

if (data_sign_ext_q == 1'b0)

rdata_h_ext = {16'h0000, data_rdata_i[15:0]};

else

rdata_h_ext = {{16{data_rdata_i[15]}}, data_rdata_i[15:0]};

end

2'b01:

begin

if (data_sign_ext_q == 1'b0)

rdata_h_ext = {16'h0000, data_rdata_i[23:8]};

else

rdata_h_ext = {{16{data_rdata_i[23]}}, data_rdata_i[23:8]};

end

2'b10:

begin

if (data_sign_ext_q == 1'b0)

rdata_h_ext = {16'h0000, data_rdata_i[31:16]};

else

rdata_h_ext = {{16{data_rdata_i[31]}}, data_rdata_i[31:16]};

end

2'b11:

begin

if (data_sign_ext_q == 1'b0)

rdata_h_ext = {16'h0000, data_rdata_i[7:0], rdata_q[31:24]};

else

rdata_h_ext = {{16{data_rdata_i[7]}}, data_rdata_i[7:0], rdata_q[31:24]};

end

endcase // case (rdata_offset_q)

end

// sign extension for bytes

always_comb

begin

case (rdata_offset_q)

2'b00:

begin

if (data_sign_ext_q == 1'b0)

rdata_b_ext = {24'h00_0000, data_rdata_i[7:0]};

else

rdata_b_ext = {{24{data_rdata_i[7]}}, data_rdata_i[7:0]};

end

2'b01: begin

if (data_sign_ext_q == 1'b0)

rdata_b_ext = {24'h00_0000, data_rdata_i[15:8]};

else

rdata_b_ext = {{24{data_rdata_i[15]}}, data_rdata_i[15:8]};

end

2'b10:

begin

if (data_sign_ext_q == 1'b0)

rdata_b_ext = {24'h00_0000, data_rdata_i[23:16]};

else

rdata_b_ext = {{24{data_rdata_i[23]}}, data_rdata_i[23:16]};

end

2'b11:

begin

if (data_sign_ext_q == 1'b0)

rdata_b_ext = {24'h00_0000, data_rdata_i[31:24]};

else

rdata_b_ext = {{24{data_rdata_i[31]}}, data_rdata_i[31:24]};

end

endcase // case (rdata_offset_q)

end

// select word, half word or byte sign extended version

always_comb

begin

case (data_type_q)

2'b00:       data_rdata_ext = rdata_w_ext;

2'b01:       data_rdata_ext = rdata_h_ext;

2'b10,2'b11: data_rdata_ext = rdata_b_ext;

endcase //~case(rdata_type_q)

end

always_ff @(posedge clk, negedge rst_n)

begin

if(rst_n == 1'b0)

begin

CS            <= IDLE;

rdata_q       <= '0;

data_misaligned_q <= '0;

misaligned_addr_o <= 32'b0;

end

else

begin

CS            <= NS;

if (lsu_update_addr_o) begin

data_misaligned_q <= data_misaligned;

if(increase_address) begin

misaligned_addr_o <= data_addr_int;

end

end

if (data_rvalid_i && (~data_we_q))

begin

// if we have detected a misaligned access, and we are

// currently doing the first part of this access, then

// store the data coming from memory in rdata_q.

// In all other cases, rdata_q gets the value that we are

// writing to the register file

if ((data_misaligned_q == 1'b1) || (data_misaligned == 1'b1))

rdata_q  <= data_rdata_i;

else

rdata_q  <= data_rdata_ext;

end

end

end

// output to register file

assign data_rdata_ex_o = (data_rvalid_i == 1'b1) ? data_rdata_ext : rdata_q;

// output to data interface

assign data_addr_o   = data_addr_int;

assign data_wdata_o  = data_wdata;

assign data_we_o     = data_we_ex_i;

assign data_be_o     = data_be;

assign misaligned_st = data_misaligned_q;

assign load_err_o    = 1'b0;

assign store_err_o   = 1'b0;

// FSM

always_comb

begin

NS             = CS;

data_req_o     = 1'b0;

lsu_update_addr_o   = 1'b0;

data_valid_o     = 1'b0;

increase_address = 1'b0;

data_misaligned_o = 1'b0;

case(CS)

// starts from not active and stays in IDLE until request was granted

IDLE:

begin

if (data_req_ex_i) begin

data_req_o     = data_req_ex_i;

if(data_gnt_i) begin

lsu_update_addr_o   = 1'b1;

increase_address = data_misaligned;

NS = data_misaligned ? WAIT_RVALID_MIS : WAIT_RVALID;

end

else begin

NS = data_misaligned ? WAIT_GNT_MIS    : WAIT_GNT;

end

end

end //~ IDLE

WAIT_GNT_MIS:

begin

data_req_o = 1'b1;

if(data_gnt_i) begin

lsu_update_addr_o = 1'b1;

increase_address  = data_misaligned;

NS = WAIT_RVALID_MIS;

end

end //~ WAIT_GNT_MIS

// wait for rvalid in WB stage and send a new request if there is any

WAIT_RVALID_MIS:

begin

//increase_address goes down, we already have the proper address

increase_address  = 1'b0;

//tell the controller to update the address

data_misaligned_o = 1'b1;

data_req_o        = 1'b0;

lsu_update_addr_o = data_gnt_i;

if(data_rvalid_i) begin

//if first part rvalid is received

data_req_o        = 1'b1;

if(data_gnt_i) begin

//second grant is received

NS             = WAIT_RVALID;

//in this stage we already received the first valid but no the second one

//it differes from WAIT_RVALID_MIS because we do not send other requests

end

else begin

//second grant is NOT received, but first rvalid yes

//lsu_update_addr_o is 0 so data_misaligned_q stays high in WAIT_GNT

//increase address stays the same as well

NS              = WAIT_GNT; //  [1]

end

end

else begin

//if first part rvalid is NOT received

//the second grand is not received either by protocol.

//stay here

NS                 = WAIT_RVALID_MIS;

end

end

WAIT_GNT:

begin

data_misaligned_o = data_misaligned_q;

//useful in case [1]

data_req_o        = 1'b1;

if(data_gnt_i) begin

lsu_update_addr_o = 1'b1;

NS = WAIT_RVALID;

end

end //~ WAIT_GNT

WAIT_RVALID:

begin

data_req_o        = 1'b0;

if(data_rvalid_i) begin

data_valid_o = 1'b1;

NS           = IDLE;

end

else begin

NS           = WAIT_RVALID;

end

end //~ WAIT_RVALID

default: begin

NS = IDLE;

end

endcase

end

// check for misaligned accesses that need a second memory access

// If one is detected, this is signaled with data_misaligned_o to

// the controller which selectively stalls the pipeline

always_comb

begin

data_misaligned = 1'b0;

if((data_req_ex_i == 1'b1) && (data_misaligned_q == 1'b0))

begin

case (data_type_ex_i)

2'b00: // word

begin

if(data_addr_int[1:0] != 2'b00)

data_misaligned = 1'b1;

end

2'b01: // half word

begin

if(data_addr_int[1:0] == 2'b11)

data_misaligned = 1'b1;

end

default: ;

endcase // case (data_type_ex_i)

end

end

assign data_addr_int = adder_result_ex_i;

assign busy_o = (CS == WAIT_RVALID) || (data_req_o == 1'b1);

//////////////////////////////////////////////////////////////////////////////

// Assertions

//////////////////////////////////////////////////////////////////////////////

`ifndef VERILATOR

// make sure there is no new request when the old one is not yet completely done

// i.e. it should not be possible to get a grant without an rvalid for the

// last request

assert property (

@(posedge clk) ((CS == WAIT_RVALID) && (data_gnt_i == 1'b1)) |-> (data_rvalid_i == 1'b1) );

// there should be no rvalid when we are in IDLE

assert property (

@(posedge clk) (CS == IDLE) |-> (data_rvalid_i == 1'b0) );

// assert that errors are only sent at the same time as grant

assert property ( @(posedge clk) (data_err_i) |-> (data_gnt_i) );

// assert that the address does not contain X when request is sent

assert property ( @(posedge clk) (data_req_o) |-> (!$isunknown(data_addr_o)) );

`endif

Endmodule

import zeroriscy_defines::*;

`define OP_L 15:0

`define OP_H 31:16

module zeroriscy_multdiv_fast

(

input  logic        clk,

input  logic        rst_n,

input  logic        mult_en_i,

input  logic        div_en_i,

input  logic  [1:0] operator_i,

input  logic  [1:0] signed_mode_i,

input  logic [31:0] op_a_i,

input  logic [31:0] op_b_i,

input  logic [33:0] alu_adder_ext_i,

input  logic [31:0] alu_adder_i,

input  logic        equal_to_zero,

output logic [32:0] alu_operand_a_o,

output logic [32:0] alu_operand_b_o,

output logic [31:0] multdiv_result_o,

output logic        ready_o

);

logic [ 4:0] div_counter_q, div_counter_n;

enum logic [1:0] {ALBL, ALBH, AHBL, AHBH } mult_state_q, mult_state_n;

enum logic [2:0] { MD_IDLE, MD_ABS_A, MD_ABS_B, MD_COMP, MD_LAST, MD_CHANGE_SIGN, MD_FINISH } divcurr_state_q, divcurr_state_n;

logic [34:0] mac_res_ext;

logic [33:0] mac_res_q, mac_res_n, mac_res, op_reminder_n;

logic [15:0] mult_op_a;

logic [15:0] mult_op_b;

logic [33:0] accum;

logic        sign_a, sign_b;

logic        div_sign_a, div_sign_b;

logic        signed_mult;

logic        is_greater_equal;

logic        div_change_sign, rem_change_sign;

logic [31:0] one_shift;

logic [31:0] op_denominator_q;

logic [31:0] op_numerator_q;

logic [31:0] op_quotient_q;

logic [31:0] op_denominator_n;

logic [31:0] op_numerator_n;

logic [31:0] op_quotient_n;

logic [32:0] next_reminder, next_quotient;

logic [32:0] res_adder_h;

logic        mult_is_ready;

always_ff @(posedge clk or negedge rst_n) begin : proc_mult_state_q

if(~rst_n) begin

mult_state_q      <= ALBL;

mac_res_q         <= '0;

div_counter_q     <= '0;

divcurr_state_q   <= MD_IDLE;

op_denominator_q  <= '0;

op_numerator_q    <= '0;

op_quotient_q     <= '0;

end else begin

if(mult_en_i) begin

mult_state_q <= mult_state_n;

end

if(div_en_i) begin

div_counter_q     <= div_counter_n;

op_denominator_q  <= op_denominator_n  ;

op_numerator_q    <= op_numerator_n    ;

op_quotient_q     <= op_quotient_n     ;

divcurr_state_q   <= divcurr_state_n;

end

unique case(1'b1)

mult_en_i:

mac_res_q <= mac_res_n;

div_en_i:

mac_res_q <= op_reminder_n;

default:

mac_res_q <= mac_res_q;

endcase

end

end

assign signed_mult    = (signed_mode_i != 2'b00);

assign multdiv_result_o = div_en_i ? mac_res_q[31:0] : mac_res_n[31:0];

assign mac_res_ext = $signed({sign_a, mult_op_a})*$signed({sign_b, mult_op_b}) + $signed(accum);

assign mac_res     = mac_res_ext[33:0];

assign res_adder_h   = alu_adder_ext_i[33:1];

assign next_reminder = is_greater_equal ? res_adder_h                   : mac_res_q[32:0];

assign next_quotient = is_greater_equal ? op_quotient_q | one_shift     : op_quotient_q;

assign one_shift     = {31'b0, 1'b1} << div_counter_q;

/*

The adder in the ALU computes alu_operand_a_o + alu_operand_b_o which means

Reminder - Divisor. If Reminder - Divisor >= 0, is_greater_equal is equal to 1,

the next Reminder is Reminder - Divisor contained in res_adder_h and the

*/

always_comb

begin

if ((mac_res_q[31] ^ op_denominator_q[31]) == 0)

is_greater_equal = (res_adder_h[31] == 0);

else

is_greater_equal = mac_res_q[31];

end

assign div_sign_a   = op_a_i[31] & signed_mode_i[0];

assign div_sign_b   = op_b_i[31] & signed_mode_i[1];

assign div_change_sign  = div_sign_a ^ div_sign_b;

assign rem_change_sign  = div_sign_a;

always_comb

begin : div_fsm

div_counter_n    = div_counter_q - 1;

op_reminder_n    = mac_res_q;

op_quotient_n    = op_quotient_q;

divcurr_state_n  = divcurr_state_q;

op_numerator_n   = op_numerator_q;

op_denominator_n = op_denominator_q;

alu_operand_a_o  = {32'h0  , 1'b1};

alu_operand_b_o  = {~op_b_i, 1'b1};

unique case(divcurr_state_q)

MD_IDLE: begin

unique case(operator_i)

MD_OP_DIV: begin

//Check if the Denominator is 0

//quotient for division by 0

op_reminder_n    = '1;

divcurr_state_n  = equal_to_zero ? MD_FINISH : MD_ABS_A;

end

default: begin

//Check if the Denominator is 0

//reminder for division by 0

op_reminder_n     = {2'b0, op_a_i};

divcurr_state_n    = equal_to_zero ? MD_FINISH : MD_ABS_A;

end

endcase

//0 - B = 0 iff B == 0

alu_operand_a_o     = {32'h0  , 1'b1};

alu_operand_b_o     = {~op_b_i, 1'b1};

div_counter_n        = 5'd31;

end

MD_ABS_A: begin

//quotient

op_quotient_n  = '0;

//A abs value

op_numerator_n = div_sign_a ? alu_adder_i : op_a_i;

divcurr_state_n   = MD_ABS_B;

div_counter_n     = 5'd31;

//ABS(A) = 0 - A

alu_operand_a_o     = {32'h0  , 1'b1};

alu_operand_b_o     = {~op_a_i, 1'b1};

end

MD_ABS_B: begin

//reminder

op_reminder_n    = { 33'h0, op_numerator_q[31]};

//B abs value

op_denominator_n  = div_sign_b ? alu_adder_i : op_b_i;

divcurr_state_n   = MD_COMP;

div_counter_n     = 5'd31;

//ABS(B) = 0 - B

alu_operand_a_o     = {32'h0  , 1'b1};

alu_operand_b_o     = {~op_b_i, 1'b1};

end

MD_COMP: begin

op_reminder_n     = {1'b0, next_reminder[31:0], op_numerator_q[div_counter_n]};

op_quotient_n     = next_quotient;

if(div_counter_q == 5'd1)

divcurr_state_n = MD_LAST;

else

divcurr_state_n = MD_COMP;

//Division

alu_operand_a_o     = {mac_res_q[31:0], 1'b1}; //it contains the reminder

alu_operand_b_o     = {~op_denominator_q[31:0], 1'b1}; //denominator negated + 1 to do -denominator

end

MD_LAST: begin

unique case(operator_i)

MD_OP_DIV:

//this time we save the quotient in op_reminder_n (i.e. mac_res_q) since we do not need anymore the reminder

op_reminder_n  = {1'b0, next_quotient};

default:

//this time we do not save the quotient anymore since we need only the reminder

op_reminder_n  = {2'b0, next_reminder[31:0]};

endcase

//Division

alu_operand_a_o     = {mac_res_q[31:0], 1'b1}; //it contains the reminder

alu_operand_b_o     = {~op_denominator_q[31:0], 1'b1}; //denominator negated + 1 to do -denominator

divcurr_state_n = MD_CHANGE_SIGN;

end

MD_CHANGE_SIGN: begin

divcurr_state_n  = MD_FINISH;

unique case(operator_i)

MD_OP_DIV:

op_reminder_n = (div_change_sign) ? alu_adder_i : mac_res_q;

default:

op_reminder_n = (rem_change_sign) ? alu_adder_i : mac_res_q;

endcase

//ABS(Quotient) = 0 - Quotient (or Reminder)

alu_operand_a_o     = {32'h0  , 1'b1};

alu_operand_b_o     = {~mac_res_q[31:0], 1'b1};

end

MD_FINISH: begin

divcurr_state_n = MD_IDLE;

end

default:;

endcase // divcurr_state_q

end

assign ready_o  = mult_is_ready | (divcurr_state_q == MD_FINISH);

always_comb

begin : mult_fsm

mult_op_a    = op_a_i[`OP_L];

mult_op_b    = op_b_i[`OP_L];

sign_a       = 1'b0;

sign_b       = 1'b0;

accum        = mac_res_q;

mac_res_n    = mac_res;

mult_state_n = mult_state_q;

mult_is_ready = 1'b0;

unique case (mult_state_q)

ALBL: begin

//al*bl

mult_op_a = op_a_i[`OP_L];

mult_op_b = op_b_i[`OP_L];

sign_a    = 1'b0;

sign_b    = 1'b0;

accum     = '0;

mac_res_n = mac_res;

mult_state_n = ALBH;

end

ALBH: begin

//al*bh<<16

mult_op_a = op_a_i[`OP_L];

mult_op_b = op_b_i[`OP_H];

sign_a    = 1'b0;

sign_b    = signed_mode_i[1] & op_b_i[31];

//result of AL*BL (in mac_res_q) always unsigned with no carry, so carries_q always 00

accum     = {18'b0,mac_res_q[31:16]};

unique case(operator_i)

MD_OP_MULL: begin

mac_res_n = {2'b0,mac_res[`OP_L],mac_res_q[`OP_L]};

end

default: begin

//MD_OP_MULH

mac_res_n = mac_res;

end

endcase

mult_state_n = AHBL;

end

AHBL: begin

//ah*bl<<16

mult_op_a = op_a_i[`OP_H];

mult_op_b = op_b_i[`OP_L];

sign_a    = signed_mode_i[0] & op_a_i[31];

sign_b    = 1'b0;

unique case(operator_i)

MD_OP_MULL: begin

accum        = {18'b0,mac_res_q[31:16]};

mac_res_n    = {2'b0,mac_res[15:0],mac_res_q[15:0]};

mult_is_ready = 1'b1;

mult_state_n = ALBL;

end

default: begin

accum        = mac_res_q;

mac_res_n    = mac_res;

mult_state_n = AHBH;

end

endcase

end

AHBH: begin

//only MD_OP_MULH here

//ah*bh

mult_op_a = op_a_i[`OP_H];

mult_op_b = op_b_i[`OP_H];

sign_a    = signed_mode_i[0] & op_a_i[31];

sign_b    = signed_mode_i[1] & op_b_i[31];

accum[17:0 ]  = mac_res_q[33:16];

accum[33:18]  = {18{signed_mult & mac_res_q[33]}};

//result of AH*BL is not signed only if signed_mode_i == 2'b00

mac_res_n    = mac_res;

mult_state_n = ALBL;

mult_is_ready = 1'b1;

end

default:;

endcase // mult_state_q

end

endmodule // zeroriscy_mult

import zeroriscy_defines::*;

module zeroriscy_multdiv_slow

(

input  logic        clk,

input  logic        rst_n,

input  logic        mult_en_i,

input  logic        div_en_i,

input  logic  [1:0] operator_i,

input  logic  [1:0] signed_mode_i,

input  logic [31:0] op_a_i,

input  logic [31:0] op_b_i,

input  logic [33:0] alu_adder_ext_i,

input  logic [31:0] alu_adder_i,

input  logic        equal_to_zero,

output logic [32:0] alu_operand_a_o,

output logic [32:0] alu_operand_b_o,

output logic [31:0] multdiv_result_o,

output logic        ready_o

);

logic [ 4:0] multdiv_state_q, multdiv_state_n;

enum logic [2:0] { MD_IDLE, MD_ABS_A, MD_ABS_B, MD_COMP, MD_LAST, MD_CHANGE_SIGN, MD_FINISH } curr_state_q;

logic [32:0] accum_window_q;

logic [32:0] res_adder_l;

logic [32:0] res_adder_h;

logic [32:0] op_b_shift_q;

logic [32:0] op_a_shift_q;

logic [32:0] op_a_ext, op_b_ext;

logic [32:0] one_shift;

logic [32:0] op_a_bw_pp, op_a_bw_last_pp;

logic [31:0] b_0;

logic        sign_a, sign_b;

logic [32:0] next_reminder, next_quotient;

logic [32:0] op_remainder;

logic [31:0] op_numerator_q;

logic        is_greater_equal;

logic        div_change_sign, rem_change_sign;

//(accum_window_q + op_a_shift_q)

assign res_adder_l       = alu_adder_ext_i[32:0];

//(accum_window_q + op_a_shift_q)>>1

assign res_adder_h       = alu_adder_ext_i[33:1];

always_comb

begin

alu_operand_a_o   = accum_window_q;

multdiv_result_o  = div_en_i ? accum_window_q[31:0] : res_adder_l;

unique case(operator_i)

MD_OP_MULL: begin

alu_operand_b_o   = op_a_bw_pp;

end

MD_OP_MULH: begin

if(curr_state_q == MD_LAST)

alu_operand_b_o   = op_a_bw_last_pp;

else

alu_operand_b_o   = op_a_bw_pp;

end

default: begin

unique case(curr_state_q)

MD_IDLE: begin

//0 - B = 0 iff B == 0

alu_operand_a_o     = {32'h0  , 1'b1};

alu_operand_b_o     = {~op_b_i, 1'b1};

end

MD_ABS_A: begin

//ABS(A) = 0 - A

alu_operand_a_o     = {32'h0  , 1'b1};

alu_operand_b_o     = {~op_a_i, 1'b1};

end

MD_ABS_B: begin

//ABS(B) = 0 - B

alu_operand_a_o     = {32'h0  , 1'b1};

alu_operand_b_o     = {~op_b_i, 1'b1};

end

MD_CHANGE_SIGN: begin

//ABS(Quotient) = 0 - Quotient (or Reminder)

alu_operand_a_o     = {32'h0  , 1'b1};

alu_operand_b_o     = {~accum_window_q[31:0], 1'b1};

end

default: begin

//Division

alu_operand_a_o     = {accum_window_q[31:0], 1'b1}; //it contains the reminder

alu_operand_b_o     = {~op_b_shift_q[31:0], 1'b1}; //denominator negated + 1 to do -denominator

end

endcase

end

endcase

end

/*

The adder in the ALU computes alu_operand_a_o + alu_operand_b_o which means

Reminder - Divisor. If Reminder - Divisor >= 0, is_greater_equal is equal to 1,

the next Reminder is Reminder - Divisor contained in res_adder_h and the

Quotient multdiv_state_q-th bit is set to 1 using the shift register op_b_shift_q.

The

*/

always_comb

begin

if ((accum_window_q[31] ^ op_b_shift_q[31]) == 0)

is_greater_equal = (res_adder_h[31] == 0);

else

is_greater_equal = accum_window_q[31];

end

assign one_shift     = {32'b0, 1'b1} << multdiv_state_q;

assign next_reminder = is_greater_equal ? res_adder_h                   : op_remainder;

assign next_quotient = is_greater_equal ? op_a_shift_q | one_shift      : op_a_shift_q;

assign b_0             = {32{op_b_shift_q[0]}};

//build the partial product

assign op_a_bw_pp       = { ~(op_a_shift_q[32] & op_b_shift_q[0]), op_a_shift_q[31:0] & b_0 };

assign op_a_bw_last_pp  = { op_a_shift_q[32] & op_b_shift_q[0], ~(op_a_shift_q[31:0] & b_0) };

assign sign_a   = op_a_i[31] & signed_mode_i[0];

assign sign_b   = op_b_i[31] & signed_mode_i[1];

assign op_a_ext = {sign_a, op_a_i};

assign op_b_ext = {sign_b, op_b_i};

//division

assign op_remainder = accum_window_q[32:0];

assign multdiv_state_n     = multdiv_state_q - 1;

assign div_change_sign  = sign_a ^ sign_b;

assign rem_change_sign  = sign_a;

always_ff @(posedge clk or negedge rst_n) begin : proc_multdiv_state_q

if(~rst_n) begin

multdiv_state_q     <= '0;

accum_window_q   <= '0;

op_b_shift_q     <= '0;

op_a_shift_q     <= '0;

curr_state_q     <= MD_IDLE;

op_numerator_q   <= '0;

end else begin

if(mult_en_i | div_en_i) begin

unique case(curr_state_q)

MD_IDLE: begin

unique case(operator_i)

MD_OP_MULL: begin

op_a_shift_q   <= op_a_ext << 1;

accum_window_q <= {  ~(op_a_ext[32] & op_b_i[0]),  op_a_ext[31:0] & {32{op_b_i[0]}}  };

op_b_shift_q   <= op_b_ext >> 1;

curr_state_q   <= MD_COMP;

end

MD_OP_MULH: begin

op_a_shift_q   <= op_a_ext;

accum_window_q <= { 1'b1, ~(op_a_ext[32] & op_b_i[0]),  op_a_ext[31:1] & {31{op_b_i[0]}}  };

op_b_shift_q   <= op_b_ext >> 1;

curr_state_q   <= MD_COMP;

end

MD_OP_DIV: begin

//Check if the Denominator is 0

//quotient for division by 0

accum_window_q <= '1;

curr_state_q   <= equal_to_zero ? MD_FINISH : MD_ABS_A;

end

default: begin

//Check if the Denominator is 0

//reminder for division by 0

accum_window_q <= op_a_ext;

curr_state_q   <= equal_to_zero ? MD_FINISH : MD_ABS_A;

end

endcase

multdiv_state_q   <= 5'd31;

end

MD_ABS_A: begin

//quotient

op_a_shift_q   <= '0;

//A abs value

op_numerator_q <= sign_a ? alu_adder_i : op_a_i;

curr_state_q   <= MD_ABS_B;

end

MD_ABS_B: begin

//reminder

accum_window_q   <= { 32'h0, op_numerator_q[31]};

//B abs value

op_b_shift_q     <= sign_b ? alu_adder_i : op_b_i;

curr_state_q     <= MD_COMP;

end

MD_COMP: begin

multdiv_state_q   <= multdiv_state_n;

unique case(operator_i)

MD_OP_MULL: begin

accum_window_q <= res_adder_l;

op_a_shift_q   <= op_a_shift_q << 1;

op_b_shift_q   <= op_b_shift_q >> 1;

end

MD_OP_MULH: begin

accum_window_q <= res_adder_h;

op_a_shift_q   <= op_a_shift_q;

op_b_shift_q   <= op_b_shift_q >> 1;

end

default: begin

accum_window_q <= {next_reminder[31:0], op_numerator_q[multdiv_state_n]};

op_a_shift_q   <= next_quotient;

end

endcase

if(multdiv_state_q == 5'd1)

curr_state_q <= MD_LAST;

else

curr_state_q <= MD_COMP;

end

MD_LAST: begin

unique case(operator_i)

MD_OP_MULL: begin

accum_window_q <= res_adder_l;

curr_state_q   <= MD_IDLE;

end

MD_OP_MULH: begin

accum_window_q <= res_adder_l;

curr_state_q   <= MD_IDLE;

end

MD_OP_DIV: begin

//this time we save the quotient in accum_window_q since we do not need anymore the reminder

accum_window_q <= next_quotient;

curr_state_q   <= MD_CHANGE_SIGN;

end

default: begin

//this time we do not save the quotient anymore since we need only the reminder

accum_window_q <= {1'b0, next_reminder[31:0]};

curr_state_q   <= MD_CHANGE_SIGN;

end

endcase

end

MD_CHANGE_SIGN: begin

curr_state_q   <= MD_FINISH;

unique case(operator_i)

MD_OP_DIV:

accum_window_q <= (div_change_sign) ? alu_adder_i : accum_window_q;

default:

accum_window_q <= (rem_change_sign) ? alu_adder_i : accum_window_q;

endcase

end

MD_FINISH: begin

curr_state_q <= MD_IDLE;

end

default:;

endcase // curr_state_q

end

end

end

assign ready_o       = (curr_state_q == MD_FINISH) | (curr_state_q == MD_LAST & (operator_i == MD_OP_MULL | operator_i == MD_OP_MULH));

endmodule // zeroriscy_mult

module zeroriscy_prefetch_buffer

(

input  logic        clk,

input  logic        rst_n,

input  logic        req_i,

input  logic        branch_i,

input  logic [31:0] addr_i,

input  logic        ready_i,

output logic        valid_o,

output logic [31:0] rdata_o,

output logic [31:0] addr_o,

// goes to instruction memory / instruction cache

output logic        instr_req_o,

input  logic        instr_gnt_i,

output logic [31:0] instr_addr_o,

input  logic [31:0] instr_rdata_i,

input  logic        instr_rvalid_i,

// Prefetch Buffer Status

output logic        busy_o

);

enum logic [1:0] {IDLE, WAIT_GNT, WAIT_RVALID, WAIT_ABORTED } CS, NS;

logic [31:0] instr_addr_q, fetch_addr;

logic        addr_valid;

logic        fifo_valid;

logic        fifo_ready;

logic        fifo_clear;

logic        valid_stored;

//////////////////////////////////////////////////////////////////////////////

// prefetch buffer status

//////////////////////////////////////////////////////////////////////////////

assign busy_o = (CS != IDLE) || instr_req_o;

//////////////////////////////////////////////////////////////////////////////

// fetch fifo

// consumes addresses and rdata

//////////////////////////////////////////////////////////////////////////////

zeroriscy_fetch_fifo fifo_i

(

.clk                   ( clk               ),

.rst_n                 ( rst_n             ),

.clear_i               ( fifo_clear        ),

.in_addr_i             ( instr_addr_q      ),

.in_rdata_i            ( instr_rdata_i     ),

.in_valid_i            ( fifo_valid        ),

.in_ready_o            ( fifo_ready        ),

.out_valid_o           ( valid_o           ),

.out_ready_i           ( ready_i           ),

.out_rdata_o           ( rdata_o           ),

.out_addr_o            ( addr_o            ),

.out_valid_stored_o    ( valid_stored      )

);

//////////////////////////////////////////////////////////////////////////////

// fetch addr

//////////////////////////////////////////////////////////////////////////////

assign fetch_addr = {instr_addr_q[31:2], 2'b00} + 32'd4;

always_comb

begin

fifo_clear = branch_i;

end

//////////////////////////////////////////////////////////////////////////////

// instruction fetch FSM

// deals with instruction memory / instruction cache

//////////////////////////////////////////////////////////////////////////////

always_comb

begin

instr_req_o   = 1'b0;

instr_addr_o  = fetch_addr;

fifo_valid    = 1'b0;

addr_valid    = 1'b0;

NS            = CS;

unique case(CS)

// default state, not waiting for requested data

IDLE:

begin

instr_addr_o = fetch_addr;

instr_req_o  = 1'b0;

if (branch_i)

instr_addr_o = addr_i;

if (req_i & (fifo_ready | branch_i )) begin

instr_req_o = 1'b1;

addr_valid  = 1'b1;

if(instr_gnt_i) //~>  granted request

NS = WAIT_RVALID;

else begin //~> got a request but no grant

NS = WAIT_GNT;

end

end

end // case: IDLE

// we sent a request but did not yet get a grant

WAIT_GNT:

begin

instr_addr_o = instr_addr_q;

instr_req_o  = 1'b1;

if (branch_i) begin

instr_addr_o = addr_i;

addr_valid   = 1'b1;

end

if(instr_gnt_i)

NS = WAIT_RVALID;

else

NS = WAIT_GNT;

end // case: WAIT_GNT

// we wait for rvalid, after that we are ready to serve a new request

WAIT_RVALID: begin

instr_addr_o = fetch_addr;

if (branch_i)

instr_addr_o  = addr_i;

if (req_i & (fifo_ready | branch_i)) begin

// prepare for next request

if (instr_rvalid_i) begin

instr_req_o = 1'b1;

fifo_valid  = 1'b1;

addr_valid  = 1'b1;

if (instr_gnt_i) begin

NS = WAIT_RVALID;

end else begin

NS = WAIT_GNT;

end

end else begin

// we are requested to abort our current request

// we didn't get an rvalid yet, so wait for it

if (branch_i) begin

addr_valid = 1'b1;

NS         = WAIT_ABORTED;

end

end

end else begin

// just wait for rvalid and go back to IDLE, no new request

if (instr_rvalid_i) begin

fifo_valid = 1'b1;

NS         = IDLE;

end

end

end // case: WAIT_RVALID

// our last request was aborted, but we didn't yet get a rvalid and

// there was no new request sent yet

// we assume that req_i is set to high

WAIT_ABORTED: begin

instr_addr_o = instr_addr_q;

if (branch_i) begin

instr_addr_o = addr_i;

addr_valid   = 1'b1;

end

if (instr_rvalid_i) begin

instr_req_o  = 1'b1;

// no need to send address, already done in WAIT_RVALID

if (instr_gnt_i) begin

NS = WAIT_RVALID;

end else begin

NS = WAIT_GNT;

end

end

end

default:

begin

NS          = IDLE;

instr_req_o = 1'b0;

end

endcase

end

//////////////////////////////////////////////////////////////////////////////

// registers

//////////////////////////////////////////////////////////////////////////////

always_ff @(posedge clk, negedge rst_n)

begin

if(rst_n == 1'b0)

begin

CS              <= IDLE;

instr_addr_q    <= '0;

end

else

begin

CS              <= NS;

if (addr_valid) begin

instr_addr_q    <= instr_addr_o;

end

end

end

Endmodule

`include "zeroriscy_config.sv"

module zeroriscy_register_file

#(

parameter RV32E         = 0,

parameter DATA_WIDTH    = 32

)

(

// Clock and Reset

input  logic                   clk,

input  logic                   rst_n,

input  logic                   test_en_i,

//Read port R1

input  logic [4:0]             raddr_a_i,

output logic [DATA_WIDTH-1:0]  rdata_a_o,

//Read port R2

input  logic [4:0]             raddr_b_i,

output logic [DATA_WIDTH-1:0]  rdata_b_o,

// Write port W1

input  logic [4:0]              waddr_a_i,

input  logic [DATA_WIDTH-1:0]   wdata_a_i,

input  logic                    we_a_i

);

localparam    ADDR_WIDTH = RV32E ? 4 : 5;

localparam    NUM_WORDS  = 2**ADDR_WIDTH;

logic [DATA_WIDTH-1:0]      mem[NUM_WORDS];

logic [NUM_WORDS-1:1]       waddr_onehot_a;

logic [NUM_WORDS-1:1]       mem_clocks;

logic [DATA_WIDTH-1:0]      wdata_a_q;

// Write port W1

logic [ADDR_WIDTH-1:0]     raddr_a_int, raddr_b_int, waddr_a_int;

assign raddr_a_int = raddr_a_i[ADDR_WIDTH-1:0];

assign raddr_b_int = raddr_b_i[ADDR_WIDTH-1:0];

assign waddr_a_int = waddr_a_i[ADDR_WIDTH-1:0];

logic clk_int;

int unsigned i;

int unsigned j;

int unsigned k;

genvar x;

//-----------------------------------------------------------------------------

//-- READ : Read address decoder RAD

//-----------------------------------------------------------------------------

assign rdata_a_o = mem[raddr_a_int];

assign rdata_b_o = mem[raddr_b_int];

//-----------------------------------------------------------------------------

// WRITE : SAMPLE INPUT DATA

//---------------------------------------------------------------------------

cluster_clock_gating CG_WE_GLOBAL

(

.clk_i     ( clk             ),

.en_i      ( we_a_i          ),

.test_en_i ( test_en_i       ),

.clk_o     ( clk_int         )

);

// use clk_int here, since otherwise we don't want to write anything anyway

always_ff @(posedge clk_int, negedge rst_n)

begin : sample_waddr

if (~rst_n) begin

wdata_a_q        <= '0;

end else begin

if(we_a_i)

wdata_a_q <= wdata_a_i;

end

end

//-----------------------------------------------------------------------------

//-- WRITE : Write Address Decoder (WAD), combinatorial process

//-----------------------------------------------------------------------------

always_comb

begin : p_WADa

for(i = 1; i < NUM_WORDS; i++)

begin : p_WordItera

if ( (we_a_i == 1'b1 ) && (waddr_a_int == i) )

waddr_onehot_a[i] = 1'b1;

else

waddr_onehot_a[i] = 1'b0;

end

end

//-----------------------------------------------------------------------------

//-- WRITE : Clock gating (if integrated clock-gating cells are available)

//-----------------------------------------------------------------------------

generate

for(x = 1; x < NUM_WORDS; x++)

begin : CG_CELL_WORD_ITER

cluster_clock_gating CG_Inst

(

.clk_i     ( clk_int                               ),

.en_i      ( waddr_onehot_a[x]                     ),

.test_en_i ( test_en_i                             ),

.clk_o     ( mem_clocks[x]                         )

);

end

endgenerate

//-----------------------------------------------------------------------------

//-- WRITE : Write operation

//-----------------------------------------------------------------------------

//-- Generate M = WORDS sequential processes, each of which describes one

//-- word of the memory. The processes are synchronized with the clocks

//-- ClocksxC(i), i = 0, 1, ..., M-1

//-- Use active low, i.e. transparent on low latches as storage elements

//-- Data is sampled on rising clock edge

always_latch

begin : latch_wdata

// Note: The assignment has to be done inside this process or Modelsim complains about it

mem[0] = '0;

for(k = 1; k < NUM_WORDS; k++)

begin : w_WordIter

if(mem_clocks[k] == 1'b1)

mem[k] = wdata_a_q;

end

end

Endmodule

`include "zeroriscy_config.sv"

module zeroriscy_register_file

#(

parameter RV32E         = 0,

parameter DATA_WIDTH    = 32

)

(

// Clock and Reset

input  logic                   clk,

input  logic                   rst_n,

input  logic                   test_en_i,

//Read port R1

input  logic [4:0]             raddr_a_i,

output logic [DATA_WIDTH-1:0]  rdata_a_o,

//Read port R2

input  logic [4:0]             raddr_b_i,

output logic [DATA_WIDTH-1:0]  rdata_b_o,

// Write port W1

input  logic [4:0]              waddr_a_i,

input  logic [DATA_WIDTH-1:0]   wdata_a_i,

input  logic                    we_a_i

);

localparam    ADDR_WIDTH = RV32E ? 4 : 5;

localparam    NUM_WORDS  = 2**ADDR_WIDTH;

logic [NUM_WORDS-1:0][DATA_WIDTH-1:0] rf_reg;

logic [NUM_WORDS-1:0][DATA_WIDTH-1:0] rf_reg_tmp;

logic [NUM_WORDS-1:0]                 we_a_dec;

always_comb

begin : we_a_decoder

for (int i = 0; i < NUM_WORDS; i++) begin

if (waddr_a_i == i)

we_a_dec[i] = we_a_i;

else

we_a_dec[i] = 1'b0;

end

end

genvar i;

generate

// loop from 1 to NUM_WORDS-1 as R0 is nil

for (i = 1; i < NUM_WORDS; i++)

begin : rf_gen

always_ff @(posedge clk, negedge rst_n)

begin : register_write_behavioral

if (rst_n==1'b0) begin

rf_reg_tmp[i] <= 'b0;

end else begin

if (we_a_dec[i])

rf_reg_tmp[i] <= wdata_a_i;

end

end

end

// R0 is nil

assign rf_reg[0] = '0;

assign rf_reg[NUM_WORDS-1:1] = rf_reg_tmp[NUM_WORDS-1:1];

endgenerate

assign rdata_a_o = rf_reg[raddr_a_i];

assign rdata_b_o = rf_reg[raddr_b_i];

Endmodule

`ifndef VERILATOR

`include "zeroriscy_config.sv"

import zeroriscy_defines::*;

import zeroriscy_tracer_defines::*;

// Source/Destination register instruction index

`define REG_S1 19:15

`define REG_S2 24:20

`define REG_S3 29:25

`define REG_D  11:07

module zeroriscy_tracer

#(

parameter REG_ADDR_WIDTH      = 5

)

(

// Clock and Reset

input  logic        clk,

input  logic        rst_n,

input  logic        fetch_enable,

input  logic [3:0]  core_id,

input  logic [5:0]  cluster_id,

input  logic [31:0] pc,

input  logic [31:0] instr,

input  logic        compressed,

input  logic        id_valid,

input  logic        is_decoding,

input  logic        is_branch,

input  logic        branch_taken,

input  logic        pipe_flush,

input  logic        mret_insn,

input  logic        ecall_insn,

input  logic        ebrk_insn,

input  logic        csr_status,

input  logic [31:0] rs1_value,

input  logic [31:0] rs2_value,

input  logic [31:0] lsu_value,

input  logic [(REG_ADDR_WIDTH-1):0] ex_reg_addr,

input  logic        ex_reg_we,

input  logic [31:0] ex_reg_wdata,

input  logic        data_valid_lsu,

input  logic        ex_data_req,

input  logic        ex_data_gnt,

input  logic        ex_data_we,

input  logic [31:0] ex_data_addr,

input  logic [31:0] ex_data_wdata,

input  logic [31:0] lsu_reg_wdata,

input  logic [31:0] imm_u_type,

input  logic [31:0] imm_uj_type,

input  logic [31:0] imm_i_type,

input  logic [11:0] imm_iz_type,

input  logic [31:0] imm_z_type,

input  logic [31:0] imm_s_type,

input  logic [31:0] imm_sb_type

);

integer      f;

string       fn;

integer      cycles;

logic [ 4:0] rd, rs1, rs2, rs3;

typedef struct {

logic [(REG_ADDR_WIDTH-1):0] addr;

logic [31:0] value;

} reg_t;

typedef struct {

logic [31:0] addr;

logic        we;

logic [ 3:0] be;

logic [31:0] wdata;

logic [31:0] rdata;

} mem_acc_t;

class instr_trace_t;

time         simtime;

integer      cycles;

logic [31:0] pc;

logic [31:0] instr;

string       str;

reg_t        regs_read[$];

reg_t        regs_write[$];

mem_acc_t    mem_access[$];

function new ();

str        = "";

regs_read  = {};

regs_write = {};

mem_access = {};

endfunction

function string regAddrToStr(input logic [(REG_ADDR_WIDTH-1):0] addr);

begin

if (addr < 10)

return $sformatf(" x%0d", addr);

else

return $sformatf("x%0d", addr);

end

endfunction

function void printInstrTrace();

mem_acc_t mem_acc;

begin

$fwrite(f, "%t %15d %h %h %-36s", simtime,

cycles,

pc,

instr,

str);

foreach(regs_write[i]) begin

if (regs_write[i].addr != 0)

$fwrite(f, " %s=%08x", regAddrToStr(regs_write[i].addr), regs_write[i].value);

end

foreach(regs_read[i]) begin

if (regs_read[i].addr != 0)

$fwrite(f, " %s:%08x", regAddrToStr(regs_read[i].addr), regs_read[i].value);

end

if (mem_access.size() > 0) begin

mem_acc = mem_access.pop_front();

$fwrite(f, "  PA:%08x", mem_acc.addr);

end

$fwrite(f, "\n");

end

endfunction

function void printMnemonic(input string mnemonic);

begin

str = mnemonic;

end

endfunction // printMnemonic

function void printRInstr(input string mnemonic);

begin

regs_read.push_back('{rs1, rs1_value});

regs_read.push_back('{rs2, rs2_value});

regs_write.push_back('{rd, 'x});

str = $sformatf("%-16s x%0d, x%0d, x%0d", mnemonic, rd, rs1, rs2);

end

endfunction // printRInstr

function void printIInstr(input string mnemonic);

begin

regs_read.push_back('{rs1, rs1_value});

regs_write.push_back('{rd, 'x});

str = $sformatf("%-16s x%0d, x%0d, %0d", mnemonic, rd, rs1, $signed(imm_i_type));

end

endfunction // printIInstr

function void printIuInstr(input string mnemonic);

begin

regs_read.push_back('{rs1, rs1_value});

regs_write.push_back('{rd, 'x});

str = $sformatf("%-16s x%0d, x%0d, 0x%0x", mnemonic, rd, rs1, imm_i_type);

end

endfunction // printIuInstr

function void printUInstr(input string mnemonic);

begin

regs_write.push_back('{rd, 'x});

str = $sformatf("%-16s x%0d, 0x%0h", mnemonic, rd, {imm_u_type[31:12], 12'h000});

end

endfunction // printUInstr

function void printUJInstr(input string mnemonic);

begin

regs_write.push_back('{rd, 'x});

str =  $sformatf("%-16s x%0d, %0d", mnemonic, rd, $signed(imm_uj_type));

end

endfunction // printUJInstr

function void printSBInstr(input string mnemonic);

begin

regs_read.push_back('{rs1, rs1_value});

regs_read.push_back('{rs2, rs2_value});

str =  $sformatf("%-16s x%0d, x%0d, %0d", mnemonic, rs1, rs2, $signed(imm_sb_type));

end

endfunction // printSBInstr

function void printCSRInstr(input string mnemonic);

logic [11:0] csr;

begin

csr = instr[31:20];

regs_write.push_back('{rd, 'x});

if (instr[14] == 1'b0) begin

regs_read.push_back('{rs1, rs1_value});

str = $sformatf("%-16s x%0d, x%0d, 0x%h", mnemonic, rd, rs1, csr);

end else begin

str = $sformatf("%-16s x%0d, 0x%h, 0x%h", mnemonic, rd, imm_z_type, csr);

end

end

endfunction // printCSRInstr

function void printLoadInstr();

string mnemonic;

logic [2:0] size;

begin

// detect reg-reg load and find size

size = instr[14:12];

if (instr[14:12] == 3'b111)

size = instr[30:28];

case (size)

3'b000: mnemonic = "lb";

3'b001: mnemonic = "lh";

3'b010: mnemonic = "lw";

3'b100: mnemonic = "lbu";

3'b101: mnemonic = "lhu";

3'b110: mnemonic = "p.elw";

3'b011,

3'b111: begin

printMnemonic("INVALID");

return;

end

endcase

regs_write.push_back('{rd, 'x});

if (instr[14:12] != 3'b111) begin

// regular load

regs_read.push_back('{rs1, rs1_value});

str = $sformatf("%-16s x%0d, %0d(x%0d)", mnemonic, rd, $signed(imm_i_type), rs1);

end else begin

printMnemonic("INVALID");

end

end

endfunction

function void printStoreInstr();

string mnemonic;

begin

case (instr[13:12])

2'b00:  mnemonic = "sb";

2'b01:  mnemonic = "sh";

2'b10:  mnemonic = "sw";

2'b11: begin

printMnemonic("INVALID");

return;

end

endcase

if (instr[14] == 1'b0) begin

// regular store

regs_read.push_back('{rs2, rs2_value});

regs_read.push_back('{rs1, rs1_value});

str = $sformatf("%-16s x%0d, %0d(x%0d)", mnemonic, rs2, $signed(imm_s_type), rs1);

end else begin

printMnemonic("INVALID");

end

end

endfunction // printSInstr

endclass

mailbox #(instr_trace_t) instr_ex = new ();

mailbox #(instr_trace_t) instr_wb = new ();

// cycle counter

always_ff @(posedge clk, negedge rst_n)

begin

if (rst_n == 1'b0)

cycles = 0;

else

cycles = cycles + 1;

end

// open/close output file for writing

initial

begin

wait(rst_n == 1'b1);

wait(fetch_enable == 1'b1);

$sformat(fn, "trace_core_%h_%h.log", cluster_id, core_id);

$display("[TRACER] Output filename is: %s", fn);

f = $fopen(fn, "w");

$fwrite(f, "                Time          Cycles PC       Instr    Mnemonic\n");

end

final

begin

$fclose(f);

end

assign rd  = instr[`REG_D];

assign rs1 = instr[`REG_S1];

assign rs2 = instr[`REG_S2];

assign rs3 = instr[`REG_S3];

// log execution

always @(negedge clk)

begin

instr_trace_t trace;

mem_acc_t     mem_acc;

// special case for WFI because we don't wait for unstalling there

if ( (id_valid || mret_insn || ecall_insn || pipe_flush || ebrk_insn || csr_status || ex_data_req) && is_decoding)

begin

trace = new ();

trace.simtime    = $time;

trace.cycles     = cycles;

trace.pc         = pc;

trace.instr      = instr;

// use casex instead of case inside due to ModelSim bug

casex (instr)

// Aliases

32'h00_00_00_13:   trace.printMnemonic("nop");

// Regular opcodes

INSTR_LUI:        trace.printUInstr("lui");

INSTR_AUIPC:      trace.printUInstr("auipc");

INSTR_JAL:        trace.printUJInstr("jal");

INSTR_JALR:       trace.printIInstr("jalr");

// BRANCH

INSTR_BEQ:        trace.printSBInstr("beq");

INSTR_BNE:        trace.printSBInstr("bne");

INSTR_BLT:        trace.printSBInstr("blt");

INSTR_BGE:        trace.printSBInstr("bge");

INSTR_BLTU:       trace.printSBInstr("bltu");

INSTR_BGEU:       trace.printSBInstr("bgeu");

// OPIMM

INSTR_ADDI:       trace.printIInstr("addi");

INSTR_SLTI:       trace.printIInstr("slti");

INSTR_SLTIU:      trace.printIInstr("sltiu");

INSTR_XORI:       trace.printIInstr("xori");

INSTR_ORI:        trace.printIInstr("ori");

INSTR_ANDI:       trace.printIInstr("andi");

INSTR_SLLI:       trace.printIuInstr("slli");

INSTR_SRLI:       trace.printIuInstr("srli");

INSTR_SRAI:       trace.printIuInstr("srai");

// OP

INSTR_ADD:        trace.printRInstr("add");

INSTR_SUB:        trace.printRInstr("sub");

INSTR_SLL:        trace.printRInstr("sll");

INSTR_SLT:        trace.printRInstr("slt");

INSTR_SLTU:       trace.printRInstr("sltu");

INSTR_XOR:        trace.printRInstr("xor");

INSTR_SRL:        trace.printRInstr("srl");

INSTR_SRA:        trace.printRInstr("sra");

INSTR_OR:         trace.printRInstr("or");

INSTR_AND:        trace.printRInstr("and");

// SYSTEM (CSR manipulation)

INSTR_CSRRW:      trace.printCSRInstr("csrrw");

INSTR_CSRRS:      trace.printCSRInstr("csrrs");

INSTR_CSRRC:      trace.printCSRInstr("csrrc");

INSTR_CSRRWI:     trace.printCSRInstr("csrrwi");

INSTR_CSRRSI:     trace.printCSRInstr("csrrsi");

INSTR_CSRRCI:     trace.printCSRInstr("csrrci");

// SYSTEM (others)

INSTR_ECALL:      trace.printMnemonic("ecall");

INSTR_EBREAK:     trace.printMnemonic("ebreak");

INSTR_MRET:       trace.printMnemonic("mret");

INSTR_WFI:        trace.printMnemonic("wfi");

// RV32M

INSTR_PMUL:       trace.printRInstr("mul");

INSTR_PMUH:       trace.printRInstr("mulh");

INSTR_PMULHSU:    trace.printRInstr("mulhsu");

INSTR_PMULHU:     trace.printRInstr("mulhu");

INSTR_DIV:        trace.printRInstr("div");

INSTR_DIVU:       trace.printRInstr("divu");

INSTR_REM:        trace.printRInstr("rem");

INSTR_REMU:       trace.printRInstr("remu");

{25'b?, OPCODE_LOAD}:       trace.printLoadInstr();

{25'b?, OPCODE_STORE}:      trace.printStoreInstr();

default:           trace.printMnemonic("INVALID");

endcase // unique case (instr)

// replace register written back

foreach(trace.regs_write[i]) begin

if ((trace.regs_write[i].addr == ex_reg_addr) && ex_reg_we)

trace.regs_write[i].value = ex_reg_wdata;

end

// look for data accesses and log them

if (ex_data_req) begin

if(~ex_data_gnt) begin

//we wait until the the gnt comes

do @(negedge clk);

while (!ex_data_gnt);

end

mem_acc.addr = ex_data_addr;

mem_acc.we   = ex_data_we;

if (mem_acc.we)

mem_acc.wdata = ex_data_wdata;

else

mem_acc.wdata = 'x;

//we wait until the the data instruction ends

do @(negedge clk);

while (!data_valid_lsu);

if (~mem_acc.we)

//load operations

foreach(trace.regs_write[i])

trace.regs_write[i].value = lsu_reg_wdata;

trace.mem_access.push_back(mem_acc);

end

trace.printInstrTrace();

end

end // always @ (posedge clk)

endmodule

`endif